3GPP TSG-RAN WG2 Meeting #127bis	R2-2407982

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.4

Source:	NERCDTV

Title:			  Discussion on support of broadcast service in NTN

WI Code:       NR_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

A new WI was agreed for R19 NTN. And one objective of R19 NTN is to study and specify support of broadcast service as follows [1]:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

And in the meeting of RAN2 125bis, R19 NTN was discussed in RAN2 for the first time and some agreements were made for support of broadcast service [2]:

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

And in RAN2 126 meeting, there are also some agreements achieved for support of broadcast service as follows [3]:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

And in the meeting of RAN2 127, the agreements below were further made [4]:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we will discuss on the details on signalling of the intended service area of a broadcast service based on the agreements above, and give some proposals.

# Discussion

As agreed in the last meeting, the service area information can be considered to be included in SIB20/ MBSBroadcastConfiguration. And it was also agreed that when intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details).The information of an intended service area may include Area Id and the related geographical information. For example, the geographical information may include ReferenceLocation and DistanceRadius if the intended service area is indicated by TN coverage. And the configuration of an MBS session includes mbs-SessionId and other information in MBS-SessionInfo. There may be a lot of combinations to include the configuration of an MBS session and the information of the corresponding intended service area into SIB20/ MBSBroadcastConfiguration. Next we will discuss on the potential options.

One potential option is to add the information of the corresponding intended service area into the MBS-SessionInfo of a MBS session. Further, it has to be decided whether the whole information of the corresponding intended service area or only the corresponding Area Id should be added. We prefer to only include the corresponding Area Id into MBS-SessionInfo, and the related geographical information (e.g., ReferenceLocation and DistanceRadius) can be included in system information (e.g., SIB20) like for TN coverage. And there will be less impact to the standard by this way.

Observation 1：One option is to add the Area Id of the corresponding intended service area into the MBS-SessionInfo of a MBS session.

Another option is to add the information of a MBS session into the configuration of the corresponding intended service area included in system information (e.g., SIB20). Also, it has to be decided whether the whole information of the MBS session or only the corresponding mbs-SessionId should be added. We think the mbs-SessionId is enough to associate the intended service area with a MBS session and the content included in system information should be as less as possible for the efficient usage of network resource.

Observation 2：Another option is to add the mbs-SessionId of a MBS session into the configuration of the corresponding intended service area included in system information.

For the signalling structure of the intended service area of a broadcast service, we think the above two options can be considered.

Option 1: Add the Area Id of the corresponding intended service area into the MBS-SessionInfo of a MBS session.

Option 2: Add the mbs-SessionId of a MBS session into the configuration of the corresponding intended service area included in system information.

However, using SIB20 may result in signalling overhead and extra modification of SIB20 caused by service area info update. So we think option1 is more suitable than option2.

Proposal 1：Add the Area Id of the corresponding intended service area into the MBS-SessionInfo of a MBS session.

# 3. Conclusion

Based on the above discussion, we give the following observations and proposal:

Observation 1：One option is to add the Area Id of the corresponding intended service area into the MBS-SessionInfo of a MBS session.

Observation 2：Another option is to add the mbs-SessionId of a MBS session into the configuration of the corresponding intended service area included in system information.

Proposal 1：Add the Area Id of the corresponding intended service area into the MBS-SessionInfo of a MBS session.

# References

[1] RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] RAN2#125bis chairman’s notes.

[3] RAN2#126 chairman’s notes.

[4] RAN2#127 chairman’s notes.