3GPP TSG-RAN WG2 Meeting #127bis	R2-2407986

Hefei, China, 14th October – 18th October, 2024

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will firstly discuss how to avoid unnecessary RLC retransmission, and then provide our views on the timely RLC retransmission.

# 2	Discussion on avoiding unnecessary transmission

From RAN2#127 agreements [2], to avoid unnecessary RLC AM retransmission for the outdated RLC SDU (segment), RAN2 assumed that the reassembly window at the Rx RLC entity is updated in advance to update the transmitting window at the Tx RLC entity for designing the both Tx initiated and Rx initiated approaches, as given below:

- Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

The design principles of the Rx initiated and Tx initiated approaches are given in Section 2.1 and 2.2 respectively, while section 2.3 provides a comparison of these two directions and proposes a way forward.

## 2.1	Rx initiated RLC AM retransmission enhancement

According to the agreement of RAN2 #127 [2], the Rx initiated approach should be devised by considering the following factors:

- In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

In the Rx initiated approach, an outdated RLC SDU that has not been received successfully should not trigger NACK. Otherwise, the outdated RLC SDU will be retransmitted as RAN2 has agreed “Tx side just processes the Status Report as in legacy”. Rx RLC entity does not send NACK to the Tx RLC entity, which can be regarded as a fake ACK in the Tx RLC entity. An outdated RLC SDU can be defined based on a timer introduced in the Rx RLC entity. An RLC SDU is considered as outdated when the maximum waiting time for transmission is reached. For the outdated RLC SDU, the Rx RLC entity stops waiting for the RLC retransmission and updates the reassembly window. Meanwhile, the Tx RLC entity can consider the RLC SDU as successfully transmitted since there is no associated NACK included in the Status Report, so the Tx RLC entity updates its transmitting window accordingly.

At least two possible timer based options in the Rx initiated approach have been observed:

Option 1 Timer maintained for each RLC SDU: Figure 1 gives the example for the Rx initiated RLC AM retransmission enhancement solution. In Figure 1, the RLC SDUs labelled with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. As a consequence, the resultant RX_Next_Highest > RX_Next + 1 allows the Rx RLC entity to identify the missing RLC SDUs, hence starting their timers, respectively. Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, the RLC SDUs of SN = 1 and 3 are still received incompletely, and the RLC SDU of SN = 5 is missing. Then, the Rx RLC entity stops the timer of RLC SDU of SN = 2, and starts the timer of RLC SDU of SN = 5. After a while, the timer of RLC SDUs with SN = 1 and 3 are expired, the Rx RLC entity will stop waiting for RLC SDUs of SN = 1 and 3, and updates the variable RX_Next accordingly. To be more specific, the RLC SDUs with SN = 1 and 3 will neither trigger Status Report nor be NACK reported in the Status Report. Thus, in the next RLC Status Report transmitted, only RLC SDU of SN = 5 is NACK reported. Therefore, the Tx RLC entity can consider the RLC SDUs with SN 1 and 3 successfully received at the Rx RLC entity, and update the transmitting window and arrange retransmission of RLC SDU of SN = 5 only.

Figure 1 Timer maintained per RLC SDU for the Rx initiated RLC AM retransmission enhancement

Option 2 Timer maintained for the Rx RLC entity: Figure 2 illustrates another example for the Rx initiated RLC AM retransmission enhancement solution. Instead of maintaining a timer for each RLC SDU, a timer similar to t-reassembly, referred as T below, can be maintained for the Rx RLC entity. Meanwhile, a new variable RX_T can be introduced at the Rx RLC entity to determine the outdated RLC SDUs. When the timer T expires, the Rx RLC entity will consider the unsuccessfully received RLC SDUs with SN between RX_Next and RX_T as outdated, and then updates RX_Next and RX_T accordingly.

For example, the RLC SDUs with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. The resultant SN gap drives the Rx RLC entity to start the new timer T and meanwhile to set the new variable RX_T to RX_Next_Highest, i.e. SN = 5, as shown in Figure 2 (a).  Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, and the RLC SDUs of SN = 1, 3 and 5 are unsuccessfully received, as shown in Figure 2(b). After a while, as the timer T is expired, the Rx RLC entity considers the RLC SDUs with SN between RX_Next and RX_T, i.e. SN = 1 and 3, as outdated, and hence stop receiving their RLC SDUs. As the consequence depicted in Figure 2(c), the Rx RLC entity updates RX_Next to SN = 5. Since there remains the SN gap, i.e. RX_Next_Highest > RX_Next + 1, the timer will be started again, and RX_T is set to SN = 7. Moreover, the RLC SDUs with SN = 1 and 3 will neither trigger Status Report nor be NACK reported, their late received SDUs (segments) will be discarded, and only RLC SDU of SN = 5 will be NACK reported in the next RLC Status Report. Therefore, the Tx RLC entity can consider the RLC SDUs with SN 1 and 3 successfully received at the Rx RLC entity, and update the transmitting window and arrange retransmission of RLC SDU of SN = 5 only.



(a)

(b)

(c)

Figure 2 Timer maintained per entity for the Rx initiated RLC AM retransmission enhancement

Proposal 1:	For the Rx initiated approach, RAN2 to discuss whether the timer to determine the outdated RLC SDU should be maintained per RLC SDU or per Rx RLC entity.

## 2.2	Tx initiated RLC AM retransmission enhancement

Since the Tx RLC entity has the full knowledge of the remaining time of the data, it may stop retransmitting the data as soon as it becomes outdated. In the Tx approach, a notification can be further triggered and sent to the corresponding Rx RLC entity, which is relatively similar to the Rel-18 PDCP enhancement aimed for solving SN gap.

To be more specific, once the RLC SDU becomes outdated, i.e. an indication of SDU discard is sent from the PDCP layer, the Tx RLC entity can stop retransmitting the outdated data even if the associated NACK is received. Meanwhile, an RLC notification can be triggered by the Tx RLC entity to notify the Rx RLC entity of the SNs that the associated RLC SDUs will no longer be retransmitted. This is also able to trigger the Rx RLC entity to update the reassembly window accordingly, if necessary.

Figure 3 illustrates an instance of Tx initiated RLC AM enhancement solution. First of all, the transmitting side starts recording the delay budget for each RLC SDUs, e.g. RLC SDU of SN = 1,2,3 and 4, as long as they are obtained, such as starting discardTimer at the PDCP layer. When the RLC SDUs with SN = 1,2 and 3 are received incompletely at the Rx RLC entity at their initial transmission, an associated Status Report is triggered and transmitted. The Tx RLC entity then arranges the retransmission of RLC SDUs of SN = 1,2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity. Assuming that RLC SDUs of SN = 1,3,5 are not completely received at this transmission, a Status Report is sent from the Rx RLC entity in order to ask for retransmission. However, at the reception of RLC Status Report, the delay budgets of RLC SDUs of SN = 1 and 3 have been used up. As a consequence, the Tx RLC entity determines to stop retransmitting the RLC SDUs of SN = 1 and 3, and an RLC notification is triggered at the Tx RLC entity for indicating the SNs of the RLC SDUs no longer retransmitted, which can then be transmitted together with the valid data, e.g. retransmission of RLC SDU of SN = 5. When such notification is received, the Rx RLC entity can stop triggering and sending NACK for the RLC SDUs of indicated SN, and update the reassembly window accordingly.

Figure 3 Tx initiated RLC AM retransmission enhancement

Moreover, there are also two options for the Tx initiated approach for updating the transmitting window:

Option 1: The Tx RLC entity updates the transmitting window once an RLC SDU (segment) is discarded or the notification is transmitted.

Option 2: The Tx RLC entity updates the transmitting window based on the reception of the RLC Status Report.

Option 1 encourages to update the transmitting window before advancing the reassembly window, which conflicts to the agreed principle quoted in the beginning of clause 2. It may lead to desynchronization between the two windows. For example, as the reassembly window is updated based on the notification sent from the Tx RLC entity, there may be a case that the notification is lost and hence the reassembly window stalls. In such case, the Tx RLC entity may send an RLC SDU with SN exceeding the upper boundary of the reassembly window. As a result, the transmitted RLC SDU cannot be correctly received at the Rx RLC entity.

By contrast, the legacy transmitting window update mechanism can be reused in Option 2, where the Tx RLC entity updates the transmitting window when the associated ACK is received. To be more specific, the Rx RLC entity can update its reassembly window upon the reception of the RLC notification, e.g. updating RX_Next, and construct the RLC Status Report accordingly, which is relatively identical to the legacy procedure for the transmitting window update.

Hence, we believe that in the Tx initiated approach, Option 2 provides a more rational solution for maintaining the synchronization between the transmitting window and reassembly window, without imposing further specification impact on the behaviour of the Tx RLC entity.

Observation 1: The legacy RLC Status Report based window update mechanism can be reused in Tx initiated approach.

Proposal 2:	In the Tx initiated approach, Tx RLC entity updates the transmitting window when the RLC Status Report is received, as in the existing specifications.

## 2.3	Analysis of Two RLC AM retransmission enhancement solutions

As can be seen above, the Rx initiated option mainly modifies the Rx RLC entity behaviours, where new timer(s) need to be introduced to limit the maximum retransmission times per the maximum waiting duration. However, the outdated data is likely to still be retransmitted, because the Rx RLC entity does not know the precise remaining time of each RLC SDU (segment). For the Rx initiated option, the value of the new timer should be carefully selected in order to minimize the resources waste.

Observation 2: The Rx initiated approach is inaccurate due to lack of exact expiry time of each RLC SDU (segment) at the Rx RLC entity.

By contrast, since the Tx RLC entity has knowledge of the remaining time of the RLC SDU (segment), the retransmission of the outdated data can be terminated timely to avoid resource waste. Additionally, a notification can be triggered and sent from the Tx RLC entity to the corresponding Rx RLC entity, to prevent the Rx RLC entity from triggering and sending unnecessary RLC Status Report for an outdated RLC SDU, and to trigger the Rx RLC entity to update the reassembly window. However, the signalling overhead introduced by the Tx initiated approach is also evident.

Observation 3: The Tx initiated approach will introduce some signalling overheads to transmit a notification for outdated data.

As analysed above, both options can reduce unnecessary RLC AM retransmissions, but each has its own problems, i.e. additional overhead in case of the Tx initiated approach and unnecessary retransmissions for the Rx initiated approach.

Therefore, a combined solution was proposed in RAN2 127[2] to avoid transmitting the notification, as given below:

- In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In order to guarantee the window synchronization between the Tx RLC entity and the Rx RLC entity, the reassembly window should be updated before advancing the transmitting window. To be more specific, the Rx RLC entity updates the reassembly window once it determines there are obsolete RLC SDUs, while the Tx RLC entity can then update the transmitting window only when the Status Report is received, as in the legacy operation. Otherwise, if the transmitting window updates prior to updating the reassembly window, Tx RLC entity has to cope with the case when the Tx RLC entity receives the status for the RLC SDU with SN falling outside the transmitting window. The Tx RLC entity would have to, for example, ignore the status for the SDU with SN falling outside the transmitting window, which will impose more efforts on specification modification.

Proposal 3:	RAN2 to adopt the combined approach, where:

- Tx side stops retransmitting an obsolete SDUs based on the discard indication from its PDCP transmitting entity.

- Rx side determines SDU(s) to be obsolete based on a locally maintained timer.

- Tx RLC entity updates the transmitting window when the RLC Status Report is received, as in the existing specifications.

# 3. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2 126 [3] meeting, as given below:

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

The major motivation on timely RLC retransmission is to reduce RLC ARQ retransmission latency, there are three possible options:

- Alt1 Autonomous retransmission is to offer the Tx RLC entity a sort of capability to autonomously retransmit an RLC SDU (segment) even if the corresponding NACK has not been received. To be more specific, to enable autonomous RLC retransmission, an RLC SDU (segment) can be considered for retransmission when the specific condition(s) is met. For example, the corresponding remaining time drops below a threshold before the associated feedback is received, as shown in Figure 4 (a). Alternatively, autonomous RLC retransmission can be triggered when the associated feedback is not received at Tx RLC entity within a duration since its last transmission, as illustrated in Figure 4 (b). Moreover, the autonomous retransmission can be limited to be triggered based on the channel quality, e.g. RSRP, where the RLC SDU (segment) can be retransmitted when the channel quality is deteriorated, to ensure the transmission reliability of the associated RLC SDU.

(a) Retransmitting SDU#1 when its remaining time below a threshold and no associated feedback is received



(b)  Retransmitting SDU#1 when no any associated feedback received within a certain period Figure 4 potential methods for autonomous RLC retransmission

Alt2 Enhanced Status Report based retransmission is to enable more frequent RLC Status Report. To achieve this, additional Status Report triggering condition may be needed, e.g. periodical Status Report. Alternatively, as the Status Report can be triggered by the expiry of t-reassembly, a shorter value for t-reassembly can also be considered. Moreover, since the triggered Status Report may be blocked by the t-StatusProhibit, shorter t-reassembly needs to work together with a shorter t-StatusProhibit.

Alt3 Enhanced polling based retransmission tends to reuse the existing Status Report based RLC retransmission mechanism, but applies a more proactive mechanism to request a frequent Status Report to enable the RLC retransmission latency reduction. One potential enhancement is to consider the remaining time of the data as an additional factor for polling triggering. For example, when there is delay-critical RLC SDU, or when the data volume of the delay-critical SDUs exceeds a threshold, the polling can be triggered. Upon the reception of the polling, the Rx RLC entity can trigger Status Report. Similar as above, the frequent polling needs to work together with a shorter t-StatusProhibit.

In our view, Alt2 and Alt3 are quite the same and they have a common purpose to have a fast Status Report. However, from our analysis, there is no need to enhance Status Report or introduce new polling trigger. For the Status Report based enhancement, since the Status Report will be triggered when t-reassembly timer is expired, and t-reassembly timer will be started when there is missing RLC or RLC segment detected, the Status Report will be triggered with the periodicity equal to the length of t-reassembly once there is missing RLC or RLC segment. Therefore, we observe no extra benefit from introducing the periodical Status Report. From our perspective, fast Status Report triggering and the associated rapid RLC retransmission can be implemented by simply setting a short t-reassembly, while an appropriate configuration of t-StatusProhibit can effectively prevent frequent Status Report transmissions. This is similar to the polling based enhancement, where an appropriate configuration of the polling triggering condition, e.g. based on pollByte/pollPDU, is able to ensure that the polling can be triggered and transmitted timely. Moreover, it is worth noting that only the uplink scenario is considered in timely retransmission. In such case, the network can determine whether, when and how to transmit the Status Report to the UE regardless of reception of the polling or the configuration of the t-reassembly.

Observation 4: For Status Report based and polling based solutions, the fast Status Report triggering can be addressed by network configuring appropriate parameter values, e.g. t-reassembly, t-StatusProhibit, and pollByte/pollPDU.

Observation 5: In the uplink scenario, whether, when and how to trigger and transmit the RLC Status Report is up to the network implementation.

Based on the observations above, there is no need to discuss Status Report based on polling based enhancements.

Proposal 4: RAN2 does not specify any enhanced Status Report or enhanced polling based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

As discussed above, the RLC retransmission latency can be reduced by configuring an appropriate value for the parameters relevant to the Status Report triggering. However, the Status Report based retransmission is not one hundred percent reliable. In an example, it is likely that the retransmission of the RLC SDU has failed and the Status Report has not been received at the Tx RLC entity, hence imposing latency on the RLC SDU retransmission. Therefore, we believe that Alt1 can be a complementary solution to achieve the timely RLC retransmission even if network has appropriately set the parameter values related to the Status Report.

Proposal 5:	RAN2 works on the autonomous RLC retransmission, where the RLC SDU (segment) can be retransmitted based on the remaining time and reception confirmation.

# 4. Conclusion

This contribution makes the following proposals:

Avoiding unnecessary transmission

Proposal 1:	For the Rx initiated approach, RAN2 to discuss whether the timer to determine the outdated RLC SDU should be maintained per RLC SDU or per Rx RLC entity.

Observation 1: The legacy RLC Status Report based window update mechanism can be reused in Tx initiated approach.

Proposal 2:	In the Tx initiated approach, Tx RLC entity updates the transmitting window when the RLC Status Report is received, as in the existing specifications.

Observation 2: The Rx initiated approach is inaccurate due to lack of exact expiry time of each RLC SDU (segment) at the Rx RLC entity.

Observation 3: The Tx initiated approach will introduce some signalling overheads to transmit a notification for outdated data.

Proposal 3:	RAN2 to adopt the combined approach, where:

- Tx side stops retransmitting an obsolete SDUs based on the discard indication from its PDCP transmitting entity.

- Rx side determines SDU(s) to be obsolete based on a locally maintained timer.

- Tx RLC entity updates the transmitting window when the RLC Status Report is received, as in the existing specifications.

Timely RLC retransmission enhancement

Observation 4: For Status Report based and polling based solutions, the fast Status Report triggering can be addressed by network configuring appropriate parameter values, e.g. t-reassembly, t-StatusProhibit, and pollByte/pollPDU.

Observation 5: In the uplink scenario, whether, when and how to trigger and transmit the RLC Status Report is up to the network implementation.

Proposal 4: RAN2 does not specify any enhanced Status Report or enhanced polling based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

Proposal 5:	RAN2 works on the autonomous RLC retransmission, where the RLC SDU (segment) can be retransmitted based on the remaining time and reception confirmation.

# 5. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#127, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_127/Inbox/Chair_Notes/R2_127%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM_cl.docx

[3] Chair note of RAN2#126, https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_126/Inbox/Chair_Notes/R2_1256%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM_rm.docx