3GPP TSG RAN WG2 #127bis                                                                                             R2-2407987

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM.

The contribution is organized as the following. In section 2, we discuss the following aspects,

- Mixture of inter-CU LTM and intra-CU LTM

- Fast LTM recovery for inter-CU LTM

- Inter-CU SCG LTM with MCG unchanged

- Inter-CU MCG LTM with SCG kept or released

- Further co-existence scenarios in DC

Our proposals are summarized in Section 3.

Discussion

Mixture of inter-CU LTM and intra-CU LTM

In last meeting [1], RAN2 agreed to introduce a new Rel-19 ID to determine whether to perform L2 reset and security key update,

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

On the presence of the new Rel-19 ID in the LTM candidate configurations, the network needs to configure also the Rel-19 ID for all LTM candidate configurations if the network configures Rel-19 ID for one LTM candidate configuration.

Proposal 1a: If the network configures Rel-19 ID for at least one LTM candidate configuration, the network configures it also for all LTM candidate configurations and the serving cell.

If the Rel-19 ID is same for the source cell and the target cell, Rel-18 NoResetID needs to be included in this LTM candidate configuration if one CU provides more than one candidate cells, then UE can know whether it is inter-DU or intra-DU case as in R18 LTM.

Therefore, it is suggested to clarify that,

Proposal 1b: If the Rel-19 ID is same for the source cell and the target cell, UE further checks the Rel-18 NoResetID if configured and follows the R18 mechanism on handling Rel-18 NoResetID.

Fast LTM recovery in inter-CU LTM

In R18 intra-CU LTM, fast recovery to a LTM candidate is supported. However, it is not straightforward to follow the same principle for inter-CU LTM. The reason is that security key update is needed if fast recovery is performed to an inter-CU LTM candidate, which is not needed in R18 fast LTM recovery. If to support fast LTM recovery in inter-CU LTM, we need to address how UE can get the security key information of the inter-CU LTM candidate when fast LTM recovery is performed. This is a similar issue as the security key update solution for the inter-CU LTM execution. Even though SA3 is working on the security key update solution for the inter-CU LTM execution, it is unpredictable whether the final solution from SA3 can meet the security key update requirement during fast inter-CU LTM recovery. For example, if SA3 go with the MAC CE-based solution (one of the candidate solutions under discussion in SA3), it cannot meet the security key update requirement during fast inter-CU LTM recovery as there is no MAC CE from NW upon fast LTM recovery. We see two options to handle the issue,

-	Option 1: Fast LTM recovery to an inter-CU LTM candidate will be supported only if the security update mechanism concluded by SA3 can support this case.

-	Option 2: Fast LTM recovery to an inter-CU LTM candidate is supported, Send LS to SA3 to ask them to take the support of fast recovery case into consideration for the security update mechanism decision.

RAN2 needs to down select between the options above.

Therefore, it is suggested to do the down selection between the options in RAN2,

Proposal 2: On whether to fast LTM recovery to an inter-CU LTM candidate, RAN2 down selects between the following options,

- Option 1: Fast LTM recovery to an inter-CU LTM candidate will be supported only if the security update mechanism concluded by SA3 can support this case

- Option 2: Send LS to SA3 to ask them to take the support of fast recovery case into consideration for the security update mechanism decision

Inter-CU MCG LTM with SCG kept or released

There may be the case where UE in PCell_0 is configured with intra-CU PCell_1 with SCG kept, inter-CU1 PCell_2 with SCG kept as well as inter-CU2 PCell_3 with SCG released, i.e., UE is configured with mixture of intra-CU MCG LTM and inter-CU MCG LTM. The UE trajectory cannot be predicted (for example, PCell_0  PCell_3  PCell_1). As per the Rel-18 principle for intra-CU MCG LTM, the candidate configuration for PCell_1 only includes MCG configuration but not include SCG configuration (i.e., UE only clean the MCG configuration, UE keeps SCG due to optional field with need M). So upon the movement from PCell_3 to PCell_1, the SCG will be released from UE but the intention of candidate PCell_1 is to keep SCG.

To solve this issue, all MCG LTM candidate configurations should include both MCG part configuration and SCG part configuration if DC is configured in the case that at least one inter-CU MCG LTM candidate exists.

Proposal 3: If at least one inter-CU MCG LTM candidate exists, all MCG LTM candidate configurations include both MCG part configuration and SCG part configuration if DC is configured.

In last RAN2 meeting, an issue was raised by a company [2] on the co-existence case of inter-CU MCG LTM with SCG unchanged and inter-CU MCG LTM with SCG release. If some inter-CU MCG LTM candidates are configured with SCG configuration kept and some candidates are configured with SCG release are configured simultaneously. It’s possible that the NW firstly triggers LTM cell switch to an LTM candidate with SCG release, and later trigger LTM cell switch to an LTM candidate with SCG configuration but the UE does not have the DC configuration by then, which results in a SCG addition procedure.

The following solutions were proposed and discussed in last meeting but not be able to conclude it yet.

- Option 1: The NW ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch.

- Option 2: The UE does not apply the SCG configuration (if any) in the target LTM candidate configuration if the NW triggers the LTM cell switch to an LTM candidate cell with SCG configuration but the UE has released DC prior to the LTM cell switch.

- Option 3: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently. I.e. inter-CU MCG LTM with SCG addition is supported.

Option 1 puts unnecessary restriction on NW configuration. Option 2 allows UE to not follow the NW configuration which is not a regular practice in the 3GPP spec. Option 3 can be supported without spec impact.

Therefore, it is suggested to go with option 3.

Proposal 4: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently.

Inter-CU SCG LTM

Signaling aspects for inter-CU SCG LTM

On the provision of the configuration for inter-CU SCG LTM, as agreed, the MN sends it to UE via MN RRCReconfiguration. In general, there are two ways to achieve this purpose, as summarized in the table below,

Table 1: Candidate solutions for the configuration of inter-CU SCG LTM

| Candidate solutions | Issues to address |
| --- | --- |
| Option 1: reuse ltm-Config-r18 | - Issue 1: UE needs to know whether a certain candidate configuration in ltm-CandidateToAddModList-r18 received from MCG is for intra-CU MCG LTM or for inter-CU SCG LTM; - Issue 2: When the UE receives the LTM cell switch command from SCG, how to determine the candidate configuration ID included in the command refers to the configuration from MCG or the intra-CU SCG configuration. - Issue 3: How to determine the reference configuration for NW if intra-CU MCG LTM is co-existed with inter-CU SCG LTM; - Issue 4: Need to further clarify whether source SN or MN allocate the candidate configuration ID; - Issue 5: Need to further clarify whether to negotiate the candidate configuration ID with MN, if it is allocated by source SN; |
| Option 2: define new IE | - Issue 1: Need to further clarify whether source SN or MN allocate the candidate configuration ID; - Issue 2: Need to further clarify whether to negotiate the candidate configuration ID with MN, if it is allocated by source SN; |

As analyzed above, Option 2 is simpler, i.e., it mainly depends on RAN3 to further clarify the negotiation of candidate configuration ID. Besides, Option 2 is also clearer from ASN.1 point of view. Therefore, it is proposed that,

Proposal 5: A new field (e.g. ltm-DC-Config-r19) is introduced to provide the configuration for inter-CU SCG LTM.

Based on our Proposal 5, another issue is how to configure the co-existence of inter-CU SCG LTM and intra-CU SCG LTM.

In SCPAC, if the candidate configuration belongs to different SN, even more than one candidate PSCell belong to the same SN, e.g., belong to the source SN for intra-SN SCPAC, the candidate configuration for both intra-SN and inter-SN PSCell(s) can only be MN format. This is because the NW does not know how UE moves between the intra/inter-SN candidate PSCell(s). For example, UE is in PSCell_0 and configured with candidate PSCell_1 (intra-SN) and candidate PSCell_2 (inter-SN). Although the candidate PSCell_1 is an intra-SN candidate cell with source PSCell_0, UE may move along the trajectory PSCell_0  PSCell_2  PSCell_1, therefore the candidate configuration for PSCell_1 should be MN format so that it can provide the MCG configuration such as radio bear or SK-counter.

When considering inter-CU SCG LTM, the similar issue also exists. Therefore, the similar principle should be followed.

Proposal 6: If at least one inter-CU SCG LTM candidate exists, all the SCG LTM candidate configurations are provided to UE via MN format.

Inter-CU SCG LTM execution

Upon the execution of SCPAC stored in MCG VarConditionalReconfig (i.e., MN format), it is captured in TS 38.331 as following,

1> if the selected subsequent CPAC candidate configuration is stored in MCG VarConditionalReconfig: 2> for each SRB/DRB in current UE configuration: - keep the associated RLC, PDCP and SDAP entities, their state variables, buffers and timers; - release all fields related to the SRB/DRB configuration except for srb-Identity, drb-Identity, and securityConfig; 2> release/clear all current dedicated radio configuration except for the following: - the MCG C-RNTI; - the AS security configurations associated with the master key and the secondary key; - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers; - the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers; - the UE variables VarConditionalReconfig and VarServingSecurityCellSetID. 2> release/clear all current common radio configuration; 2> apply the default MAC Cell Group configuration for MCG MAC and SCG MAC as specified in 9.2.2; 2> use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG; 2> apply the default L1 parameter values as specified in corresponding physical layer specifications for the MCG and SCG;

As above, the UE applies the default MAC cell group configuration for MCG and SCG, but not performs MAC reset for MCG. The intention is to avoid the potential impact on MCG transmission upon the execution of SCPAC. Since the candidate configuration for inter-CU SCG LTM also includes the MCG configuration, the UE behavior on handling of MCG configuration can take the SCPAC as baseline upon the execution of inter-CU SCG LTM.

Proposal 7: For the MCG configuration handling upon inter-CU SCG LTM execution, the MCG handling in R18 SCPAC execution procedure is taken as baseline.

Further co-existence scenarios in DC

In Rel-18, intra-CU LTM can be configured for both MCG and SCG, and it’s up to NW implementation to trigger MCG LTM or SCG LTM, but not simultaneously. When introducing Rel-19 inter-CU LTM, it was agreed that,

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Therefore, the following co-existence scenarios need to be further clarified.

- The co-existence of inter-CU SCG LTM with intra-CU MCG LTM

- The co-existence of inter-CU MCG LTM with intra-CU SCG LTM

Scenario 1 - The co-existence of intra-CU MCG LTM and inter-CU SCG LTM

When UE is configured with both inter-CU SCG LTM and intra-CU MCG LTM, there will be the following potential execution orders.

Case 1-1: Firstly perform intra-CU MCG LTM, and followed by inter-CU SCG LTM

For inter-CU SCG LTM, the complete candidate configuration includes the SCG part and MCG part contained in the MN format RRCReconfiguration,

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

The current PCell1 (after performing the intra-MCG LTM) is different with the PCell0 (before performing the intra-MCG LTM), based on which the NW prepares the inter-CU SCG LTM configuration. Hence, when UE performs inter-CU SCG LTM, UE needs to perform cell switch from PCell1 to the original PCell0. However, PCell change is not the intention of the network upon inter-CU SCG LTM execution.

Observation 1: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs intra-CU MCG LTM and then performs inter-CU SCG LTM, PCell may be unintendedly changed back to the initial PCell.

Case 1-2: Firstly perform inter-CU SCG LTM, and followed by intra-CU MCG LTM

When UE firstly performs inter-CU SCG LTM, the candidate configuration includes both the MCG configuration and SCG configuration, which means the current MCG configuration will be changed based on the complete candidate configuration. For example, the target SN may re-locate a certain RLC bearers, to MN due to load balance. However, the complete configuration for intra-CU MCG LTM is prepared based on the condition that the RLC bearers are located in SN. So the RLC bearer related configuration is not included in the intra-CU MCG LTM configuration. When the UE perform the intra-CU MCG LTM, the radio bearers will be dropped by UE, which is not the intention of the NW.

Observation 2: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs inter-CU SCG LTM, the intra-CU MCG LTM configuration may become invalid.

Scenario 2 - The co-existence of inter-CU MCG LTM and intra-CU SCG LTM

When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, there will be the following potential execution orders.

Case 2-1: Firstly perform inter-CU MCG LTM, and followed by intra-CU SCG LTM

If UE performs inter-CU MCG LTM with SCG released, there is no problem since the release of SCG will also release the intra-CU SCG LTM configuration. If UE performs inter-CU MCG LTM with SCG kept, since the inter-CU MCG LTM may lead to the re-location of RLC bearers, similar issue like case 1-2 also exists.

Observation 3: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs inter-CU MCG LTM with SCG kept, the intra-CU SCG LTM configuration may become invalid.

Case 2-2: Firstly perform intra-CU SCG LTM, and followed by inter-CU MCG LTM

Since the intra-CU SCG LTM does not change SN, there is no problem if UE further performs inter-CU MCG LTM with SCG released. If UE performs inter-CU MCG LTM with SCG kept, similar as case 1-1, it may cause PSCell change upon performing inter-CU MCG LTM, which is not the intention of the NW.

Observation 4: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs intra-CU SCG LTM and then performs inter-CU MCG LTM execution, the PSCell may be unintendedly changed back to the initial PSCell.

In summary, for both Scenario 1 and Scenario 2, the execution of one type LTM may cause some issue when followed by UE executing the other type LTM. To handle this issue, some companies proposed that UE can automatically release the invalid LTM configuration due to above co-existence. We do not prefer this way as in Rel-18, the LTM configuration can only be explicitly released by the NW. From our perspective, since the NW controls UE to perform intra or inter-CU MCG/SCG LTM, it can be up to NW implementation to ensure the validation of the LTM configuration, i.e., if above co-existence is configured, the NW can re-configure the LTM configuration if it finds the above error cases.

Proposal 8a: For the coexistence of MCG LTM configuration and SCG LTM configuration, the following scenarios are supported:

- Coexistence of intra-CU MCG LTM configuration and inter-CU SCG LTM configuration

- Coexistence of inter-CU MCG LTM configuration and intra-CU SCG LTM configuration

Proposal 8b: When LTM execution (either intra-CU or inter-CU) is performed at one CG (either MCG or SCG), It’s up to NW implementation to release or reconfigure the LTM configuration at another CG (either SCG or MCG) to ensure its validity.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs intra-CU MCG LTM and then performs inter-CU SCG LTM, PCell may be unintendedly changed back to the initial PCell.

Observation 2: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs inter-CU SCG LTM, the intra-CU MCG LTM configuration may become invalid.

Observation 3: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs inter-CU MCG LTM with SCG kept, the intra-CU SCG LTM configuration may become invalid.

Observation 4: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs intra-CU SCG LTM and then performs inter-CU MCG LTM execution, the PSCell may be unintendedly changed back to the initial PSCell.

Mixture of inter-CU LTM and intra-CU LTM

Proposal 1a: If the network configures Rel-19 ID for at least one LTM candidate configuration, the network configures it also for all LTM candidate configurations and the serving cell.

Proposal 1b: If the Rel-19 ID is same for the source cell and the target cell, UE further checks the Rel-18 NoResetID if configured and follows the R18 mechanism on handling Rel-18 NoResetID.

Fast LTM recovery in inter-CU LTM

Proposal 2: On whether to fast LTM recovery to an inter-CU LTM candidate, RAN2 down selects between the following options,

- Option 1: Fast LTM recovery to an inter-CU LTM candidate will be supported only if the security update mechanism concluded by SA3 can support this case

- Option 2: Send LS to SA3 to ask them to take the support of fast recovery case into consideration for the security update mechanism decision

Inter-CU MCG LTM with SCG kept or released

Proposal 3: If at least one inter-CU MCG LTM candidate exists, all MCG LTM candidate configurations include both MCG part configuration and SCG part configuration if DC is configured.

Proposal 4: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently.

Inter-CU SCG LTM

Proposal 5: A new field (e.g. ltm-DC-Config-r19) is introduced to provide the configuration for inter-CU SCG LTM.

Proposal 6: If at least one inter-CU SCG LTM candidate exists, all the SCG LTM candidate configurations are provided to UE via MN format.

Proposal 7: For the MCG configuration handling upon inter-CU SCG LTM execution, the MCG handling in R18 SCPAC execution procedure is taken as baseline.

Further co-existence scenarios in DC

Proposal 8a: For the coexistence of MCG LTM configuration and SCG LTM configuration, the following scenarios are supported:

- Coexistence of intra-CU MCG LTM configuration and inter-CU SCG LTM configuration

- Coexistence of inter-CU MCG LTM configuration and intra-CU SCG LTM configuration

Proposal 8b: When LTM execution (either intra-CU or inter-CU) is performed at one CG (either MCG or SCG), It’s up to NW implementation to release or reconfigure the LTM configuration at another CG (either SCG or MCG) to ensure its validity.

Reference

[1] Draft_RAN2_127_Meeting_Report, https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_127/Report

[2] R2-2406419	 Discussion on inter-CU LTM, ZTE