3GPP TSG-RAN WG2 Meeting #127bis                                                              R2-2407988

Hefei, China, Oct 14th – 18th, 2024

Source:              CATT

Title:	             L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the open issues on L1 event triggered measurement reporting.

The contribution is organized as the following. In section 2, we discuss the following aspects,

- Node to generate the event configuration

- Event evaluation procedure

- Configuration signalling aspects

- Other conditions to trigger measurement report

- Measurement report MAC CE

Our proposals are summarized in Section 3.

Discussion

Node to generate the event configuration

RAN2 has agreed that event triggered report config is provided in serving cell config. Based on this, it is straightforward that Source DU determines the event configuration of the candidates for the initial LTM cell switch, and each Candidate DU determines the event configuration for other candidates which is used when the candidate becomes the new serving cell.

Therefore, it is proposed to clarify that,

Proposal 1a: Source DU determines the event configuration of the candidates for the initial LTM cell switch.

Proposal 1b: Candidate DU determines the event configuration for other candidates which is used when the candidate becomes the new serving cell.

Event Configuration signalling aspects

For the configuration signaling, the agreements are as follows,

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

Based on the agreement above, we need further clarify the further details of the measurement resource configuration and the event triggered report configuration.

For measurement resource configuration, we need further clarify whether it is a new IE in LTM-config, or just to reuse the R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18). It seems no strong reason to define new IE, we suggest to reuse the R18 LTM CSI resource configuration signalling structure. Considering CSI-RS is also supported for event triggering L1 measurement report. Extension to include the CSI-RS is needed.

Proposal 2: RAN2 assumes R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18) is reused as the measurement resource configuration of event triggered L1 measurement reporting with extension to include CSI-RS configuration. It can be revisited if critical issue is found.

Based on the agreement that Event triggered report config is provided in serving cell config, we can further clarify that the exact position is right under CSI-MeasConfig in serving cell config. And each event triggered report config is identified by an event config id.

Proposal 3: Event triggered report configuration list is provided right under CSI-MeasConfig in serving cell configuration. Each event triggered report configuration is identified by a new id.

How to associate  one event configuration to one measure resource configuration

The next question is how to associate one event configuration to a measure resource configuration. There are following possible options,

- Option 1: define new list, each entry is identified by a new ID and associate one event configuration and one measure resource configuration.

Similar as in legacy L3 measurement, define a new list, each entry of the list is identified by a new id and each id is mapped to an index of an event configuration and an index of a resource configuration. UE performed event triggered measurement report based on the configured id.

- Option 2: Reuse R18 LTM CSI Report Config(i.e., reusing LTM-CSI-ReportConfig-r18), the index of an event triggered report config is included in the IE

If to reuse the LTM CSI Report Config (LTM-CSI-ReportConfig) for event triggering reporting, the index of an event configuration can be included in it, which will associate the measurement resource configuration and the event configuration. When the index of an event configuration is present, UE will trigger the measurement report based on the event.

One example as following:

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

<omitted>

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...

[[

ltm-Event-ConfigId-r19                     LTM-Event-ConfigId-r19

]]

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

Both options are workable.RAN2 needs to do the down selection between the options above.

Proposal 4: To associate one event configuration(s) to one measure resource configuration. The following options are for further discussion,

‐	Option 1: A new id is introduced to associate an event configuration(s) and a measure resource configuration.

‐	Option 2: The index of an event configuration is included in LTM CSI ReportConfig.

Event evaluation procedure

How to perform event evaluation in MAC

We have agreed that MAC layer handles the event evaluation. For the detailed procedure of the event evaluation, there are two options,

- Option 1: The entire event evaluation procedure is handled by MAC based on the latest L1 measure results reported by L1.

For Option 1, all the event evaluation procedure is handled by MAC based on the L1 measure results reported by L1.

- Option 2: L1 evaluates whether the latest measured result of one candidate beam fulfilled the event or not, and indicate the event evaluation instance to MAC.

For Option 2, it is similar as the manner used in Beam Failure Detection and Recovery procedure (i.e., beam failure instance indication is reported from the lower layers to the MAC entity).the event evaluation are partially performed by L1.

Since anyway lower layer should send the L1 measure results to MAC as the measurement report is carried in MAC CE, it is easy to go with option 1.

Proposal 5: L1 reports the L1 measure results to MAC and then MAC handles the whole event evaluation procedure.

How to trigger the measurement reporting

RAN2 has agreed that,

- Any beam in candidate RS configuration can be used for LTM event evaluation.

However, it is necessary to clarify how to trigger measure reporting. Someone may interpret the above agreement in a different way. e.g., even though any beam can be used for event evaluation, UE only trigger the measurement reporting when a certain number of beams of candidate cell have fulfilled the event. We understand such mechanism is not necessary as it can be seen as a kind of cell level evaluation which has been excluded last meeting. Considering the evaluation is based on beam level, any new beam fulfils its associated event during TTT should be reported to NW.

Therefore, it is proposed to clarify that,

Proposal 6: UE initiates measurement reporting when any new candidate beam fulfills its associated event during TTT.

TTT timer handling when current beam changes

It is possible that the current beam of serving cell may change via the newly indicated TCI state during the event evaluation. It is necessary to clarify the UE behavior for such case.i.e, whether UE should reset the TTT timer.

If TTT timer is reset when the current beam of serving cell changes,it will increase the difficulty of fulfilling the event and NW may not be able to get the L1 measure result timely. As a result, NW can not trigger the LTM cell switch when it is necessary. Moreover,the intention of event triggering is to send the measurement results to NW when the beam quality of serving cell become bad and/or the beam quality of candiate cell is better than the beam quality of serving cell. It does not matter which beam is the current beam of serving cell. Hence,there is no need to reset the TTT timer when the current beam of serving cell changes from one to another. Anyway the consequence of keeping the evaluation upon best beam changes is that more/early L1 measurement results report may be triggerred, it doesn’t bring negative impact on the NW, and it could provide more infomration to the NW, so we suggest to continue the TTT upon current beam of serving changes.

Therefore, it is proposed that,

Proposal 7: For event LTM2/LTM3/LTM5, TTT timer is not reset when the current beam of serving cell changes.

RS type alignment for event evaluation

We have agreed to support the beam config of both SSB and CSI-RS in L1 measurement resource configuration. However, it is not clear whether both SSB and CSI-RS are always configured for a LTM CSI Resource or one of them can be configured alone. Hence how to determine the RS type used for event evaluation is not clear either.

In L3 measurement, Either SSB or CSI-RS, or both can be included in a measure object. And RS type is included in report configuration as follows,

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

It is straightforward to follow the same principle for LTM event triggered L1 measurement,

Proposal 8a: Either SSB or CSI-RS, or both SSB and CSI-RS can be included in one L1 measurement resource configuration.

Proposal 8b: To indicate the RS type to measure, RS type is included in an event triggered report configuration.

RAN2 agreed that Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation,

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Since the RS type in a new indicated TCI state for serving cell may be changed dynamically, the RS type of serving beam and candidate beam may become not aligned at some moment, i.e., RS type of serving beam is different from the RS type indicated in the corresponding event triggered report configuration. To ensure that same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5, we need to clarify that UE performs the event evaluation only when the RS type of the current beam (i.e. a beam corresponding to the indicated TCI state) is same as the RS type indicated in the corresponding event triggered report configuration.

Proposal 8c: For event LTM3 or LTM5, UE performs the event evaluation only when the RS type of the current beam (i.e. a beam corresponding to the indicated TCI state) is same as the RS type indicated in the corresponding event triggered report configuration.

Other conditions to trigger measurement report

Measurement reporting when the leaving condition is met

In the legacy L3 measurement, UE can be configured to send the measurement report when the leaving condition is met for a cell which was reported previously. It is useful for NW to know when a cell is not good enough anymore as it will enable the NW to exclude this cell when determine a target cell for handover, similar intention as legacy L3 measurement. In L3 measurement, the report on leaving condition fulfilled is configurable. For L1 event measurement report, the same mechanism can be used.

Therefore, it is proposed that,

Proposal 9: UE can trigger the measurement report MAC CE when the leaving condition is met. It can be enabled/disabled by NW.

Periodical measurement reporting for the triggered event

In the legacy L3 measurement, reporting interval and reporting amount can be configured for event triggered measurement. UE can send the measurement results periodically according to the configured reporting amount after the event has been triggered. Similar mechanism can be supported in LTM as it is beneficial for the network to get the latest quality status/quality changing trend of the reported cells.

Therefore, it is proposed that,

Proposal 10: UE can send the MAC CE measurement report periodically after the event has been triggered, i.e., reportAmount and reportInterval can be configured optionally to UE.

Measurement report MAC CE

In last RAN2 meeting, it was agreed to use MAC CE for the measurement report,

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

We need further consider the contents of the MAC CE measurement report. There are several aspects to discuss,

- Maximum number of the reported beams

- What candidate beams to be reported

- Whether to always report the serving cell beam

- Contents of a reported beam

- UE procedure to transmit measurement report MAC CE

Maximum number of the reported beams

When the measurement report is triggered, the UE may have multiple measurement results available for multiple beams of multiple cells, but it does not necessarily to report all of them as some of them may be not useful for the network. In L3 measurement the maximum numbers of neighboring cells/beams that can be included in the measurement results are configurable. Similar limitation can be used for LTM event triggered L1 measurement reporting.

Proposal 11: The max number of candidate beams to include in the MAC CE measurement report is configured by gNB, which is to be decided by RAN1.

What candidate beams to be reported

In L3 measurement, the cells which have triggered the event condition are maintained in cellsTriggeredList for a measure id. The measure results of the cells in cellsTriggeredList can be included in one measurement report according to the limitation on the max number of non-serving cells. Same mechanism can be adopted for LTM event triggering measurement reporting. More specifically, for beams which are associated to the same event configuration, UE maintain a list of all the beams which have fulfilled the event condition. When the measurement reporting is triggered, some or all of them are included in the measurement report according to limitation on the max number of candidate beams configured by gNB.

Therefore, it is proposed that,

Proposal 12: Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the MAC CE measurement report according to limitation on the max number of candidate beams configured by gNB.

Whether to always report the serving cell beam

In L3 measurement, measure results of serving cells are always reported together with the measure results of the triggered cells. NW could make handover decision based on both the serving cell measurement results and neighbor cell measurement results. But for LTM event triggering measurement, it seems no need to make the serving cell reporting mandatory as the NW could get the L1 measurement results of beams of serving cell based on CSI-ReportConfig as in legacy L1 measurement. Hence, it can be up to network to configure whether to include it in the measurement report.

Proposal 13: Whether to include the beam of the serving cell in the measurement report MAC CE is configured by gNB.

Contents of a reported beam

When receiving a measurement report, NW needs to know some basic information of a reported beam, e.g., the beam measure result, the identity of the beam and which candidate cell it belongs to, and the identity of the triggered event. Some of the information may be reported explicitly or implicitly depending on the RRC signalling design for the event configuration and resource configuration, e.g., UE can just report the index which is used to associate an event configuration to a measure resource configuration, the NW will know associated event configuration.

Hence, the information of a reported beam in a measurement report at least includes,

‐	The beam measure results

‐	The beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information

Proposal 14: For a reported beam in the MAC CE measurement report, the information at least includes (explicitly or implicitly),

‐	Beam measure results

‐	Beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information(e.g., the event config id)

For the value range of the beam measure result, it needs inputs from RAN1 as it is PHY provides the beam measure results to MAC and then MAC include it in the MAC CE message.

Proposal 15: The value range of the beam measure result in the measurement report MAC CE is to be decided by RAN1.

UE procedure to transmit measurement report MAC CE

When the measurement reporting is triggered, UE sends the measurement report MAC CE to network. The measurement report MAC CE contained in a MAC PDU will be sent on PUSCH. Regarding how to allocation the needed PUSCH resource, Generally the legacy procedure can be reused, i.e., if UL resources is available when UE needs to transmit measurement report MAC CE,UE sends the measurement report MAC CE directly. Otherwise, UE dynamically request the needed PUSCH resources.

Proposal 16a: For the PUSCH resource allocation for transmitting measurement report MAC CE, legacy procedure is reused.

However RAN2 should discuss the priority of the L1 measurement report MAC CE and whether dedicated SR resource for the L1 measurement report MAC CE is needed.

In the current MAC spec, UL MAC CEes have different LCP priorities as follows,

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; ……

The MAC entity shall prioritize any MAC CE listed in a higher order than data from any Logical Channel. We need to discuss the proper priority for the L1 measurement report MAC CE. Considering that NW may make LTM decision based on the 1 measurement report, it may be better to send the measurement report MAC CE with a higher priority.

Proposal 16b:  Measurement report MAC CE should be transmitted with a high LCP priority. FFS the exact priority.

In legacy, dedicated scheduling request (SR) resources can be used to achieve low latency. UE should transmit the L1 measurement report MAC CE as soon as possible as that NW can trigger the LTM cell switch timely. Hence, it is beneficial to support dedicated scheduling request (SR) resources for requesting UL resource to transmit L1 measurement report MAC CE.

Proposal 16c: Dedicated scheduling request (SR) resource is supported for requesting UL resource to transmit measurement report MAC CE.

Conclusion

Based on the previous analysis in section 2, our proposals are summarized as follows:

Node to generate the event configuration

Proposal 1a: Source DU determines the event configuration of the candidates for the initial LTM cell switch.

Proposal 1b: Candidate DU determines the event configuration for other candidates which is used when the candidate becomes the new serving cell.

Event Configuration signalling aspects

Proposal 2: RAN2 assumes R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18) is reused as the measurement resource configuration of event triggered L1 measurement reporting with extension to include CSI-RS configuration. It can be revisited if critical issue is found.

Proposal 3: Event triggered report configuration list is provided right under CSI-MeasConfig in serving cell configuration. Each event triggered report configuration is identified by a new id.

Proposal 4: To associate one event configuration(s) to one measure resource configuration. The following options are for further discussion,

‐	Option 1: A new id is introduced to associate an event configuration(s) and a measure resource configuration.

‐	Option 2: The index of an event configuration is included in LTM CSI ReportConfig.

Event evaluation procedure

Proposal 5: L1 reports the L1 measure results to MAC and then MAC handles the whole event evaluation procedure.

Proposal 6: UE initiates measurement reporting when any new candidate beam fulfills its associated event during TTT.

Proposal 7: For event LTM2/LTM3/LTM5, TTT timer is not reset when the current beam of serving cell changes.

Proposal 8a: Either SSB or CSI-RS, or both SSB and CSI-RS can be included in one L1 measurement resource configuration.

Proposal 8b: To indicate the RS type to measure, RS type is included in an event triggered report configuration.

Proposal 8c: For event LTM3 or LTM5, UE performs the event evaluation only when the RS type of the current beam (i.e. a beam corresponding to the indicated TCI state) is same as the RS type indicated in the corresponding event triggered report configuration.

Other conditions to trigger measurement report

Proposal 9: UE can trigger the measurement report MAC CE when the leaving condition is met. It can be enabled/disabled by NW.

Proposal 10: UE can send the MAC CE measurement report periodically after the event has been triggered, i.e., reportAmount and reportInterval can be configured optionally to UE.

Measurement report MAC CE

Proposal 11: The max number of candidate beams to include in the MAC CE measurement report is configured by gNB, which is to be decided by RAN1.

Proposal 12: Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the MAC CE measurement report according to limitation on the max number of candidate beams configured by gNB.

Proposal 13: Whether to include the beam of the serving cell in the measurement report MAC CE is configured by gNB.

Proposal 14: For a reported beam in the MAC CE measurement report, the information at least includes (explicitly or implicitly),

‐	Beam measure results

‐	Beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information(e.g., the event config id)

Proposal 15: The value range of the beam measure result in the measurement report MAC CE is to be decided by RAN1.

Proposal 16a: For the PUSCH resource allocation for transmitting measurement report MAC CE, legacy procedure is reused.

Proposal 16b:  Measurement report MAC CE should be transmitted with a high LCP priority. FFS the exact priority.

Proposal 16c: Dedicated scheduling request (SR) resource is supported for requesting UL resource to transmit measurement report MAC CE.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- Draft_RAN2_127_Meeting_Report,https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_127/Report

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification,18.2.0