3GPP TSG-RAN WG2 Meeting #127                                                                    R2-2407990

Hefei, China, 14th - 18th October, 2024

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In the RAN2#127 meeting [1], on-demand SSB was discussed, with the following agreements:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we further discuss the remaining issues (e.g., OD-SSB transmission indication, Measurement on OD-SSB, OD-SSB measurement of neighbor cell) in section 2.Our proposals are summarized in Section 3.

Discussion

OD-SSB transmission indication

RAN2 has agreed that RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. Regarding the detailed cases to indicate the initial activation/deactivation state of OD-SSB configuration, it is straightforward to refer to the condition to indicate the initial SCell state (i.e., sCellState-r16) in legacy specification. The following are the conditions for sCellState configuration in the current specification.

| ScellAddSync | The field is optionally present, Need N: - in the masterCellGroup at - SCell addition, - reconfiguration with sync, - resume of an RRC connection. - in the secondaryCellGroup, when the SCG is not indicated as deactivated at: - SCG activation while the SCG was previously deactivated, - SCell addition, - reconfiguration with sync. It is absent otherwise. |
| --- | --- |

In our view, RRC based OD-SSB transmission indication at least support the above conditions. For the FFS on reconfiguration, similar to SCell activation/deactivation, we understand there is no need to support it as it can be indicated via MAC CE if the state of OD-SSB transmission changes when SCell is reconfigured.

Proposal 1a:  RRC based OD-SSB transmission indication is not supported for SCell reconfiguration case.

Proposal 1b: RRC based OD-SSB transmission indication support the following cases，

- SCell addition,

- reconfiguration with sync,

- resume of an RRC connection,

- SCG activation while the SCG was previously deactivated.

In last RAN2 meeting, it was agreed to introduce new MAC-CE for OD-SSB transmission indication. There is a FFS on whether we need further optimization for Scenario #2A. More specifically, the issue is whether the new MAC CE for OD-SSB transmission also indicates the activation/deactivation of SCell in Scenario #2A. We will analyze this issue from the following aspects:

- The definition of Scenario #2A

Scenario #2A refers to “when UE receives SCell activation command”. If the new MAC CE only indicates the transmission of OD-SSB, two MAC CEs (the new MAC CE for OD-SSB transmission indication and the legacy SCell Activation/Deactivation MAC CE) are sent to UE, for Scenario #2A, then UE will handle these MAC CEs one after the other, which means the activation/deactivation of OD-SSB transmission and activation/deactivation of SCell are not performed at the same time from UE point of view. This is not aligned with the definition of Scenario #2A.

Observation 1: For Scenario #2A, if the new MAC CE only indicates the transmission of OD-SSB, the activation/deactivation of OD-SSB transmission and activation/deactivation of SCell are not performed at the same time from UE point of view. This is not aligned with the definition of Scenario #2A.

- Payload size and signaling overhead

Obviously, compared to the new MAC CE that indicates both OD-SSB transmission and SCell activation/deactivation, the payload size of the new MAC CE that only indicates the transmission of OD-SSB is smaller. However, if the new MAC CE only indicates the transmission of OD-SSB, NW will also need to indicate SCell activation/deactivation via SCell activation/deactivation MAC CE unless all SCells indicating on-demand SSB transmission are in Scenario #2. Overall, the required payload is similar (bits for OD-SSB transmission + bits for SCell activation/deactivation) regardless of whether the new MAC CE indicates the activation/deactivation of SCell, and when OD-SSB transmission and SCell activation/deactivation are indicated within one MAC CE, one MAC CE signaling can be saved.

Observation 2: Signaling overhead can be reduced if OD-SSB transmission activation/deactivation and SCell activation/deactivation are indicated within one MAC CE for Scenario #2A.

- Spec impact

It is necessary to specify the behavior of UE on OD-SSB transmission after receiving the new MAC CE, regardless of whether the new MAC CE indicates the activation/deactivation of SCell. In addition, if the new MAC CE for OD-SSB transmission also indicates the activation/deactivation of SCell in Scenario #2A, an additional UE behavior for SCell activation/deactivation after receiving the new MAC CE need to be specified, similar to the UE receiving SCell Activation/Deactivation MAC CE or enhanced SCell Activation/Deactivation MAC CE.

Observation 3: If the new MAC CE for OD-SSB transmission also indicates the activation/deactivation of SCell in Scenario #2A, an additional UE behavior for SCell activation/deactivation after receiving the new MAC CE need to be specified.

According the above analyses, we slightly prefer that the new MAC CE for OD-SSB transmission can also indicate the SCell activation/deactivation in Scenario # 2A.

Proposal 3: The new MAC CE for OD-SSB transmission can also indicate the SCell activation/deactivation in Scenario # 2A.

Based on RAN1 discussion, multiple candidate values can be configured by RRC, e.g., periodicity of OD-SSB, and the applicable value can be indicated by MAC CE for OD-SSB transmission indication. Therefore, 1bit indication may not be suitable for OD-SSB transmission.

Furthermore, during RAN1#118 meeting, RAN1 sent an LS [2] to RAN2, RAN1 respectfully requests RAN2 to design MAC CE based signaling to indicate OD-SSB transmission for Scenarios #2 and #2A, and RAN1 is working on the further details needed for RRC and MAC CE based signaling to indicate OD-SSB transmission and will inform RAN2 as further agreements are made.

Observation 4: RAN1 is already working on the further details of the new MAC CE for OD-SSB transmission.

Therefore, RAN2 can wait for RAN1 further details on the other content of the new MAC CE for OD-SSB transmission.

Proposal 4: RAN2 waits RAN1 progress on the other content of the new MAC CE for OD-SSB transmission.

Measurement on OD-SSB

- Performing L3 measurement

The configuration of SMTC and ssb-ToMeasure for OD-SSB was discussed at the last RAN2 meeting. Based on the current specification, the UE is only capable of measuring SSB within SMTC. SMTC is configured in MO and is per frequency. The transmission of the OD-SSB is not periodic, but triggered by the NW. For the OD-SSB measurement of SCell, the following solutions are proposed:

- Option1: UE starts to RRM measure on OD-SSB within SMTC after receiving the new MAC CE for OD-SSB transmission.

- Option2: RRM measurement on OD-SSB is not based on SMTC. UE measures OD-SSB based on the on-demand SSB configuration and MAC CE indication.

For option1, UE measures OD-SSB within SMTC after receiving OD-SSB transmission indication. The SMTC used for the RRM measurement may be changed when the UE receives the transmission indication, e.g., change from the SMTC of always-on SSB to the SMTC of OD-SSB, or change from the SMTC with OD-SSB periodicity N1 to the SMTC with OD-SSB periodicity N2, which may have an impact on RAN4.

In addition, currently one cell can only be configured with one SMTC. If the SCell is configured with always-on SSB and OD-SSB, the legacy SMTC is not applicable to OD-SSB. Especially when multiple OD-SSB periodicities are configured, introducing one additional SMTC for OD-SSB may not be sufficient, if the purpose is to immediately measure and report the measurement result according to the periodicity of OD-SSB after receiving OD-SSB transmission indication. And if more than one additional SMTCs for OD-SSB are introduced for RRM measurement on OD-SSB, the relationship between additional SMTCs for OD-SSB and the received new MAC CE for OD-SSB transmission needs to be introduced. More specification impacts are required.

For option2, UE measures the OD-SSB within the transmission duration based on the configuration and transmission indication of the OD-SSB for each SCell, which conflicts with existing spec and may impact RAN4 as well as measurement time for the SCell is updated upon receiving MAC CE indication. Compared to option 1, option 2 does not change the existing MO configuration, and a separate measurement method is used for OD-SSB.

According the above analyses, we slightly prefer option 2.

Proposal 5: RRM measurement on OD-SSB measurement is not based on SMTC, i.e., UE measures OD-SSB based on the on-demand SSB configuration and MAC CE indication.

- L3 measurement reporting

- Scenario #2

For Case #1, UE does not expect to measure SSB when on-demand SSB transmission is deactivated. The measurement reporting configuration can be configured with legacy, and UE reports the measurement results based on the measurement configuration after the valid measurement results are available.

Observation 5: For Scenario #2 with Case #1, there is no spec impact on measurement reporting.

According to RAN4 [3], for known cells, there is a limit to the time between the measurement reporting and UE receiving the SCell activation command.

8.3.2	SCell Activation Delay Requirement for Deactivated Scell

SCell in FR1 is known if it has been meeting the following conditions:

- During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified

in clause 9.2 and 9.3.

- the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time

when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

…

For Case #2, UE can measure the always-on SSB, and UE can measure the OD-SSB after OD-SSB is activated. If the measurement reporting of always-on SSB is triggered too early, and the time between the measurement reporting and UE receiving the SCell activation command exceeds the limit, it is beneficial to quickly measure and report the measurement results after receiving the OD-SSB transmission indication, which can change the SCell from unknown cell to known cell to accelerate the activation of SCell. However, according to the existing spec, for Scenario #2 with Case #2, if the event triggered reporting is configured, UE may not trigger the measurement reporting after receiving the OD-SSB transmission indication, e.g. the measurement reporting for the cell is triggered long along and no new applicable cell is fulfilled the event triggered condition. So in order to change the SCell from unknown cell to known cell, a new trigger conditions of measurement reporting should be introduced, i.e., UE triggers a new measurement reporting after the OD-SSB is activated via MAC CE.

Proposal 6: For Scenario #2 with Case #2, a new measurement reporting is triggered after the OD-SSB is activated via MAC CE.

- Scenario #2A

Enhancement for multiple unknown SCells’ activation was discussed at RAN4. The LS [4] received from RAN4 at the RAN2 #123 bis meeting is shown below.

Enhancement for multiple FR2 unknown SCells’ activation Agreements: - Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band. - Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized TCI activation command transmitted is based on L3 measurement results or L1 measurement results? Agreements: - If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on: - the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command. - the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

Based on the LS, after receiving SCell activation command, L3 measurement reporting can be used to assistant the TCI activation command and accelerate the activation of SCell.

Observation 6: During SCell activation procedure, L3 measurement reporting can be used to assistant the TCI activation command and accelerate the activation of SCell.

The similar function for L3 measurement reporting can be reused for Scenario #2A. If the new MAC CE only indicates the transmission of OD-SSB, the existing L3 measurement reporting trigerred by SCell activation command mentioned above [5] can be reused, i.e. RRM measurement report is triggered when SCell activation command MAC CE is received. . And if the new MAC CE for OD-SSB transmission can also indicate the SCell activation/deactivation, RRM measurement report will be initiated when the new MAC CE is received.

Proposal 7: For Scenario #2A, RRM measurement reporting can also be triggered by MAC CE to accelerate the activation of SCell.

- 2.3 OD-SSB measurement of neighbor cell

According to the agreement of “the UE expects to measure SSB when on-demand SSB transmission is activated”, there is a question about whether UE can measure the OD-SSB of a neighbor cell when the OD-SSB transmission of the neighbor cell is activated. According to the WID [6], OD-SSB transmission is only applicable to SCell. Indicating the OD-SSB transmission of the neighbor cell to the UE is not supported. Therefore, OD-SSB measurement should not impact RRM measurement on neighbor cell.

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 8: OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Conclusion

According to the analysis in section 2, our observations and proposals are as follows,

Observation 1: For Scenario #2A, if the new MAC CE only indicates the transmission of OD-SSB, the activation/deactivation of OD-SSB transmission and activation/deactivation of SCell are not performed at the same time from UE point of view. This is not aligned with the definition of Scenario #2A.

Observation 2: Signaling overhead can be reduced if OD-SSB transmission activation/deactivation and SCell activation/deactivation are indicated within one MAC CE for Scenario #2A.

Observation 3: If the new MAC CE for OD-SSB transmission also indicates the activation/deactivation of SCell in Scenario #2A, an additional UE behavior for SCell activation/deactivation after receiving the new MAC CE need to be specified.

Observation 4: RAN1 is already working on the further details of the new MAC CE for OD-SSB transmission.

Observation 5: For Scenario #2 with Case #1, there is no spec impact on measurement reporting.

Observation 6: During SCell activation procedure, L3 measurement reporting can be used to assistant the TCI activation command and accelerate the activation of SCell.

OD-SSB transmission indication

Proposal 1a:  RRC based OD-SSB transmission indication is not supported for SCell reconfiguration case.

Proposal 1b: RRC based OD-SSB transmission indication support the following cases，

- SCell addition,

- reconfiguration with sync,

- resume of an RRC connection,

- SCG activation while the SCG was previously deactivated.

Proposal 3: The new MAC CE for OD-SSB transmission can also indicate the SCell activation/deactivation in Scenario # 2A.

Proposal 4: RAN2 waits RAN1 progress on the other content of the new MAC CE for OD-SSB transmission.

Measurement on OD-SSB

Proposal 5: RRM measurement on OD-SSB measurement is not based on SMTC, i.e., UE measures OD-SSB based on the on-demand SSB configuration and MAC CE indication.

Proposal 6: For Scenario #2 with Case #2, a new measurement reporting is triggered after the OD-SSB is activated via MAC CE.

Proposal 7: For Scenario #2A, RRM measurement reporting can also be triggered by MAC CE to accelerate the activation of SCell.

OD-SSB measurement of neighbor cell

Proposal 8: OD-SSB SCell operation has no impacts to RRM measurement on neighbor cell.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #RAN2 127

- R1-2407438, LS to RAN2 for on-demand SSB SCell operation, RAN1#118

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12).

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis

- R2-2313936, Introduction of FR2 SCell enhancements

- RP-242354 Revised WID: Enhancements of network energy savings for NR