3GPP TSG-RAN WG2 Meeting #127bis                                                   	R2-2408000

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on DSR enhancement

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will further discuss the issues on DSR enhancement including the remaining time design, the case of non-delay critical data ahead of delay critical data and whether to include PSI, our views are also given in this paper.

Discussion

The enhanced DSR agreements in RAN2#127 meeting report [1] are as follows, more details on the FFS issues are discussed in this section.

RAN2#127 meeting agreements: - Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

The remaining time design

According to legacy description on DSR as below, the Remaining Time in DSR MAC CE is the shortest remaining time, anyway, this shortest remaining time is naturally less than the remainingTimeThreshold which is used to trigger the DSR.

TS 38.323 Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. TS 38.321 RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. …. The fields in the DSR MAC CE are defined as follows: - LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported; - Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

In R19 enhanced DSR, the basic and original purpose to enhance DSR is to give buffer size to a remaining time with finer granularity to help network allocating UL grant, so, for delay-critical data, it is reasonable to assume that the reporting remaining time thresholds (range) in enhanced BSR is also less than the triggering threshold (i.e., agreed as legacy remainingTimeThreshold) for enhanced DSR, otherwise, if the reporting threshold for a remaining time is higher than the triggering threshold, it is not clear how the network uses this buffer size information and its non-delay critical remaining time, and consequently, the PDCP and RLC layers have to define another procedure similar to legacy triggering threshold to notice MAC the data within this higher remaining time threshold is available.

Proposal 1: The reporting remaining time thresholds need to be less than the triggering threshold in enhanced DSR to offer more granularity about the delay status.

Based on above analysis, the reporting remaining time thresholds had better to be less than the triggering threshold. Since it has been agreed that the triggering threshold is the legacy remainingTimeThreshold, then it will have no impact to legacy delay critical data definition.

Proposal 2: If the reporting remaining time thresholds are less than the trigerring threshold, and based on the triggering threshold with legacy remainingTimeThreshold as agreed, there is no impact to legacy delay critical data definition for enhance DSR.

The issue on non-delay critical data ahead of delay critical data

In RAN2#127 meeting, some companies proposed that the non-delay critical data could be ahead of delay critical data in the buffer. If the UE needs to schedule the data in buffer in order, i.e., the non-delay critical data ahead of delay critical data should be scheduled firstly than the data after it, then to let network have a correct understanding to UE buffer, below two options could be considered:

- Option 1: the buffer size of non-delay critical data could be given separately from the buffer size of delay-critical buffer size in enhanced DSR, the order of non-delay critical/delay critical buffer size could follow the order of data in buffer size, besides, the remaining time is only reflected the remaining time for delay critical data rather than the non-delay critical data.

- Option 2: the buffer size of non-delay critical data could be added with the buffer size of associated delay critical data, the total buffer size will be reported to network, the remaining time is only reflected the remaining time for delay critical data rather than the non-delay critical data.

For above two options, option 1seesmly more clearly and could give network a full picture of buffer size for different delay status, but considering that the UE will not perform the intra-LCH prioritization, which means that the UE will put the data to the resource according the their order in the buffer, so it is not necessary to let network know how many non-delay critical data before the delay-critical data in buffer queue, anyway, the UE requires the network to give UL grant to schedule delay critical data together with the non-delay critical data ahead of as soon as possible,  so option 2 is preferable, option 2 will also spend less signaling overhead compared to option 1.

Proposal 3: For non-delay critical data ahead of delay critical data, the buffer size of non-delay critical data could be added with the buffer size of delay critical data, the total buffer size is reported to network, and the associated remaining time is computed based on the remaining time of delay critical data.

A new DSR MAC CE

The legacy DSR MAC CE is as follows, it is not easy to extend legacy DSR to include buffer size information for different remaining time since the number of remaining time of each LCG may be various, it is suggested to introduce a new MAC CE to enhance DSR. For the remaining time, there are 16 candidates for legacy discardTimer in ASN.1 coding, so the maximum number of remaining time range is 16 for enhanced DSR. It is suggested to shorten the number of remaining time to 2 or 4, the finer granularity for remaining time may be not needed for scheduling.

TS 38.321 figure 6.1.3.72-1: DSR MAC CE

Proposal 4: Introduce a new MAC CE for enhanced DSR, the number of candidate remaining time for one LCG can be 2 or 4.

Whether to include PSI

For enhanced DSR, if the PSI is reported to network, the network may configure the UL grant based on different PSI information from UEs, however, UE will not perform any different handling in MAC scheduling procedure for different PSI, so the benefit of this method is limited and it is not necessary to introduce DSR with PHI information.

Proposal 5: The PSI is not included in enhanced DSR.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: The reporting remaining time thresholds need to be less than the triggering threshold in enhanced DSR to offer more granularity about the delay status.

Proposal 2: If the reporting remaining time thresholds are less than the trigerring threshold, and based on the triggering threshold with legacy remainingTimeThreshold as agreed, there is no impact to legacy delay critical data definition for enhance DSR.

Proposal 3: For non-delay critical data ahead of delay critical data, the buffer size of non-delay critical data could be added with the buffer size of delay critical data, the total buffer size is reported to network, and the associated remaining time is computed based on the remaining time of delay critical data.

Proposal 4: Introduce a new MAC CE for enhanced DSR, the number of candidate remaining time for one LCG can be 2 or 4.

Proposal 5: The PSI is not included in enhanced DSR.

Reference

- RAN2#127 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei