3GPP TSG-RAN WG2 Meeting #127bis                                                   R2-2408001

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on XR-specific RLC enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the issues on timely RLC retransmissions for XR, our views on autonomous retransmission, retransmission based on enhanced status report and retransmission based on enhanced polling are given.

Discussion

In RAN2#126 meeting, the agreements on timely retransmissions on RLC layer were as follows[1], three options were proposed for further study. More details are discussed for these options in this section.

- To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

On autonomous retransmission

In this option, RLC retransmission is not performed based on RLC status report but depends on blind retransmission. During this procedure, the RLC will retransmit the RLC SDU which may be already successfully received at Rx side, which will result in resource waste and impact on system capacity considering the data volume for XR service is indeed more than legacy service. Besides, in congested scenario, this behavior will also deteriorate the congestion.

Observation 1: RLC autonomous retransmission will reduce the radio resource usage, have impact to system capacity and deteriorate the congestion.

If still going to this way, the specific condition to trigger the blind retransmission should be defined, such as based on the remaining PSDB or the failed retransmissions number from HARQ feedback.

For the remaining PSDB based method, and the method of failed retransmissions number from HARQ feedback, considering the channel quality is variable and the XR delay requirement is stringent, it is not easy to find a suitable value to determine when is the best opportunity to perform the autonomous retransmission and keep balance between the XR QoS and other benefits, such as the capacity and radio resource usage.

Observation 2: It is difficult to determine a suitable value of the conditions to trigger the blind retransmission.

Based on above analysis, it is proposed:

Proposal 1: Not to support the RLC autonomous retransmission for R19 XR.

Retransmission based on enhanced status report

Since RAN2 agreed to focus on the enhancements for UL traffic, so the status report here is transmitted from network to UE. For network, except the polling and legacy timer-triggered way to transmit the status report, if needed, network could send the status report based on several new methods. For example, the network could evaluate the time elapsing since receiving first SDU segment or the other SDUs, the channel quality status, the congestion status to determine when to send the status report to accelerate the RLC retransmission, all of them could be considered as network implementation, thus it is not necessary to specify the enhanced method to trigger status report at network side.

Proposal 2: Enhanced status report for UL data transmission can be left to gNB implementation, no spec impact.

Retransmission based on enhanced polling

5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. <--skip--> Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Based on above procedure for polling[2], it shows that the number of PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL could be used to trigger a poll in an AMD PDU. For UL XR traffic, the value associated to above two parameters could be configured to be short one by network, which could trigger PDCP status report more quickly. The short value for pollPDU and pollByte absolutely have the same function as a timer, thus is not necessary to introduce a new timer to trigger the polling. Obviously, the overhead used by polling and the following status report are acceptable, the RLC retransmission triggered by this type of status report could give Rx more opportunity to receive the not-acknowledged RLC SDU or segment and try to receive it successfully under a limited delay budget, which also avoids the resource waste compared to the blind retransmission method without considering the latest ACK/NACK information at Rx Side.

Moreover, the condition to trigger polling could be at the moment when the data become delay critical. In this condition, most of the RLC SDU or segment with NACK indication needs be retransmitted as soon as possible, it is a reasonable condition to trigger the polling.

Proposal 3: Enhanced Polling could be used for RLC timely retransmission, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: the UE can use the delay criticality of the AMD PDU to trigger the polling for timely RLC retransmission.

Conclusion

According to the analysis in section 2, it is proposed:

Observation 1: RLC autonomous retransmission will reduce the radio resource usage, have impact to system capacity and deteriorate the congestion.

Observation 2: It is difficult to determine a suitable value of the conditions to trigger the blind retransmission.

Proposal 1: Not to support the RLC autonomous retransmission for R19 XR.

Proposal 2: Enhanced status report for UL data transmission can be left to gNB implementation, no spec impact.

Proposal 3: Enhanced Polling could be used for RLC timely retransmission, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: the UE can use the delay criticality of the AMD PDU to trigger the polling for timely RLC retransmission.

Reference

- RAN2#126 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei

- TS 38.322 V18.1.0 Radio Link Control (RLC) protocol specification