3GPP TSG-RAN WG2 Meeting #127bis										R2-2408004

Hefei, China, Oct 14th–18th, 2024

Agenda item:		8.5.2

Title: 	On-demand SSB SCell operation in connected mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The following agreements have been achieved last meeting [1]:

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this paper, we share some understanding on the remaining open issues.

- Discussion

  - Configuration of on-demand SSB

Understanding on the existing SCell Addition/Modification and activation/deactivation:

SCell can be added and modified via SCell addition/modification with the initial state set to set as “deactivated”. When a SCell is added but deactivated, UE will still measure SSB, if configured, but would not monitor PDCCH.

Figure 1. Understanding on case#1

As we agreed last meeting that For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated, the SCell addition/modification with the initial state set to “deactivated” cannot be directly apply for on demand SSB case#1. Additional indication is needed to inform UE that there is no need to perform SSB measurements.

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Figure 2: Understanding on case#2

For case#2, we understand the always on SSB can be SSB with larger periodicity for energy saving purpose while the SSB after on demand triggering would be more frequently transmitted. In this case, if UE does not measure the always on SSB, the case#2 would be the same as case#1 and NW would not be aware of the radio quality of the SCell and there would be no gain compared to case#1 while more power consumption is required for the always on SSB transmission.

However, if UE measure the always on SSB before on demand SSB triggering, NW can get the measurement report in advance and decide whether to trigger on demand SSB transmission and activate the SCell.

Thus, we see benefits if UE measure the always on SSB before on demand SSB transmission is activated for case#2.

Proposal 1: For case#2, UE should measure the always on SSB before on demand SSB transmission is activated.

Then in case#2, the SCell addition/modification with the initial state set to “deactivated” can be applied, with which UE would measure the always on SSB but not monitor the PDCCH. And the SSB transmission update is expected upon on demand SSB transmission triggering.

Observation 2: For case#2, the SCell addition/modification with initial state set to “deactivated” for the corresponding SCell can be reuse while UE needs to be aware of the SSB transmission update upon SSB transmission triggering.

Proposal 2a: For on demand SSB configuration, the existing SCell addition/modification is reused with the initial state set to “deactivated” for the corresponding SCell for both case#1 and case#2.

Proposal 2b: For case#1, an additional indication should be introduced to indicate that UE does not need to measure SSB for the corresponding SCell.

  - On-demand SSB procedure

The on demand SSB procedure can be triggered when UL or DL traffic arrives and more activate SCells are needed. Thus we understand both UE and NW trigger can be considered.

Proposal 3: The on demand SSB procedure can be triggered by NW or UE.

    - On demand SSB procedure triggered by NW

Issue1: On demand SSB activation MAC CE design

As we agreed last meeting that New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE, we understand a MAC-CE as follows can be introduced with each bit indicating the transmission of SSB for each SCell.

Figure 3: A example of on demand SSB activation MAC CE of four octets

For case#1 with Scenario#2, the above example of SSB transmission indication MAC CE would be sufficient and UE would start to measure the SSB if the bit for the corresponding SCell is set to 1.

Proposal 4a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

For case#2 with Scenario#2, two options can be considered:

- Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

Option 1: Two sets of SSB configuration via RRC with 1 bit per SCell in MAC CE to switch

- Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 4b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

--Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

--Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Issue 3: On demand SSB deactivation

There has been discussion on how to indicate the transmission of on demand SSB while we have not mentioned whether we can deactivate the on demand SSB transmission. Per our understanding, the deactivation via MAC CE should be also supported.

For case#1, UE would stop measurements on the SSB upon reception of the deactivation. For case#2, UE is informed of the SSB transmission update, e.g. periodicity update, based on the deactivation indication.

Proposal 5: On demand SSB deactivation via MAC CE should be supported:

--For case#1, UE stop SSB measurements upon reception of the deactivation.

--For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

Issue 2: Signaling optimization for Scenario#2a

For Scenario#2a, the main point is that the on demand SSB activation/deactivation and SCell activation/deactivation happens at the same time and we have agreed that We will not change legacy SCell activation/deactivation MAC CE.

For case#1 with Scenario#2a, since there is no SSB transmission before on demand SSB activation, NW would not activate the corresponding SCell directly. For case#2, since there is always on SSB, it is possible that NW can activate SCell based on the measurements of the always on SSB directly.

Proposal 6a: For case#2 with Scenario#2a, NW is allowed to activate SCell according to the measurement results performed on always-on SSB directly.

In addition, it is possible that NW would like to activate/deactivate on demand SSB transmission for one SCell while perform SCell activation/deactivation on another SCell. Another case is that NW may deactivate on demand SSB transmission and deactivate this SCell at the same time. All these cases can be considered for signaling optimization, which is common for case#1 and case#2.

Proposal 6b: For Scenario2a, the following cases can be considered for signaling optimization:

- NW activate/deactivate on demand SSB transmission for one SCell while perform SCell activation/deactivation on another SCell at the same time.

- NW deactivate on demand SSB transmission and deactivate this SCell at the same time.

- NW activate SCell according to the measurement results based on always on SSB directly without activating on demand SSB transmission for case#2.

    - On demand SSB procedure triggered by UE

From RAN2’s perspective, UE can send on demand SSB request to network via UE assistance information if we go for pure RAN2 solution without much impact in RAN1. Upon reception of the request, it is still up to network to decide whether to activate the on demand SSB transmission while the activation/deactivation procedure for NW triggered procedure can be reused.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Configuration of on-demand SSB

Observation 1: For case#1, the SCell addition/modification can be reused with additional indication to inform UE that there is no need to perform SSB measurements and PDCCH monitoring.

Proposal 1: For case#2, UE should measure the always on SSB before on demand SSB transmission is activated.

Observation 2: For case#2, the SCell addition/modification with initial state set to “deactivated” for the corresponding SCell can be reuse while UE needs to be aware of the SSB transmission update upon SSB transmission triggering.

Proposal 2a: For on demand SSB configuration, the existing SCell addition/modification is reused with the initial state set to “deactivated” for the corresponding SCell for both case#1 and case#2.

Proposal 2b: For case#1, an additional indication should be introduced to indicate that UE does not need to measure SSB for the corresponding SCell.

On-demand SSB procedure

Proposal 3: The on demand SSB procedure can be triggered by NW or UE.

Proposal 4a: For case#1, one bit per SCell should be introduced in the MAC CE to indicate the on demand SSB activation. UE starts to measure SSB upon reception of the on demand SSB activation MAC CE for a SCell if the corresponding bit is set to 1.

Proposal 4b: For case#2, two options can be considered for the SSB transmission update upon on demand SSB activation:

--Option 1: Two sets of SSB transmission per SCell are configured via RRC with one, e.g.set#1, for the always on SSB before on demand SSB activation and the other, e.g. set#2, for the SSB transmission after the activation. UE measure SSB according to set#1 after the SCell is added and switch to set#2 upon reception of the on demand SSB activation MAC CE with the corresponding bit set to 1.

--Option 2: The updated SSB configuration, e.g. the periodicity, is configured via on demand SSB activation MAC CE.

Proposal 5: On demand SSB deactivation via MAC CE should be supported:

--For case#1, UE stop SSB measurements upon reception of the deactivation.

--For case#2, UE is informed of the SSB transmission update, e.g. the periodicity update, upon reception of the deactivation.

Proposal 6a: For case#2 with Scenario#2a, NW is allowed to activate SCell according to the measurement results performed on always-on SSB directly.

Proposal 6b: For Scenario2a, the following cases can be considered for signaling optimization:

- NW activate/deactivate on demand SSB transmission for one SCell while perform SCell activation/deactivation on another SCell at the same time.

- NW deactivate on demand SSB transmission and deactivate this SCell at the same time.

- NW activate SCell according to the measurement results based on always on SSB directly without activating on demand SSB transmission for case#2.

Proposal 7: UE request the on demand SSB transmission via UE assistance information. It is up to network to decide whether to activate the on demand SSB transmission and the activation/deactivation procedure for NW triggered procedure can be reused.

- Reference

- Draft_RAN2_127_Meeting_Report_v1