3GPP TSG-RAN WG2 Meeting #127bis										R2-2408005

Hefei, China, Oct 14th–18th, 2024

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1 in idle and inactive mode

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In the latest RAN#105, the on demand SIB1 in idle and inactive mode has been turn from study phase into normative phase with case#2 only:

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this paper, we share some consideration on the remaining issues for case#2.

- Discussion

  - UL WUS configuration

The following agreements have been reached in the past for UL WUS [1-3]:

Agreements at RAN2#125bis:

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

Agreements at RAN2#126:

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

Agreements at RAN2#127:

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Firstly we need to consider what kind of information should be included in the UL WUS configuration.

- Random access related configuration

We have clarify whether to support 2-step RACH procedure for on demand SIB1 request, which will impact the content of the RACH configuration. From our perspective, we see benefits in using the 2-step RACH procedure to reduce the latency.

Proposal 1: 2-step RACH based on demand SIB1 procedure is supported.

We have agreed that the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline, but currently we have SI-RequestConfig and SI-RequestConfigRepetition. We understand that SI-RequestConfigRepetition is used for Msg1 repetition while there is no need to support Msg1 repetition as we expect UE to stay in Cell A as long as possible and only request SIB1 from NES Cell when needed thus there is no motivation to enhance the coverage of NES cell.

Proposal 2: Msg1 repetition is not supported for on demand SIB1 request for requesting to the NES Cell.

For the following parameters in SI-RequestConfig, we understand si-RequestPeriod and si-RequestResources are not needed as they are mainly used to associate the SI message with the RACH resources while for on demand SIB1 case, UE does not need to differentiate.

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}     OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

Proposal 3: si-RequestPeriod and si-RequestResources are not needed for the on demand SIB1 request.

Apart from the above two parameters, we understand the following parameters in RACH-ConfigGeneric and ssb-perRACH-Occasion are needed:

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

Apart from the above parameters, we understand the following parameters in RACH-ConfigCommon are also needed:

- totalNumberOfRA-Preambles

- CB-PreamblesPerSSB

- ra-Msg3SizeGroupA

- messagePowerOffsetGroupB

- numberOfRA-PreamblesGroupA

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Proposal 4: The followings parameter are needed for Msg1 based on demand SIB1 request and should be configured per NES cell:

- ssb-perRACH-Occasion

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles

- CB-PreamblesPerSSB

- ra-Msg3SizeGroupA

- messagePowerOffsetGroupB

- numberOfRA-PreamblesGroupA

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

If we support 2-step RACH based procedure, the following parameters from RACH-ConfigGenericTwoStepRA and RACH-ConfigCommonTwoStepRA are also needed:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-TotalNumberOfRA-Preambles

- msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB

- msgA-CB-PreamblesPerSSB-PerSharedRO

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

Proposal 5: If 2-step RACH based on demand SIB1 request procedure is supported, the following parameters are needed and should be configured per NES cell:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-TotalNumberOfRA-Preambles

- msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB

- msgA-CB-PreamblesPerSSB-PerSharedRO

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

- Triggering condition of on demand SIB1 request

The following agreements have been made before on the triggering condition of WUS transmission:

Triggering condition of WUS transmission

If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

With the above agreement, we understand the NES cells would be included as neighbour cells in the cell reselection information provided in SIB3 and SIB4 and UE would trigger on demand SIB1 procedure if the candidate target cell for cell reselection is a NES cell. With such understanding, the existing cell reselection parameters can be reused.

Proposal 6a: An NES cell should be configured as a neighbor cell in the cell reselection information provided via SIB3 or SIB4. UE trigger on demand SIB1 procedure if the candidate target cell for cell reselection is an NES cell.

Proposal 6b: The existing cell reselection parameters in SIB2/3/4 can be reused for UE to select the target NES cell to trigger on demand SIB1 request.

With above consideration, we understand the extra configuration needs to be provided for on demand SIB1 would be the random access related parameters. We prefer a clean solution thus prefer to introduce a new SIB for this.

Proposal 7: A new SIB is introduced to provide the random access related parameters for NES cell.

  - UE behavior related to OD-SIB1 request

The following agreement has been made with FFS left on the UE behavior:

Agreements at RAN2#125bis

Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements at RAN2#126

RAR monitoring for MSG1 based OD-SIB1 REQ

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

We understand for the Msg1 based on demand procedure, UE can start to acquire the requested SIB1 upon reception of RAR.For 2-step RACH based on demand procedure, UE would start to acquire the requested SIB1 upon reception of MsgB.

Proposal 8a: UE start to acquire the requested SIB1 upon reception of RAR for Msg1 based on demand procedure.

Proposal 8b: If 2-step RACH based on demand SIB1 procedure, UE start to acquire the requested SIB1 upon reception of the MsgB.

  - Access restriction for legacy UEs

The following agreements have been at RAN2#127bis [2]:

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

For option 2, it is possible to indicate that there is no SIB1 via ssb-SubcarrierOffset.

-----------------------------------------------Field description of ssb-SubcarrierOffset----------------------------------------------

ssb-SubcarrierOffset

Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). For operation with shared spectrum channel access in FR1 (see 37.213 [48]), this field corresponds to kSSB, and kSSB is obtained from  (see TS 38.211 [16], clause 7.4.3.1); the LSB of this field is used also for deriving the QCL relation between SS/PBCH blocks as specified in TS 38.213 [13], clause 4.1.

The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13].

This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13).

-----------------------------------------------Field description of ssb-SubcarrierOffset----------------------------------------------

But according to the following description in TS38.321, we understand ssb-SubcarrierOffset with value larger than 11 can be used to indicate that there is no SIB1 for FR2 but would not work for FR1. Thus option 2 only works for FR2 not FR1.

ssb-SubcarrierOffset                INTEGER (0..15)

-----------------------------------------------------------------From TS38.213------------------------------------------------------------

Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in clause 13, is present if kSSB<24 [4, TS 38.211] for FR1 or if kSSB<12 for FR2. The UE determines from MIB that a CORESET for Type0-PDCCH CSS set is not present if kSSB>23 for FR1 or if kSSB>11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.

-----------------------------------------------------------------From TS38.213------------------------------------------------------------

Observation: The ssb-SubcarrierOffset in MIB with value larger than 11 can be used to indicate no SIB1 for FR2 but cannot be used to indicate no SIB1 for FR1.

With this in mind, we understand option 1/3 can be considered for further down selection while option 2 should be excluded.

Proposal 9: Down select from the following options on handling the legacy UEs:

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

UL WUS configuration

Proposal 1: 2-step RACH based on demand SIB1 procedure is supported.

Proposal 2: Msg1 repetition is not supported for on demand SIB1 request for requesting to the NES Cell.

Proposal 3: si-RequestPeriod and si-RequestResources are not needed for the on demand SIB1 request.

Proposal 4: The followings parameter are needed for Msg1 based on demand SIB1 request and should be configured per NES cell:

- ssb-perRACH-Occasion

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- preambleTransMax

- powerRampingStep

- ra-ResponseWindow

- totalNumberOfRA-Preambles

- CB-PreamblesPerSSB

- ra-Msg3SizeGroupA

- messagePowerOffsetGroupB

- numberOfRA-PreamblesGroupA

- ra-ContentionResolutionTimer

- rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Proposal 5: If 2-step RACH based on demand SIB1 request procedure is supported, the following parameters are needed and should be configured per NES cell:

- msgA-PRACH-ConfigurationIndex

- msgA-RO-FDM

- msgA-RO-FrequencyStart

- msgA-ZeroCorrelationZoneConfig

- msgA-PreamblePowerRampingStep

- msgA-PreambleReceivedTargetPower

- msgB-ResponseWindow

- preambleTransMax

- msgA-TotalNumberOfRA-Preambles

- msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB

- msgA-CB-PreamblesPerSSB-PerSharedRO

- msgA-SSB-SharedRO-MaskIndex

- groupB-ConfiguredTwoStepRA

- msgA-PRACH-RootSequenceIndex

- msgA-TransMax

- msgA-RSRP-Threshold

- msgA-RSRP-ThresholdSSB

- msgA-SubcarrierSpacing

- msgA-RestrictedSetConfig

Proposal 6a: An NES cell should be configured as a neighbor cell in the cell reselection information provided via SIB3 or SIB4. UE trigger on demand SIB1 procedure if the candidate target cell for cell reselection is an NES cell.

Proposal 6b: The existing cell reselection parameters in SIB2/3/4 can be reused for UE to select the target NES cell to trigger on demand SIB1 request.

Proposal 7: A new SIB is introduced to provide the random access related parameters for NES cell.

UE behavior related to OD-SIB1 request

Proposal 8a: UE start to acquire the requested SIB1 upon reception of RAR for Msg1 based on demand procedure.

Proposal 8b: If 2-step RACH based on demand SIB1 procedure, UE start to acquire the requested SIB1 upon reception of the MsgB.

Access restriction for legacy UEs

Proposal 9: Down select from the following options on handling the legacy UEs:

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

- Reference

- R2-2404102 RAN2#125bis Meeting Report

- R2-2406202 RAN2#126 Meeting Report

- Draft_RAN2_127_Meeting_Report_v2