3GPP TSG-RAN WG2 Meeting #127bis									R2-2408009

Hefei, China, Oct 14th – 18th, 2024                                            Revision of R2-2406449

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. In RAN#103 meeting, the WID was further refined in [1] with the following objective for LP-WUS in connected:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2#126 meeting, the following agreements have been made for Procedures for LP-WUS in RRC_CONNECTED state [3]:

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In RAN2#127 meeting, the following agreements have been made for Procedures for LP-WUS in RRC_CONNECTED [4]:

- For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS in RRC_CONNECTED state when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not. In this case, for Option 1-1, Network couldn’t know whether UE starts the drx-onDurationTimer.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE. So that network can determine whether and when to activate or deactivate the LP-WUS monitoring in RRC_CONNECTED state, LP-WUS monitoring entry and exit condition(e.g. RSRP and optional RSRQ threshold) should be configured to UE(e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message .

To keep alignment between UE LP-WUS monitoring and gNB LP-WUS transmission, solution 2 is preferred. For light signaling load and not impact RAN1 on DCI design, MAC CE is suggested to be used for network to activate or deactivate the LP-WUS explicitly.

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

RAN1 has identified the following 3 options for the LP-WUS monitoring with C-DRX configured

About Option 1-1 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state.

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

  - This option may replace DCP functionality

In RAN2 #126 meeting, the following agreements have been made for Option 1-1:

- For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In RAN1, it is assumed that ‘Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.’

The understanding in RAN1 aligns with RAN2. Option 1-1 can be taken as the baseline for LP-WUS monitoring with C-DRX configured, and RAN2 can study the LP-WUS monitoring with C-DRX configured on top of the Option-1-1.

About Option 1-2 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-1-2: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

      - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

    - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

This option can be used to decrease the service delay, e.g. when DL data burst arrives outside the C-DRX active time, LP-WUS can trigger the UE to enter the C-DRX active time and monitor PDCCH for DL scheduling.

Option1-2 has been agreed in RAN1 as below

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

While we agreed in RAN2 that:

- For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Basically, the understanding in RAN1 aligns with RAN2.

And RAN1 reached the following working assumption in RAN1#118:

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UENote: Above can be revisited considering RAN2 decisions.

From RAN2’s perspective, the understanding on option 1-1 and 1-2 aligns with RAN1 and we would like to confirm the above working assumption in RAN2.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

In RAN2#117 meeting, it has been agreed that “For option 1-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) “.

In RAN2#127 meeting, the following three options have been provided:

- Option 1: legacy drx-InactivityTimer is started after LP-WUS triggers the UE to perform PDCCH.

- Option 2: legacy drx-onDurationTimer is started after LP-WUS triggers the UE to perform PDCCH monitoring.

- Option 3: a new defined timer is started after LP-WUS triggers the UE to perform PDCCH.

Considering that in the current specification, the legacy drx-InactivityTimer is started only when PDCCH indicates a new transmission, but when UE starts the timer after LP-WUS triggers the UE to perform PDCCH monitoring, PDCCH has not been received yet.

And considering the PDCCH search space configuration(e.g. monitoringSlotPeriodicityAndOffset), there may be a time GAP between the timer starting and PDCCH received, which may be several milliseconds or several seconds, but the legacy drx-InactivityTimer can be set to a small value(e.g. 0ms, 1ms etc), the value of drx-InactivityTimer  configured may be not backward compatible if the legacy drx-onDurationTimer is used as the timer to be started after LP-WUS triggers the UE to perform PDCCH monitoring.

Observation 1: The legacy drx-InactivityTimer is not suitable to be used as the timer that is started after LP-WUS triggers the UE to perform PDCCH.

Since the timer started after LP-WUS triggers the UE to perform PDCCH monitoring is to define the UE PDCCH monitoring behavior, we understand the UE behavior while the timer running is similar as the legacy drx-onDurationTimer, either legacy drx-onDurationTimer or a new defined timer is feasible. But considering the jitter of data burst arriving time and/or quasi-periodic traffic, data burst may not come or may not come immediately after the legacy drx-onDurationTimer start, and some wait time may be necessary, the legacy drx-onDurationTimer should be set to relatively large value; but when LP-WUS received, data burst will come definitively. So the timer to be started by LP-WUS triggering can be shorter than the legacy drx-onDurationTimer. For flexibility, a newly defined timer with similar name as drx-onDurationTimer (e.g. drx-onDurationTimer-r19) is started after LP-WUS triggers the UE to perform PDCCH monitoring.

Observation 2: Either legacy drx-onDurationTimer or a new defined timer is feasible to be used as the timer started after LP-WUS triggers the UE to perform PDCCH, but a new timer is more flexible.

Proposal 3a: For Option 1-2, a newly defined timer (e.g. drx-onDurationTimer-r19) is started after LP-WUS triggers the UE to perform PDCCH monitoring.

Considering that the LP-WUS can be used in RRC_CONNECTED state only when C-DRX is configured, and  C-DRX parameters are configured per DRX Group, the LP-WUS parameter(.e.g. the newly defined timer) should also be configured pder DRX group.

Proposal 3b: The LP-WUS parameter (e.g. the newly defined timer) is configured per DRX group..

While the newly defined timer (e.g. drx-onDurationTimer-r19) is running, UE should monitor PDCCH as legacy: UE starts drx-InactivityTimer when PDCCH indicates a new transmission, UE start drx Retransmission Timer when detects a transmission failure, etc. And UE can transmit periodic SRS and semi-persistent SRS, report semi-persistent CSI configured on PUSCH etc.

In summary, while the newly defined timer (e.g. drx-onDurationTimer-r19) is running, UE should follow the UE’s legacy behavior in active time.

Proposal 4: While the newly defined timer (e.g. drx-onDurationTimer-r19) is running, UE should follow the UE’s legacy behavior in active time: UE monitors PDCCH, starts drx-InactivityTimer when PDCCH indicates a new transmission, starts drx Retransmission Timer when detects a transmission failure, transmits periodic SRS and semi-persistent SRS, reports semi-persistent CSI configured on PUSCH etc.

Thus, the active time definition can be updated as follows to cover the UE behavior when the newly defined timer (e.g. drx-onDurationTimer-r19) is running:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer, drx-InactivityTimer, drx-onDurationTimer-r19 configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

With the updated definition of active time, UE will follow the legacy UE’s behavior in active time when the newly defined timer (e.g. drx-onDurationTimer-r19) is running, and no other UE behavior should be defined when the newly defined timer (e.g. drx-onDurationTimer-r19) is running.

Considering that legacy, in case DCP is monitored but not detected, the legacy drx-onDurationTimer is not started and UE does not enter into active time. Same mechanism can be reused for LP-WUS in RRC_CONNECTED, e.g. If LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started and the UE does not enter into active time. .

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

For Option 1-2, as ‘UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)’ in RAN1, UE can determine the LP-WUS monitoring occasion using the similar formula as that to start legacy drx-onDurationTimer. Considering that LP-WUS monitoring consumes much less UE power than PDCCH monitoring, and smaller LP-WUS cycle can decrease service delay, one LP-WUS cycle can be configured, which is less than the C-DRX cycle(e.g. less than the long DRX cycle and the short DRX cycle, if configured) and not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which should be less than the C-DRX cycle, and does not differentiate long LP-WUS cycle and short LP-WUS cycle.

In Rel-18, non-integer C-DRX cycle is defined to match the non-integer traffic periodicity (e.g. XR periodicity), considering the low power consumption, small granularity of LP-WUS cycle (e.g. value of 1ms) can be used to match the non-integer C-DRX periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

In RAN2#117 meeting, it has been agreed that “For Option 1-2,The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring.”, that means that a timeOffset should be defined for the GAP between LP-WUS monitoring and the start of newly defined timer (e.g. drx-onDurationTimer-r19). Since legacy drx-startOffset is defined to determine the PDCCH monitoring start occasion, drx-startOffset - timeOffset can be used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion.

Proposal 7: RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. drx-onDurationTimer-r19).

Similar as the drx-onDurationTimer start occasion, For Option 1-2, the LP-WUS monitoring occasion can be determined with the LP-WUS cycle and LP-WUS startoffset(e.g. drx-startOffset - timeOffset) as follows: if LP-WUS is configured and activated, the UE monitors the LP-WUS at the SFN and subframe determined by:

[(SFN × 10) + subframe number -( drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) =0:

Proposal 8: the LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

With this method, if the length of the newly defined timer (e.g. drx-onDurationTimer-r19) is set to the same value as the legacy drx-onDurationTimer, and the DRX cycle is a multiple of the LP-WUS-cycle, then Option 1-2 can cover the option 1-1 for PDCCH monitoring; and if the length of the newly defined timer (e.g. drx-onDurationTimer-r19) is set to the same value as the legacy drx-onDurationTimer, and LP-WUS-cycle is set to the same value as the DRX cycle, then option 1-2 is the same as option 1-1. .

Proposal 9: With proposal 8, option 1-1 can be considered a special case of option 1-2 with the LP-WUS-cycle set to the same value as the DRX cycle.

About Option 1-3 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

This option can be used to decrease the UE power when intermittent DL data burst arrives at gNB, e.g. in the C-DRX active time, only if LP-WUS is detected, UE monitors PDCCH. Since LP-WUS consumes little UE power, a small C-DRX cycle, large drx-OndurationTimer and/or large drx-inactiveTimer can be configured to keep the UE always in the DRX active time. This solution can also reduce the UE power consumption, but the following issue should be considered: Whether the LP-WUS will be monitored when the HARQ timer is running. If LP-WUS will be monitored when the HARQ timer is running, the UE performance should be evaluated, e.g. whether UE can monitor LP-WUS and perform main radio Tx/Rx simultaneously; if LP-WUS will be monitored only after the HARQ timer expires, the LP-WUS monitoring may lead PDCCH monitoring delay, and further lead traffic transmission delay, which depends on the RAN1 design for LP-WUS, e.g. the time GAP between LP-WUS monitoring and PDCCH monitoring.

Observation 3: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 10: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- Conclusion

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

Observation 1: The legacy drx-InactivityTimer is not suitable to be used as the timer that is started after LP-WUS triggers the UE to perform PDCCH.

Observation 2: Either legacy drx-onDurationTimer or a new defined timer is feasible to be used as the timer started after LP-WUS triggers the UE to perform PDCCH, but a new timer is more flexible.

Proposal 3a: For Option 1-2, a newly defined timer (e.g. drx-onDurationTimer-r19) is started after LP-WUS triggers the UE to perform PDCCH monitoring.

Proposal 3b: The LP-WUS parameter (e.g. the newly defined timer) is configured per DRX group..

Proposal 4: While the newly defined timer (e.g. drx-onDurationTimer-r19) is running, UE should follow the UE’s legacy behavior in active time: UE monitors PDCCH, starts drx-InactivityTimer when PDCCH indicates a new transmission, starts drx Retransmission Timer when detects a transmission failure, transmits periodic SRS and semi-persistent SRS, reports semi-persistent CSI configured on PUSCH etc.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which should be less than the C-DRX cycle, and does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 7: RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. drx-onDurationTimer-r19).

Proposal 8: the LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

Proposal 9: With proposal 8, option 1-1 can be considered a special case of option 1-2 with the LP-WUS-cycle set to the same value as the DRX cycle.

Observation 3: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 10: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, RAN#103, March, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_RAN2_127_Meeting_Report_v1

- Annex

RAN1#116 meeting agreements:

R1-2401698 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded. Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one R1-2401836 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1#116bis meeting agreements:

R1-2403517 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) R1-2403713 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-1-2: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

RAN1#117 meeting agreements:

R1-2405471 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR R1-2405592 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for Mos - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured R1-2405659 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN1#118 meeting agreements:

R1-2407301 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) R1-2407378 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption - From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE - Note: Above can be revisited considering RAN2 decisions. - R1-2407516 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) - Agreement - Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network - Agreement - LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)