3GPP TSG-RAN WG2 Meeting #127bis                                                           R2-2408015

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.8.2

Source:	vivo

Title:	Discussion on Cell Bar Control for DL Coverage Enhancement

Document for:	Discussion and Decision

- Introduction

In RAN2#127, the impacts on SSB periodicity extension and the cell-barred issue for DL coverage were discussed, and the following agreements were made correspondingly [1].

RAN2#127 Agreements: - From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects). - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

The decision on RAN2 impacts on SSB extension depends on RAN1 progress, i.e., whether to extend the SSB periodicity larger than 160ms. Regarding the configuration/impacts of the DTX/DRX, even if RAN1 response LS [2] is received, no information can help RAN2 to progress the discussion. For the cell bar control, it is an RAN2 issue and the solution is still FFS for now.

In the contribution, we focus on the cell bar control issue for DL coverage enhancement.

- Discussion

The SSB periodicity may be extended to support the dynamic power sharing in the NR-NTN network. However, extending the SSB periodicity may have some impacts on legacy UEs or R19 UEs not supporting the feature:

- legacy UE may not understand the extended SMTC/gap configuration and the extended SSB periodicity configuration;

- legacy UEs and R19 UEs not supporting the SSB extension may continuously perform RLM even if there is no SSB transmission, which may result in frequent RLF.

- Legacy UEs and R19 UEs not supporting the SSB extension may monitor all PDCCH monitoring occasions in each paging occasion, even though there is no paging message due to the extended SSB periodicity.

Observation: Legacy UE and R19 UE not supporting the SSB periodicity extension cannot work well in the cell with SSB periodicity extension.

To avoid such an issue, the legacy UEs and the R19 UEs not supporting the SSB periodicity extension should not be allowed to access the cell with SSB periodicity extension via the cell barred mechanism, i.e., the network with SSB periodicity extension can use the legacy cell barred indication (cellBarredNTN is set to barred) to bar the legacy UEs and the R19 UEs not supporting the SSB periodicity extension.

However, for the R19 UEs supporting the SSB extension, it is still unclear whether the cell with SSB extension can be accessed or not. Thus a new R19 cell barred indication for SSB extension should be introduced. A cell with SSB periodicity extension always includes the R19 cell barred indication, the R19 UEs supporting the SSB periodicity should ignore the legacy cell barred indication and decide whether the cell can be accessed based on the value of the R19 cell barred indication.

The following are summarized UE behaviours when accessing an NTN cell with SSB extension.

| Cell access for NTN | cellBarred | cellBarredNTN=barred | R19 cellBarred for SSB extension is present |
| --- | --- | --- | --- |
| Legacy NTN-capable UEs | Ignore | access is not allowed; | N/A |
| R19 NTN-capable UEs not supporting the SSB periodicity | Ignore | access is not allowed; | N/A |
| R19 NTN-capable UEs supporting the SSB periodicity | Ignore | Ignore | If set to barred, UE is not allowed to access; If set to notBarred, UE is allowed to access. |

Proposal 1: Introduce a R19 cell barred indication for SSB periodicity extension in SIB1 if RAN1 agrees to extend the SSB periodicity.

Proposal 2: R19 UEs not supporting the SSB extension can be barred from accessing the NTN cells with SSB periodicity extension by the legacy NTN cell barred indication.

Proposal 3: For R19 UEs supporting the SSB periodicity extension, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the R19 cell barred indication is not present;

- consider the cell is not allowed for access if the R19 cell barred indication is set to “barred”;

- consider the cell is allowed for access if the R19 cell barred indication is set to “not barred”.

- Conclusion

In this contribution, we have further discussed the cell bar control for DL coverage enhancement due to SSB periodicity extension (if RAN1 agrees to do that). And we make the following proposals:

Observation: Legacy UE and R19 UE not supporting the SSB periodicity extension cannot work well in the cell with SSB periodicity extension.

Proposal 1: Introduce a R19 cell barred indication for SSB periodicity extension in SIB1 if RAN1 agrees to extend the SSB periodicity.

Proposal 2: R19 UEs not supporting the SSB extension can be barred from accessing the NTN cells with SSB periodicity extension by the legacy NTN cell barred indication.

Proposal 3: For R19 UEs supporting the SSB periodicity extension, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the R19 cell barred indication is not present;

- consider the cell is not allowed for access if the R19 cell barred indication is set to “barred”;

- consider the cell is allowed for access if the R19 cell barred indication is set to “not barred”.

- Reference

[1] Draft_R2_127_Meeting_Report_v1.

[2] R1-2407538, Reply LS on DL coverage enhancements.