3GPP TSG-RAN WG2 Meeting #127bis                                                              R2-2408016

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.8.4

Source:	vivo

Title:	Further Discussion on MBS Broadcast Provision in NTN

Document for:	Discussion and Decision

- Introduction

In the previous RAN2 meeting, there was a lot of progress on MBS broadcast provision within the intended service area. The achieved agreements are listed below for reference [1]:

RAN2#127 Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions. - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2). - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

In this contribution, we would like to further discuss the detailed solutions to MBS broadcast provision in NTN from network signalling and UE behavior perspectives.

- Discussion

  - 2.1	Network signalling

In the previous meeting, a warm discussion was launched to discuss whether SIB20 or broadcast MCCH to the service area information. Unfortunately, no decision was made for the siganlling details. In our understanding, the existing broadcast MCCH is still the best choice. In the following, we compare the SIB20 and MCCH solutions.

SIB20 is mainly introduced for MCCH scheduling. So, the main argument for SIB20 carrying is to facilitate the UE to skip (re)acquiring up-to-date MCCH if it is not in any intended service area of its interested broadcast services. First, the network has to associate the TMGI (can be 50-bit overhead based on the ASN.1 siganlling below) and service area on a per service basis in the SIB20 so that the UE can know the service area is applicable to which broadcast service. Without a doubt, duplicated indicating the TMGI in both SIB20 and MCCH leads to a larger overhead.

TMGI-r17 ::=                     SEQUENCE {

plmn-Id-r17                      CHOICE {

plmn-Index                       INTEGER (1..maxPLMN),

explicitValue                    PLMN-Identity

},

serviceId-r17                    OCTET STRING (SIZE (3))

What’s worse, we don’t think the UE implementation can skip to (re)acquire up-to-date MCCH even if it is not in any intended service area of its interested broadcast services. This is because the NW may provide broadcast sessions associated with a service area and some other broadcast sessions that are not associated with the service area within the current serving cell. Then, the UE anyway has to check the MCCH to figure out whether the service (not associated with the service area) that the user is interested in is provided or not in the MCCH. In other words, even though the UE knows the intended service area of all the services it is interested in, it still has to check the MCCH for the broadcast PTM configuration addition/update of the service that is not associated with the service area. Meanwhile, the change of service area may also lead to system information change notifications, resulting in unnecessary SIB1 reacquisition regarding the UEs without broadcast service interest or broadcast capability.

There might be one more issue for SIB20 carrying. SIB20 is updated in the SI window for SIB20 in the next available SI modification period while the broadcast MCCH is updated in the next avail MCCH modification period. It is hard for the network to allocate those two occasions for SIB20 update and MCCH update together in the time domain, as shown in Figure 1. As a result, when the NW decides to change the service area information and the PTM configuration for a broadcast service, the NW basically has to wait for MCCH update and SIB20 until MTCH scheduling. This is to guarantee the UE always has obtained the latest SIB20 for service area information and MCCH for broadcast PTM configuration, at the cost of network scheduling flexibility. Otherwise, if the NW schedules the MTCH before the SIB20 update, the UE may use the wrong area information to receive the broadcast service.

Figure 1: MTCH scheduling after updated MCCH and updated SIB20

For MCCH, as TMGI is already in the broadcast PTM configuration, no additional TMGI indication is needed. We can use the nonCriticalExtension field to carry the service area information for broadcast reception and service continuity, respectively. Then, the first entry of the area information list is associated with the first entry of the MBS session information list. Further, similar to the TN coverage information solution, we can use the area ID(s) to associate the broadcast session and the actual service area, especially in the case where multiple broadcast services have the same intended area. An example of ASN.1 siganlling is given below.

Proposal 1: MCCH (i.e. MBSBroadcastConfiguration) is used to include intended service area information.

Proposal 2: Service area ID is used for broadcast service and intended service area association.

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v19xy-IEs                                                           OPTIONAL

}

MBSBroadcastConfiguration-v19xy-IEs ::= SEQUENCE {

mbs-ServiceAreaInfoList-r19         MBS-ServiceAreaInfoList-r19         OPTIONAL,   -- Need R

mbs-SessionAreaList-r19      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionAreaIdList-r19                                         OPTIONAL,   -- Need R

nonCriticalExtension             SEQUENCE {}             OPTIONAL

}

MBS-SessionAreaIdList-r19                   SEQUENCE (SIZE (1..maxSession-AreaInfo-r19)) OF MBS-AreaId-r19

MBS-SessionAreaInfoList-r19        ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionAreaInfo-r19

MBS-ServiceAreaInfoList-r19 ::=      SEQUENCE (SIZE (1..maxNrofMBSArea-r19) OF MBS-ServiceAreaInfo-r19

MBS-ServiceAreaInfo-r19 ::=       SEQUENCE {

mbs-AreaId-r19                 MBS-AreaId-r19,

referenceLocation-r19       	ReferenceLocation-r17,

distanceRadius-r19          	INTEGER(0..65535)

}

MBS-AreaId-r19 ::=  INTEGER (1.. maxNrofMBSArea-r19)

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

  - 2.2	UE behavior

With the network siganlling as mentioned above, UE can interpret the intended service area for the MBS broadcast session. Then if the UE is located within the service area of a broadcast session that the user is interested in, broadcast PTM configuration can be applied for broadcast reception, as agreed. We intuitively think this logic also applies to broadcast frequency prioritization and MBS Interest Indication.

In TS 38.304, the following frequency prioritization behavior is specified for MBS broadcast,

5.2.4.1 Reselection priorities handling -------------------------<the unrelated part is omitted>----------------------- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell. -------------------------<the unrelated part is omitted>-----------------------

We think the UE may set one frequency associated with broadcast service provision with the highest frequency priority only when it will be in the intended service area after the cell reselection. Specifically, if UE is receiving or interested in receiving an MBS broadcast service associated with the intended service area and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session when its location is within the intended service area of the corresponding session after cell reselection.

Proposal 3: Frequency prioritization may be performed only when the UE is located within that service area of the broadcast service of its interested broadcast services.

In addition, in TS 38.331, the following MBS Interest Indication reporting behavior is specified for MBS broadcast,

5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

Similarly, as MBS Interest Indication (MII) is used for subsequent broadcast reception after HO, the CONNECTED UE may only report it to the network when it is inside the intended service area. Then the NW can choose a suitable cell providing the broadcast service for this UE. On opposite, if the UE is outside the intended service area, even SIB21 or nonServingCellMII is indicated by the NW, the UE doesn't need to indicate the MII since the UE anyway should not receive the broadcast service. From the network point of view, no broadcast-specific HO strategy is needed. Thus we propose that,

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

3 Conclusion

In this contribution, we have further discussed network siganlling and UE broadcast reception behavior when MBS broadcast service is provided within the intended service area. And we make the following proposals:

Network signalling:

Proposal 1: MCCH (i.e. MBSBroadcastConfiguration) is used to include intended service area information.

Proposal 2: Service area ID is used for broadcast service and intended service area association.

UE behavior:

Proposal 3: Frequency prioritization may be performed only when the UE is located within that service area of the broadcast service of its interested broadcast services.

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

4 Reference

[1] Draft_R2_127_Meeting_Report_v1.