3GPP TSG-RAN WG2 Meeting #127bis										    R2-2408023

Hefei, China, Oct 14th – 18th, 2024

Source: 	Xiaomi

Title:  	Clarification on the maximum number of SI for Msg3-based on-demand SI request

Agenda Item:	6.1.3

Document for:	Discussion and Decision

# Introduction

According to the current RRC specification, the maximum number of SI messages can be 64. However, the maximum number of bits in the Msg3-based SI request message is 32. In this contribution, we discuss how the gNB or the UE handles the Msg3-based SI request message when the maximum number of SI messages is more than 32.

# Discussion

## Msg3-based on-demand SI request

38.331: si-SchedulingInfo SI-SchedulingInfo OPTIONAL, -- Need R SI-SchedulingInfo ::= SEQUENCE { schedulingInfoList SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo, si-WindowLength ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280, s2560-v1710, s5120-v1710 }, si-RequestConfig SI-RequestConfig OPTIONAL, -- Cond MSG-1 si-RequestConfigSUL SI-RequestConfig OPTIONAL, -- Cond SUL-MSG-1 systemInformationAreaID BIT STRING (SIZE (24)) OPTIONAL, -- Need R ... } si-SchedulingInfo-v1700 SI-SchedulingInfo-v1700 OPTIONAL, -- Need R SI-SchedulingInfo-v1700 ::= SEQUENCE { schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17, dummy SI-RequestConfig OPTIONAL } SchedulingInfo2-r17 ::= SEQUENCE { si-BroadcastStatus-r17 ENUMERATED {broadcasting, notBroadcasting}, si-WindowPosition-r17 INTEGER (1..256), si-Periodicity-r17 ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo-r17 SIB-Mapping-v1700 } SIB-TypeInfo-v1700 ::= SEQUENCE { sibType-r17 CHOICE { type1-r17 ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20, sibType21, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1,...}, type2-r17 SEQUENCE { posSibType-r17 ENUMERATED {posSibType1-9, posSibType1-10, posSibType2-24, posSibType2-25, posSibType6-4, posSibType6-5, posSibType6-6, posSibType2-17a-v1770, posSibType2-18a-v1770, posSibType2-20a-v1770, spare6, spare5, spare4, spare3, spare2, spare1,...}, encrypted-r17 ENUMERATED { true } OPTIONAL, -- Need R gnss-id-r17 GNSS-ID-r16 OPTIONAL, -- Need R sbas-id-r17 SBAS-ID-r16 OPTIONAL -- Cond GNSS-ID-SBAS } }, valueTag-r17 INTEGER (0..31) OPTIONAL, -- Cond NonPosSIB areaScope-r17 ENUMERATED {true} OPTIONAL -- Need S } maxSI-Message INTEGER::= 32 -- Maximum number of SI messages

In Rel-17, RAN2 introduced a new list of SI messages, which is configured by si-SchedulingInfo-v1700. Then the overall SI messages which can be provided by a cell include the SI messages configured by both si-SchedulingInfo and si-SchedulingInfo-v1700 in SIB1. As either si-SchedulingInfo or si-SchedulingInfo-v1700 can provide maximum 32 (i.e. maxSI-Message) SI messages, the maximum number of SI messages which can be provided by a cell is 64.

Observation 1: When si-SchedulingInfo-v1700 is configured, the maximum number of SI messages which can be provided by a cell is 64.

38.331: RRCSystemInfoRequest-IEs ::= SEQUENCE { requested-SI-List BIT STRING (SIZE (maxSI-Message)), --32bits spare BIT STRING (SIZE (12)) } RRC-PosSystemInfoRequest-r16-IEs ::= SEQUENCE { requestedPosSI-List BIT STRING (SIZE (maxSI-Message)), --32bits spare BIT STRING (SIZE (11)) } maxSI-Message INTEGER::= 32 -- Maximum number of SI messages

Figure 1: Maximum number of SI messages and maximum number of bits requested-SI-List

According to the Msg3-based on-demand SI request procedure, the UE can send requested-SI-List (or requestedPosSI-List) to request one or more on-demand SI message(s) by using a concatenated SI message list containing the SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700. However, as requested-SI-List has maximum 32 bits in which each bit corresponds to an entry in the concatenated SI message list, the UE behaviours on how to set the bits in requested-SI-List would be unclear when the concatenated SI message list has more than 32 entries.

Observation 2: The maximum number of bits for requested-SI-List is 32.

Observation 3: The UE behaviours on how to set the bits in requested-SI-List would be unclear when the concatenated SI message list used by requested-SI-List has more than 32 entries.

According to the above observations, we think that we could have the following options to clarify the UE behaviours or the network configuration:

- Option 1: The network ensures that the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is 32.

- Option 2: If the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is more than 32, the UE chooses the first 32 entries in the list of concatenated SI messages used by requested-SI-List.

Option 1 has no impacts on the UE, but would introduce restrictions on the gNB implementation so that the gNB can only configured maximum 32 SI messages when si-SchedulingInfo-v1700 is present and when the Msg3-based on-demand SI request is enabled.

For Option 2, it is unclear whether the legacy UE has already be implemented as the UE behaviours described in Option 2. If the legacy UE is implemented differently from Option 2, we would require extra UE capability bit to differentiate the UE behaviours of Option 2 from the legacy UE. The benefit of Option 2 is that the network would have more flexibility of providing more SI messages when si-SchedulingInfo-v1700 is present and when the Msg3-based on-demand SI request is enabled.

As both Option 1 and Option 2 would impact the legacy behaviours of either the gNB or the UE, we think that RAN2 can discuss which Option can be adopted so that the impacts on the legacy gNB/UE can be avoided/minimized.

Proposal: RAN2 is kindly requested to discuss the following options for the Msg3-based on-demand SI request:

- Option 1: The network ensures that the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is 32.

- Option 2: If the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is more than 32, the UE chooses the first 32 entries in the list of concatenated SI messages used by requested-SI-List.

# Conclusion

According to the above analysis, we have the following observations and proposal:

Observation 1: When si-SchedulingInfo-v1700 is configured, the maximum number of SI messages which can be provided by a cell is 64.

Observation 2: The maximum number of bits for requested-SI-List is 32.

Observation 3: The UE behaviours on how to set the bits in requested-SI-List would be unclear when the concatenated SI message list used by requested-SI-List has more than 32 entries.

Proposal: RAN2 is kindly requested to discuss the following options for the Msg3-based on-demand SI request:

- Option 1: The network ensures that the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is 32.

- Option 2: If the maximum number of the concatenated SI messages provided by both si-SchedulingInfo and si-SchedulingInfo-v1700 is more than 32, the UE chooses the first 32 entries in the list of concatenated SI messages used by requested-SI-List.

# Reference

[1] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.