3GPP TSG-RAN WG2 Meeting #127bis	R2-2408031

# Hefei, China, 14 – 18 October 2024

Agenda item:		8.1.5

Source:	Xiaomi

Title:	AI/ML model transfer

Document for:		Discussion and Decision

Introduction

Following solutions and corresponding pros and cons were captured in TR 38.843 clause 7.2.1.4:

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

In this contribution, we discuss AI/ML model transfer methods.

Discussion

Model transfer options to be discussed in RAN2

Training data termination point for each use case is summarized in Table 1 below according to TR 38.843. Only UE sided model is considered since model transfer between UE and network is the focus of current work (i.e. model transfer between network entities, e.g. from OAM to gNB, is not considered). In addition, CSI compression is not considered since it is kept as a study objective in Rel-19, according to updated WID RP-242399 [3].

Table 1: Training data termination point for UE sided models

| Use case | UE (no 3GPP impact) | gNB (1a/1b) | LMF (3a/3b) | CN (2a/2b) | OAM (4b) | UE side OTT server (4a) |
| --- | --- | --- | --- | --- | --- | --- |
| CSI prediction – UE sided | Yes | FFS2 |  | FFS1 | FFS1 | Yes |
| Beam management – UE sided | Yes | FFS2 |  | FFS1 | FFS1 | Yes |
| Positioning – UE sided | Yes |  | FFS2 | FFS1 | FFS1 | Yes |

Note 1: out of scope of RAN2.

Note 2: depends on RAN1 progress.

According to TR 38.843, solution 2a/2b/4b are out of scope of RAN2, while solution 1a/1b/3a/3b depend on RAN1 progress. Solution 2a/2b/4b are also related to UE side data collection options 1b/2/3, on which RAN/RAN2 is waiting for feedback from SA WGs after sending LS RP-242389 [4]. It is proposed that RAN2 will not discuss solution 2a/2b/4b before receiving feedback from SA WGs on UE side data collection options in RAN#106.

Proposal 1: RAN2 will not discuss solution 2a/2b/4b before receiving feedback from SA TSG/WGs on UE side data collection options in RAN#106.

Solution 1a/1b/3a/3b depend on RAN1 progress. It should be noted that in RAN1 LS R1-2308730, RAN1 states that “RAN1 did not reply on the different NW entities for training (gNB/CN/LMF/OAM) as it is out of RAN1’s expertise that RAN1 cannot confirm.” Therefore RAN2 need to decide whether RAN2 will wait for RAN1’s progress (e.g. sending a LS to RAN1 asking to initiate related discussion) or RAN2 can start the corresponding discussion.

Proposal 2: For solution 1a/1b/3a/3b, RAN2 discuss between the following 2 options:

- RAN2 will only start discussing solution 1a/1b/3a/3b after confirmation from RAN1, e.g. RAN2 can send a LS to RAN1 to initiate related discussion.

- RAN2 start the discussion on solution 1a/1b/3a/3b without waiting for RAN1’s confirmation.

In following sections, we discuss details about model transfer options assuming that RAN2 will discuss solution 1a/1b/3a/3b.

Correction of TR 38.843

In TR 38.843, following analysis is captured regarding solution 2a and 3a.

Table 7.3.1.4-2 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 2a and 3a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - Model size >45kBytes is not supported based on existing number of RRC segments - Core Network supports NAS signalling segmentation - LMF supports LPP signalling segmentation | If NAS/LMF does not do segmentation for model transfer/delivery, it may need RRC segmentation, and extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Supported with limitation: - For Solution 2a, support within AMF coverage area based on NAS signalling segmentation - For Solution 3a, support within LMF coverage area based on LPP signalling segmentation | Note: Supporting service continuity across AMF/LMF is out of RAN scope and needs coordination with Core Network groups |

The above analysis assumes that NAS layer supports segmentation. However, from NAS specifications (TS 24.501 and 24.007), it seems that NAS does not support segmentation, therefore correction is needed for TR 38.843.

Proposal 3: “NAS signalling segmentation” is removed from TR 38.843 Table 7.3.1.4-2.

An example of correction is shown below:

Table 7.3.1.4-2 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 2a and 3a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - Model size >45kBytes is not supported based on existing number of RRC segments - LMF supports LPP signalling segmentation | If LMF does not do segmentation for model transfer/delivery or NAS signalling is used for model transfer/delivery, it may need RRC segmentation, and extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Supported with limitation: - For Solution 3a, support within LMF coverage area based on LPP signalling segmentation | Note: Supporting service continuity across AMF/LMF is out of RAN scope and needs coordination with Core Network groups |

Necessity to have partial model update as comparison area

In email discussion “[POST123bis][016][AI/ML] Model transfer (Intel)” [1], one open issue is “Proposal 1: RAN2 to continue discussion on the definition of partial model update and whether this needs to be supported during model transfer/delivery if needed.” This is related to the model format captured in Table 4.3-1 in TR 38.843, as copied below:

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |

In recent RAN1 meetings, case z2, z3, and z5 were deprioritized, therefore only case y, z1, and z4 can be considered in Rel-19. For cases y and z1, model transfer format is transparent to 3GPP system, therefore there is no difference for the model transfer options under discussion. Cases z4 is an open format, and partial model update can be supported for both CP and UP based solutions. Again, there is no difference for the model transfer options. Therefore partial model update should not be an area to compare model transfer options.

Proposal 4: Partial model update is not an area to compare model transfer options.

Partial model update is mainly a stage-3 signaling design detail aspect, and can be discussed after RAN2 selects model transfer option(s) to be standardized.

Proposal 5: Whether / how to support partial model update during model transfer can be decided after RAN2 selects model transfer option(s) to be standardized.

Model delivery between UE and gNB (solution 1a and 1b)

UP solution (1b)

In the UP model delivery solution (1b) between UE and gNB, existing user plane is not applicable since it terminates at UE and UPF. A new layer can be introduced to handle the AI model transfer functionality. Furthermore, other AI related functionalities, e.g. data collection, model registration, may also be supported in the new layer, if existing protocol layers cannot support the functionalities.

The protocol stack of new layer is depicted in Figure 1 below. The new layer is temporarily called AI for simplicity.

Figure 1: Protocol stack for UP based model delivery

The AI layer provides following services to upper layer:

- AI Model delivery;

- …

The AI layer expects following services from lower layer:

- Integrity protection, ciphering and loss-less in-sequence delivery of information without duplication;

To support model delivery, AI layer delivers the AI model to the lower layer, i.e. PDCP. New bearer or existing bearer can be used to carry AI model. Existing user plane transmission procedure can be reused and it is expected that there are no impacts to lower layers. The AI model can be delivered by separate radio bearers. Since the new layer terminates at UE and gNB, the UP solution is in RAN2 scope without involvement of other WGs.

Proposal 6: New layer on top of PDCP can be introduced to support UP AI model delivery between UE and gNB. The work can be done within RAN2 without involvement of other WGs.

In TR 38.843, aspect A4 of solution 1b is as follows:

| Discussion Area | Current status and Gaps | RAN specification impact |
| --- | --- | --- |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - QoS management at gNB if PDU session is terminated at gNB is not supported | Identify a solution to support QoS management at gNB for model transfer when PDU session is terminated at gNB if needed |

Contrary to what is captured in above table, the QoS management at gNB for DRB is not a gap for solution 1b. The reason is that when DRB is terminated at gNB, QoS management is completely under gNB control and it is purely a gNB implementation issue, e.g. configuring DRB parameters for AI model transfer properly to meet the QoS requirement. This is similar to solution 1a, where SRB is used for AI model delivery. In both 1a and 1b, gNB is responsible to guarantee the QoS of AI model delivery and how to achieve that is completely up to gNB implementation. We’d like to propose to modify A4 for solution 1b as follows:

| Discussion Area | Current status and Gaps | RAN specification impact |
| --- | --- | --- |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - QoS management at gNB can be handled by gNB implementation |  |

Proposal 7: For A4 aspect of solution 1b, QoS management is up to gNB implementation. In the comparison table of TR 38.843, remove the “RAN specification impact” part, and modify the “Current status” part to “QoS management at gNB can be handled by gNB implementation”.

CP solution (1a)

In TR 38.843, A2 and A4 aspects for solution 1a are captured as below:

Table 7.3.1.4-1 Analysis of current status and gaps, and potential 
RAN specification impact for Solution 1a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Transmission is restarted upon mobility | - Requires service continuity support for SRBs with segmentations. - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Procedure latency depends on model size and SRB priority | Impact on SRB in DL, e.g., a new SRB with configurable priority, etc. |

Regarding aspect A2, since all stored PDCP SDUs and PDCP PDUs of SRBs are discarded during PDCP re-establishment, service continuity across gNBs is not supported for SRB. Therefore if service continuity for model transfer should be supported for solution 1a, then enhancement is needed to support SRB service continuity during mobility.

Proposal 8: If service continuity for model transfer should be supported for solution 1a, enhancement is needed to support service continuity of SRB during mobility.

As for aspect A4, if the AI model is large, the RRC message may be segmented into multiple parts. As in TS 38.300 clause 7.10 shown below, it is required that “all segments of an RRC message are transmitted before sending another RRC message”. This means that when transmitting a large AI model, other RRC messages, which could be delay sensitive, might be delayed.

An RRC message may be segmented in case the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size. Segmentation is performed in the RRC layer using a separate RRC PDU to carry each segment. The receiver reassembles the segments to form the complete RRC message. All segments of an RRC message are transmitted before sending another RRC message. Segmentation is supported in both uplink and downlink as specified in TS 38.331 [12].

To resolve this issue, new SRB for AI model transfer should be introduced, so that segmented RRC messages of AI model do not delay the transmission of other RRC messages.

Proposal 9: If solution 1a is supported for AI model transfer, a new SRB is introduced.

Model delivery between UE and LMF (solution 3a and 3b)

In solution 3b, AI/ML model is transferred between LMF and UE via LPP signalling carried as UP data, as specified in TS 23.273 [2]. Therefore in both solution 3a and 3b, AI/ML model are carried in LPP signalling. When LPP signalling is carried in CP (solution 3a) and UP (solution 3b), the underlying transport protocols above L2 protocols (PDCP/RLC/MAC) in air interface are NAS/RRC signalling and IP based protocols, respectively. From RAN2 perspective, there might be no difference between Option 3a and 3b since both options require LPP enhancements to support AI model transfer/delivery.

Proposal 10: If AI/ML model transfer/delivery from LMF to UE is supported, both solution 3a and 3b are supported.

When inference is performed by UE, the AI model may come from the OTT server of the UE. In this case, UE may need to provide the AI model information/function to LMF. The existing LPP message could be considered, for example, UE provides the AI model information/function to LMF by LPP ProvideCapabilities.

Proposal 11: When inference is performed by UE and the AI model comes from UE OTT server, UE may provide the AI model information/function to LMF for LMF to manage AI models. The existing LPP message, for example, the LPP ProvideCapabilities can be considered.

Proactive / reactive model delivery

In RAN2#123 meeting, it was agreed to support reactive model delivery while proactive model delivery is FFS. In reactive model delivery, AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites. On the contrary, in proactive model delivery, AI/ML models are pre-downloaded to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

It is helpful to first discuss which entity initiates the model delivery. From RAN2 perspective, we focus on gNB and LMF as network entities. Considering that model delivery might consume significant amount of system resource and typically network is in charge of UE behavior, it is proposed that gNB and LMF can initiate model delivery, while UE cannot.

Proposal 12: Model transfer/delivery can be initiated by gNB and LMF (if model delivery from LMF to UE is supported), but not by UE.

For reactive model delivery, the initiation of model delivery is due to change in scenarios, configurations, or sites. Change of sites can be handled in mobility procedure, e.g. target gNB can initiate model delivery to UE after handover. For change of scenarios and configurations, existing measurements and location information can help gNB and LMF to decide whether to initiate model delivery. For example, based on measurement results, gNB and LMF can detect change of scenarios and configurations, and initiate model delivery accordingly. RAN2 needs to discuss whether current measurements are sufficient, or additional measurements are needed to facilitate model delivery.

Proposal 13: RAN2 to discuss whether existing measurements are sufficient to help gNB/LMF to detect/predict change of scenarios/configurations for initiation of model delivery.

Conclusion

In this contribution, we discuss AI/ML model delivery methods, and propose the following:

Proposal 1: RAN2 will not discuss solution 2a/2b/4b before receiving feedback from SA TSG/WGs on UE side data collection options in RAN#106.

Proposal 2: For solution 1a/1b/3a/3b, RAN2 discuss between the following 2 options:

- RAN2 will only start discussing solution 1a/1b/3a/3b after confirmation from RAN1, e.g. RAN2 can send a LS to RAN1 to initiate related discussion.

- RAN2 start the discussion on solution 1a/1b/3a/3b without waiting for RAN1’s confirmation.

Proposal 3: “NAS signalling segmentation” is removed from TR 38.843 Table 7.3.1.4-2.

Proposal 4: Partial model update is not an area to compare model transfer options.

Proposal 5: Whether / how to support partial model update during model transfer can be decided after RAN2 selects model transfer option(s) to be standardized.

Proposal 6: New layer on top of PDCP can be introduced to support UP AI model delivery between UE and gNB. The work can be done within RAN2 without involvement of other WGs.

Proposal 7: For A4 aspect of solution 1b, QoS management is up to gNB implementation. In the comparison table of TR 38.843, remove the “RAN specification impact” part, and modify the “Current status” part to “QoS management at gNB can be handled by gNB implementation”.

Proposal 8: If service continuity for model transfer should be supported for solution 1a, enhancement is needed to support service continuity of SRB during mobility.

Proposal 9: If solution 1a is supported for AI model transfer, a new SRB is introduced.

Proposal 10: If AI/ML model transfer/delivery from LMF to UE is supported, both solution 3a and 3b are supported.

Proposal 11: When inference is performed by UE and the AI model comes from UE OTT server, UE may provide the AI model information/function to LMF for LMF to manage AI models. The existing LPP message, for example, the LPP ProvideCapabilities can be considered.

Proposal 12: Model transfer/delivery can be initiated by gNB and LMF (if model delivery from LMF to UE is supported), but not by UE.

Proposal 13: RAN2 to discuss whether existing measurements are sufficient to help gNB/LMF to detect/predict change of scenarios/configurations for initiation of model delivery.

- References

[1] R2-2312035, Intel, "summary of [POST123bis][016][AI/ML] Model transfer (Intel)"

[2] 3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2"

[3] RP-242399, Qualcomm, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface"

[4] RP-242389, RAN, "LS on AIML for data collection"