3GPP TSG-RAN WG2 Meeting #127bis	R2-2408034

# Hefei, China, 14 – 18 October 2024

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

Introduction

In updated XR WID [1], XR rate control was added as one objective:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

In this contribution, we discus XR rate control.

Discussion

Review of existing rate control mechanisms

Currently, RAN-assisted codec adaptation can be used to adjust DL/UL data rate, with the combination of signalling in air interface and upper layer (e.g. RTP layer). The high level concept is specified in TS 38.300 clause 16.2.1.1:

RAN-assisted codec adaptation provides a means for the gNB to send codec adaptation indication with recommended bit rate to assist the UE to select or adapt to a codec rate for MMTEL voice or MMTEL video. The RAN-assisted codec adaptation mechanism supports the uplink/downlink bit rate increase or decrease. For a bearer associated with configuration of MBR greater than GBR, the recommended uplink/downlink bit rate is within boundaries set by the MBR and GBR of the concerned bearer.

For uplink or downlink bit rate adaptation, gNB may send a recommended bit rate to the UE to inform the UE on the currently recommended transport bit rate on the local uplink or downlink, which the UE may use in combination with other information to adapt the bit rate, e.g. the UE may send a bit rate request to the peer UE via application layer messages as specified in TS 26.114 [24], which the peer UE may use in combination with other information to adapt the codec bit rate. The recommended bit rate is in kbps at the physical layer at the time when the decision is made.

The recommended bit rate for UL and DL is conveyed as a MAC Control Element (CE) from the gNB to the UE as outlined in Figure 16.2.1.1-1.

Figure 16.2.1.1-1: UL or DL bit rate recommendation

Based on the recommended bit rate from the gNB, a UE may initiate an end-to-end bit rate adaptation with its peer (UE or MGW). The UE may also send a query message to its local gNB to check if a bit rate recommended by its peer can be provided by the gNB. The UE is not expected to go beyond the recommended bit rate from the gNB.

The recommended bit rate query message is conveyed as a MAC CE from the UE to the gNB as outlined in Figure 16.2.1.1-2.

Figure 16.2.1.1-2: UL or DL bit rate recommendation query

Recommended bit rate MAC CE format is specified in TS 38.321, as below:

Figure 6.1.3.20-1: Recommended bit rate MAC CE

The recommended bit rate is per LCH, and is applicable for DL or UL. The Bit Rate is in the range of 0~8000 kbps, and bit rate multiplier (field X, RRC IE bitRateMultiplier) can be in the set of {x40, x70, x100, x200}, which means that the supported data rate is in the range of 0~1.6 Gbps.

Is existing mechanism sufficient?

The objective for Rel-19 rate control is to “specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion”. Existing Recommended bit rate MAC CE can fulfill the overall target of rate adaptation. So the question is whether existing Recommended bit rate MAC CE is sufficient or not.

Recommended bit rate MAC CE is applicable for MMTEL voice or video. From scenario perspective, RAN2 needs to discuss whether it is also applicable for XR services.

Proposal 1: RAN2 to discuss whether Recommended bit rate MAC CE is applicable for XR services, from usage scenario perspective.

Another aspect for discussion is the maximum data rate to be supported. The maximum data rate of Recommended bit rate MAC CE is 1.6 Gbps. During Rel-18 work on BSR enhancement, it was agreed in RAN2#124 meeting that “	The maximum buffer size can be determined based on the ratio between maximum link rate (60Mbps) and minimum frame rate (15 fps), which is 750KB.”, which is based on contribution R2-2313560. In TR 38.835, it is stated that “Regarding the bit rates, between 10 and 200Mbps can be expected for XR depending on frame rate, resolution and codec efficiency (see TR 26.926 [6] and TR 26.928 [7]).” It is obvious that 60 Mbps and 200 Mbps are well below 1.6 Gbps.

Proposal 2: RAN2 to confirm that 1.6 Gbps is sufficient for the maximum data rate for rate control.

Due to the usage of bitRateMultiplier, there are two issues of existing Recommended bit rate MAC CE.

- Duplicated code points: An example is that 100 times of 40 kbps is 4000 kbps, which is another code point when multiplier is not used. There are 56 codes points used in the MAC CE, for data rate in the range of 0~8000 kbps. Considering 4 multipliers, there are 56 * 5 = 280 code points. However, there are only 196 unique values among the code points, which means that (280 – 196) / 280 = 30% code points are wasted.

- The code points are not evenly distributed in linear or exponential scale. For example, following are a subset of codepoints in ascending order. It can be seen that there are code points close to each other, e.g. 450000 and 455000, and code points with larger distance like 800000, 900000, and 1000000. This is not an efficient way to utilize the available code points.

| Code point value | Ratio relative to previous value |
| --- | --- |
| 450000 | 1.071 |
| 455000 | 1.011 |
| 490000 | 1.077 |
| 500000 | 1.021 |
| 525000 | 1.050 |
| 550000 | 1.048 |
| 560000 | 1.018 |
| 600000 | 1.071 |
| 650000 | 1.083 |
| 700000 | 1.077 |
| 750000 | 1.071 |
| 800000 | 1.067 |
| 900000 | 1.125 |
| 1000000 | 1.111 |

Observation 1: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

Proposal 3: RAN2 to discuss whether improvement on the index to bit rate mapping is needed for rate control.

Rate control granularity

In Recommended bit rate MAC CE, rate control is per LCH. If RAN2 agrees to introduce a new MAC CE for rate control, then discussion might be needed on whether to keep using per LCH granularity, or to consider alternatives like per QoS flow control, or per DRB control.

Since the rate adjustment is eventually done in application layer, it is expected that the adjustment is in QoS flow granularity. One example is that per QoS flow Bit Rate is signalled as QoS parameter from 5GC to gNB (TS 38.413 clause 9.3.1.10 GBR QoS Flow Information). However, with proper network configuration of QoS flow to DRB mapping, per LCH adjustment can achieve similar result as per QoS flow adjustment, and is already used in Recommended bit rate MAC CE. In addition, since rate control will be specified as MAC layer, it is more natural to have per LCH granularity.

Proposal 4: Rate control is performed in LCH granularity, unless critical issue is identified.

Split bearer in DC

In DC scenario, split bearer can be used for XR services to boost the throughput. If both MN and SN send MAC CEs for rate control, some discussion might be needed on UE behavior, e.g. whether UE follows latest MAC CE from MN or SN.

Proposal 5: RAN2 to discuss whether split bearer should be considered for rate control.

Conclusion

In this contribution, we discuss XR rate control, and have the following observation:

Observation 1: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

We propose the following:

Proposal 1: RAN2 to discuss whether Recommended bit rate MAC CE is applicable for XR services, from usage scenario perspective.

Proposal 2: RAN2 to confirm that 1.6 Gbps is sufficient for the maximum data rate for rate control.

Proposal 3: RAN2 to discuss whether improvement on the index to bit rate mapping is needed for rate control.

Proposal 4: Rate control is performed in LCH granularity, unless critical issue is identified.

Proposal 5: RAN2 to discuss whether split bearer should be considered for rate control.

- References

[1] RP-241771, Nokia (Rapporteur), “Revised WID on XR (eXtended Reality) for NR Phase 3”