3GPP TSG-RAN WG2 Meeting #127bis	           R2-2408037

Hefei, China, Oct 14th – Oct 18th, 2024

Agenda item:	8.1.2.2

Source:	China Telecom

Title:	Discussion on LCM for UE-sided model for BM

Document for:	Discussion

- Introduction

During RAN2#127 meeting, RAN2 sent an LS to RAN1 to inform the agreements on applicable functionality reporting for beam management UE-sided model and ask RAN1 some questions related to applicable functionality reporting and activation necessary for RAN2 progress. Since RAN1 has not replied the LS, in this paper, we would like to discuss about another part of LCM, i.e. necessary procedures to configure the UE for training UE-sided model for beam management use case.

- Discussion

In our understanding, generally, the following aspects need to be considered for configuring the UE for training UE-sided model for beam management use case:

- The consistency of NW-side additional condition across training and inference for UE-sided model

- Who initiate UE-sided model training data collection

- How to enable/configure the UE to perform data collection for UE-side model training

- When to stop UE-sided model training data collection

- How to stop UE-sided model training data collection

During Rel-18 study for AI/ML, the impact of mismatched beam information (e.g., beam angle, beam widths) of Set B and Set A across training and inference was studied and a significant performance loss was identified with the mismatch. Thus, to ensure the consistency of Set A and Set B on beam information across training and inference is very important. The following RAN1 and RAN2 agreements were achieved for the consistency of NW-side additional condition across training and inference for UE-sided model:

- RAN2#126 For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning.

- RAN1#116bis Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. - RAN1#118 Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

An associated ID is introduced to provide consistency information, and the consistency across training and inference can be assumed by the UE. Associated ID can be used for UE categorizing the measurements (as model inputs/labelling during training), and associated ID can also be used in the inference operation. For example, UE can assume the similar properties of a DL Tx beam for the resources with the same associated ID.

For UE-sided model training for BM case, the training data collection procedure can be initiated by the network or triggered by the UE and decided by the network. On the other hand, since the UE-sided data collection solutions are still under study, it is not clear the location of the UE-sided model training server/entity. So, it’s better to keep both options for now.

Regarding the ‘network-initiated’ method, according to RAN2#125bis meeting agreements, the UE can report supported functionalities included in UE capability, and “supported” doesn’t mean necessarily that the UE has the model available. The training data collection procedure can be initiated after the UE reporting supported functionalities. Taking the functionalities supported by the UE into account, the network determines whether to initiate the UE-sided model training (for which case) or not and the proper data collection configurations. RRC signaling, e.g. RRCReconfiguration message can be used for the network to initiate the UE-sided model training procedure.

- RAN2#125bis Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Regarding the ‘UE-triggered, network-decided’ method, RRC signaling can be used for the UE to trigger and the network to initiate the UE-sided model training procedure. It is possible that the UE can indicate its intended UE-sided model training cases/subcases if it is interested in UE-sided model training data collection via UAI message. Then the network may determine whether to initiate the UE-sided model training (for which case) or not and the proper data collection configurations. The network can configure data collection for UE-sided model training via RRCReconfiguration message.

Figure 1 network-initiated UE-sided model training procedure

Figure 2 UE-triggered, network-decided UE-sided model training procedure

Proposal 1: For BM case, there are two options for initiating the data collection procedure for UE-sided model training:

Option1: the UE can send UAI message to trigger the network to configure data collection for UE-sided model training.

Option2: the network initiate the data collection procedure for UE-sided model training via RRCReconfiguration message.

Regarding the data collection related configuration(s), the network needs to configure resources for Set B and Set A for the UE to realize model training, and it/their associated ID(s) for each (sub)use case in relation with NW-sided additional conditions to the UE. For example, the UE need to know the exact resources configured for Set B and Set A. In addition, there may be more than one Set B configured by the network. How to realize such configuration needs to be further discussed. The data collection configuration content/requirement need RAN1’s input.

After receiving the data collection configuration, the UE collects the training data corresponding to the associated ID(s). AI/ML models are trained or updated based on the collected data corresponding to the associated ID(s). Another issue is whether the associated ID is a global ID or a cell-specific ID. Whether the associated ID is a global ID or a cell-specific ID has impact on the design of data collection configuration. We need wait for RAN1’s progress on this issue.

Proposal 2: The network can configure Set A and Set B resources for UE-sided model training for each BM (sub)use case in relation with associated ID(s) to the UE via RRCReconfiguration message, details of design need RAN1’s further input, e.g. whether the associated ID is a global ID or a cell-specific ID.

Regarding when and how to stop UE side model training data collection, generally, the discussion is associated with another agenda item ‘UE-sided data collection’ and related to the solution (i.e. option 1a/1b/2/3) selected. Therefore we suggest to wait for the further progress and discuss in the agenda item ‘UE-sided data collection’.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: For BM case, there are two options for initiating the data collection procedure for UE-sided model training:

Option1: the UE can send UAI message to trigger the network to configure data collection for UE-sided model training.

Option2: the network initiate the data collection procedure for UE-sided model training via RRCReconfiguration message.

Proposal 2: The network can configure Set A and Set B resources for UE-sided model training for each BM (sub)use case in relation with associated ID(s) to the UE via RRCReconfiguration message, details of design need RAN1’s further input, e.g. whether the associated ID is a global ID or a cell-specific ID.

4	References

- [1] RP-241153, Status report for WI on AI/ML for NR air interface, 3GPP TSG RAN Meeting#104, Qualcomm, CAICT, Ericsson.

- [2] RP-241910, Status report for WI on AI/ML for NR air interface, 3GPP TSG RAN Meeting#105, Qualcomm, CAICT, Ericsson.