# 3GPP TSG RAN WG2 Meeting #128      	                        R2-2408039
Hefei, China, 14th – 18rd Oct, 2024

Agenda item:	8.1.3

Source:	China Telecom

Title:	Discussion on NW-sided data collection for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

The Rel-19 WID of AI/ML for air interface (WID RP-234039) was agreed in RAN#102 [1]. One of the objectives is to specify data collection for NW-sided model. In RAN2#125b [2], RAN2#126 [3] and RAN2#127 [4], NW-side data collection was discussed, and the following agreements were made with several FFSs.

Agreements (RAN2#125b)

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT.

Agreements for beam management (RAN2#126)

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model.

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

Agreements

1. 	As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

2. 	UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

3. 	When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

4. 	Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

5. 	FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

6. 	FFS on event based data collection/logging

7. 	On-demand request from the network is supported.   FFS details on signalling

In this contribution, we further discuss NW-sided data collection for AI/ML based beam management.

# 2 Discussion

## 2.1 Procedure of NW-side data collection between gNB and UE

Based on the agreements from the previous meetings, the gNB-centric and OAM-centric data collection approaces were considered for NW-side data collection, and the procedure between RAN and UE is common to both gNB-centric and OAM-centric data collection for NW-side data collection. The procedure of NW-sided data collection between gNB and UE is illustrated in Figure .1.

Figure. 1 Procedure of NW-side data collection between gNB and UE

In the following, we discuss the remaining issues from below steps.

- Step 1: RRC configuration

- Step 2: Perform data collection

- Step 3: Trigger reporting of measurements

## 2.2 Step 1: RRC configuration

### 2.2.1 Measurement period for UEs to start and stop collecting training data

NW-side data collection framework is used to collect data for training NW-sided model. In the previous meetings, RAN2 agreed that the immediate MDT is the baseline framework for NW-side data collection. For immediate MDT, measurement configuration (i.e. MeasConfig) from gNB to UE is included in RRCReconfiguration message.

Observation 1: Measurement configurations for NW-side data collection are included in RRCReconfiguration message.

Sometimes, to train the NW-sided model, it is necessary to collect training data from a certain measurement period. The measurement period can be represented by the start time and stop time for collecting training data. With the measurement period, the UEs collect training data from the start time of the measurement period to the stop time of the measurement period.

Observation 2: Measurement period for NW-side data collection is needed.

In TS 32.422 [4], collection period parameter for immediate MDT measurements (e.g. M6, M7) is defined, which defines the measurement period that should be used to collect MDT measurements. The collection period, as one of the measurement configurations, may be included in RRCReconfiguration message.

Observation 3: Measurement period for immediate MDT measurements (e.g. M6, M7) is configured by including the collection period parameter in RRCReconfiguration message.

According to the observations 1, 2, and 3, the network may need to control the measurement period for collecting training data for NW-sided model. In the legacy immediate MDT solution, the measurement period is configured by including the collection period parameter in RRCReconfiguration message. Therefore, to configure the measurement period for collecting training data for NW-sided model, we have following proposal:

Proposal 1: RRCReconfiguration message should be enhanced to configure the collection period that should be used for collecting the NW-sided training data from UEs.

### 2.2.2 Reporting Periodicity for UEs to report training data

As for the measurement reporting, RAN2 agreed to enhance the immediate MDT framework to support periodical reporting. As our understanding, the measurement reporting periodicity needs to be configured. Similar to the measurement period, the network may need to control the periodicity for reporting the NW-side training data. So, we have the following observation and proposal:

Observation 4: Reporting Periodicity for NW-side data reporting is needed.

Proposal 2: RRCReconfiguration message should be enhanced to configure the reporting periodicity that should be used for UEs reporting the NW-side training data.

## 2.3 Step 2: Perform data collection

The agreements made in RAN2#127 [4] are as follows:

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. When UE reaches its buffer limitation, the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging can be controlled based on power state of the UE.

Observation 5: The measurements for data collection purposes and logging can be controlled based on the minimum AS layer memory size supported by the UE and the power state of the UE.

Then, the only remaining issue is the below FFS made in RAN2#127 [4]:

4. 	Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

Based on the agreements made in RAN2#127 [4] and observation 5, in the case where the measurements for data collection purposes and logging is controlled based on the minimum AS layer memory size supported by the UE

the UE stops measurement when the minimum AS layer memory size is reached. Similarly, in our understanding, in the case where the measurements for data collection purposes and logging is controlled based on the power state of the UE, the UE may stop measurement when the UE power state is not sufficient.

Therefore, we have following proposal:

Proposal 3: The UE should be allowed to stop measurement for data collection purposes and logging based on its power state.

## 2.4 Step 3: Trigger reporting of measurements

Based on the agreements from RAN2#126 [3] and RAN2#127 [4], it was agreed to support periodical reporting and network request reporting. But whether to support event-based reporting is still FFS. In the following, we discuss the remaining issues related to event-based reporting.

Based on the observation 5 and proposal 3, we think at least the following events need to be supported:

- AS buffer event-based reporting: When UE reaches its buffer limitation, the UE stops measurement and triggers the event-based reporting.

- UE power state-based reporting: When UE power state is not sufficient, the UE stops measurement and triggers the event-based reporting.

So, we have the following proposal:

Proposal 4: Event-based reporting should be supported, which supports at least the following events:

- AS buffer event-based reporting: When UE reaches its buffer limitation, the UE stops measurement and triggers the event-based reporting.

- UE power state-based reporting: When UE power state is not sufficient, the UE stops measurement and triggers the event-based reporting.

# 3 Conclusion

In this contribution, we further discuss NW-side data collection for AI/ML based beam management. We have following observations:

Observation 1: Measurement configurations for NW-side data collection are included in RRCReconfiguration message.

Observation 2: Measurement period for NW-side data collection is needed.

Observation 3: Measurement period for immediate MDT measurements (e.g. M6, M7) is configured by including the collection period parameter in RRCReconfiguration message.

Observation 4: Reporting Periodicity for NW-side data reporting is needed.

Observation 5: The measurements for data collection purposes and logging can be controlled based on the minimum AS layer memory size supported by the UE and the power state of the UE.

Based on observations, we have following proposals:

Proposal 1: RRCReconfiguration message should be enhanced to configure the collection period that should be used for collecting the NW-sided training data from UEs.

Proposal 2: RRCReconfiguration message should be enhanced to configure the reporting periodicity that should be used for UEs reporting the NW-side training data.

Proposal 3: The UE should be allowed to stop measurement for data collection purposes and logging based on its power state.

Proposal 4: Event-based reporting should be supported, which supports at least the following events:

- AS buffer event-based reporting: When UE reaches its buffer limitation, the UE stops measurement and triggers the event-based reporting.

- UE power state-based reporting: When UE power state is not sufficient, the UE stops measurement and triggers the event-based reporting.

# 4 References

[1] RP-234039, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#125b, Chair Notes.

[3] RAN2#126, Chair Notes.

[4] RAN2#127, Chair Notes.

[5] TS 32.422-v18.2.0, Trace control and configuration management, 2024-03.