3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408044

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.4

Source:	China Telecom

Title:	Procedures for LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting, some progress has been made in RAN2 for option 1-2, which are shown as follows [1].

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Meanwhile, RAN1 also achieved some agreements for LP-WUS in RRC_CONNECTED mode. In this contribution, we will continue to analyze the procedures for LP-WUS in RRC_CONNECTED.

- Discussion

For the LP-WUS procedures to trigger PDCCH monitoring, we have discussed the understanding of Option 1-2 from RAN2 perspective in the last meeting. We need to further discuss it based on existing progress. For Option 1-2, LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. Regarding whether UE performs PDCCH monitoring during legacy C-DRX active time, RAN1 provides two options as follows:

- Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. This method can be understood as adding additional LP-WUS monitoring outside C-DRX active time based on Option 1-1, which means that there are two different mechanisms to monitor PDCCH during the outside C-DRX active time.

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. This is an independent process to monitor PDCCH triggered by LP-WUS during the whole DRX cycle.

For both Option 1-2-1 and Option 1-2-2, after LP-WUS triggers the UE to perform PDCCH monitoring, UE starts a timer. In our understanding, UE doesn’t need to monitor LP-WUS within the duration when UE monitors PDCCH triggered by LP-WUS.

Proposal 1: For Option 1-2, there is no need to monitor LP-WUS when UE monitors PDCCH triggered by LP-WUS.

As RAN2 has agreed that the timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring for Option 1-2. The specific configuration method for the time offset can be discussed by RAN2. The simplest method is through SIB to indicate the time offset from the RAN2 perspective. MR may select its sleep state according to the configured time offset. Moreover, it can also configured by special RRC signalling.

Proposal 2: RAN2 assumes that the time offset can be configured by SIB or RRC signalling for Option 1-2. Details are FFS.

To reasonably configure the above time offset, UE may report its supported minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. RAN1 has the following relevant agreement in RAN1#118 meeting [2].

Agreement Select one of the following alternatives in RAN1#118bis Alt 1 - For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2 - For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network

From RAN2 perspective, we think the number of values for each SCS reported by UE can be one or multiple, which can wait for RAN1’s further discussion. No matter which option is selected, RAN2 needs to determine the UE capability IE carrying the message of minimum time gaps, i.g. defining a new UE capability IE or reusing a legacy UE capability IE. Then, after receiving the reported values, the network should configure the value of time offset to the UE. In this way, UE can select sleep states accordingly to achieve the purpose of energy saving.

Proposal 3: RAN2 needs to determine the UE capability IE for reporting the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

Based on the discussion process of RAN1 and RAN2, we think Option 1-3 for LP-WUS is not necessary. For Option 1-3, it indicates that the UE monitors the LP-WUS within the legacy drx-onDurationTimer. The application scenario is limited to the traffic with jitter. What’s more, the energy-saving gain is not sufficient compared with Option 1-2. Therefore, we don’t need to study Option 1-3 at this stage.

Proposal 4: Option 1-3 is not necessary for LP-WUS.

As for the activation and deactivation of LP-WUS monitoring, RAN1 has reached the following agreements in RAN1#116 meeting [3]:

Agreement - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Based on the above agreement, in case LP-WUS monitoring is enabled, there would exist at least one mechanism for activation/deactivation of LP-WUS monitoring, which can improve LP-WUS monitoring efficiency on the condition of the poor channel. In our view, we think that Option 2 is more flexible. It can be a new signalling or existing signalling to activate/deactivate LP-WUS monitoring. However, the detailed design also depends on RAN1’s further discussion. For Option 3, if the channel quality is not good enough to transmit LP-WUS when the timer is still running, it may be invalid for UE to monitor the LP-WUS. Thus, this option is not more suitable than Option 2. Option 4 is also feasible as discussed in the SI stage, we can postpone the discussion on it according to the progress of RAN1.

Proposal 5: In RRC_CONNECTED mode, option 2 is more flexible for the activation/deactivation of LP-WUS monitoring. Wait for RAN1’s progress.

- Conclusion

In this contribution, we further discuss the procedures for LP-WUS in RRC_CONNECTED. The proposals are summarized as follows.

Proposal 1: For Option 1-2, there is no need to monitor LP-WUS when UE monitors PDCCH triggered by LP-WUS.

Proposal 2: RAN2 assumes that the time offset can be configured by SIB or RRC signalling for Option 1-2. Details are FFS.

Proposal 3: RAN2 needs to determine the UE capability IE for reporting the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 4: Option 1-3 is not necessary for LP-WUS.

Proposal 5: In RRC_CONNECTED mode, option 2 is more flexible for the activation/deactivation of LP-WUS monitoring. Wait for RAN1’s progress.

- References

- RAN2#127 chairman notes.

- RAN1#118 chairman notes.

- RAN1#116 chairman notes.