3GPP TSG-RAN WG2 Meeting #127bis	R2-2408053

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on Inter-CU LTM in DC

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding inter-CU LTM in DC, the following agreements were reached.

Agreements on inter-CU LTM:

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

However, there are still some open issues on the details of inter-CU LTM in DC. In this contribution, we would like to share our opinions on support of inter-CU LTM in DC.

- Discussion

Stage-2 procedure for inter-CU SCG LTM

In last RAN2 meeting, it was agreed that only SN-initiated inter-SN LTM without MN changed is supported in Rel-19. In previous RAN2 meeting, the following agreements were reached for inter-CU LTM in SA. For the SN-initiated inter-SN LTM, similar mechnism could be used.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

For the SN-initiated inter-SN LTM, it also includes LTM preparation, the early synchronization, LTM cell switch execution and LTM cell switch completion. For the LTM preparation phase, it has been agreed that the source SN initiates the preparation. When the source SN decides to configure the inter-CU SCG LTM, it can send the LTM request and related information (such as the candidate cell information, early sync request information, reference configuration) to the MN. The MN will send the LTM request to the target candidate SN(s). Based on the LTM request, the candidate SN(s) will generate lower layer configuration of each accepted candidate cell and send the relate configuration to the MN.

Proposal 1: For the preparation phase of SN-initiated inter-SN SCG LTM, RAN2 confirms the following network interaction.

- When the source SN decides to configure the inter-CU SCG LTM, it sends the LTM request and related information to the MN.

- The MN sends the LTM request to the related target candidate SN(s).

- Based on the LTM request, the candidate SN(s) will generate lower layer configuration of each accepted candidate cell and send the related configuration to the MN.

For inter-CU LTM in non-DC scenario, the source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB and generates the common CSI resource configuration for L1 measurement on candidate cells. Since there is no direct interaction between the source SN and candidate SN(s), it may need to clarify which node is responsible in this case. If it is still the source SN to collect the related configuration and information and generate the common CSI resource configuration, there would be more interaction between the MN, source SN and candidate SN(s). It is better for RAN2 to clarify this issue and signalling details could be up to RAN3 decision.

Proposal 2: For SN-initiated inter-SN SCG LTM, RAN2 clarifies which node is in charge of collection of LTM related configuration and information and generation of the common CSI resource configuration.

- Option 1: MN collects the configurations and information of candidate cells from multiple candidate gNB and generates the common CSI resource configuration for L1 measurement on candidate cells.

- Option 2: MN collects the configurations and information of candidate cells from multiple candidate gNB and sends them to the source SN. The source SN generates the common CSI resource configuration for L1 measurement on candidate cells.

For the interaction between MN, source SN and candidate target SN(s), the preparation phase of SCPAC could be used as a baseline. Based on the agreements and above analysis, we try to give a potential stage-2 signalling flow for SN-initiated inter-SN SCG LTM as illustrated in Fig.1, which includes LTM preparation, the early synchronization, LTM cell switch execution and LTM cell switch completion. The signalling details could be up to RAN3 decision.

Proposal 3: RAN2 considers the stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed, as illustrated in Fig.1.

Fig.1 Stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed

Inter-CU MCG LTM

Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN or release the SN. For inter-CU MCG LTM with SCG unchanged or released, we can follow the similar principle that the target MN decides whether to keep or release the SN. For the case that the target MN decides to change the SN, the handling of SCG configuration could be up to NW implementation (e.g. release or reconfiguration), as agreed in last meeting.

Proposal 4: Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline.

Proposal 5: For inter-CU MCG LTM with SCG unchanged or released, the target MN decides whether to keep or release the SN.

- Conclusion

In this contribution, we discuss inter-CU LTM related aspects in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: For the preparation phase of SN-initiated inter-SN SCG LTM, RAN2 confirms the following network interaction.

- When the source SN decides to configure the inter-CU SCG LTM, it sends the LTM request and related information to the MN.

- The MN sends the LTM request to the related target candidate SN(s).

- Based on the LTM request, the candidate SN(s) will generate lower layer configuration of each accepted candidate cell and send the related configuration to the MN.

Proposal 2: For SN-initiated inter-SN SCG LTM, RAN2 clarifies which node is in charge of collection of LTM related configuration and information and generation of the common CSI resource configuration.

- Option 1: MN collects the configurations and information of candidate cells from multiple candidate gNB and generates the common CSI resource configuration for L1 measurement on candidate cells.

- Option 2: MN collects the configurations and information of candidate cells from multiple candidate gNB and sends them to the source SN. The source SN generates the common CSI resource configuration for L1 measurement on candidate cells.

Proposal 3: RAN2 considers the stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed, as illustrated in Fig.1.

Proposal 4: Regarding the interaction between network nodes of inter-CU MCG LTM with SCG unchanged or released, the Inter-MN handover with/without MN initiated SN change procedure specified in TS 37.340 could be used as a baseline.

Proposal 5: For inter-CU MCG LTM with SCG unchanged or released, the target MN decides whether to keep or release the SN.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

3GPP TS 38.300 18.1.0: " NR; NR and NG-RAN Overall Description; Stage 2".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB