3GPP TSG-RAN WG2 Meeting #127bis	R2-2408055

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on conditional intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In RAN#105 meeting, the revised WID on Rel-19 NR Mobility Enhancements Phase 4 was approved. Regarding the condition LTM, the following objective was clarified.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint.

In this contribution, we would like to share our opinions on the general aspects on conditional Intra-CU LTM in non-DC case.

- Discussion

Stage-2 procedure for intra-CU conditional LTM

To support the conditional intra-CU LTM, the procedure of intra-CU LTM could be used as a baseline. For stage-2 procedure of intra-CU conditional LTM, it could include conditional LTM preparation, the early synchronization, conditional LTM evaluation and execution and conditional LTM completion.

For conditional LTM preparation in intra-CU case, the conditional LTM configuration may include not only the LTM configuration similar to intra-CU LTM, but also the execution conditions for triggering the conditional LTM. For the preparation on the LTM configuration of candidate cell(s), the interaction between the gNB-CU and gNB-DU follows the same signalling procedure for intra-CU LTM. The gNB-CU decides the execution conditions and send the related conditional LTM configuration to the UE. Whether to have a separate LTM candidate configuration of conditional LTM or to share the LTM candidate configuration of legacy LTM could be further discussed.

Proposal 1: Regarding conditional LTM preparation in intra-CU case, the interaction between the gNB-CU and gNB-DU follows the same signalling procedure for intra-CU LTM.

Proposal 2: The gNB-CU is responsible for the decision of execution conditions for conditional LTM.

Proposal 3: Whether to have a separate LTM candidate configuration of conditional LTM or to share the LTM candidate configuration of legacy LTM could be further discussed.

Regarding the early synchronization phase, the early TA acquisition shall be supported in order to reduce the latency of handover. Since the NW is not aware of the triggering of conditional LTM by the UE, the PDCCH order triggered early TA acquisition does not work. UE-based TA measurement could be used for conditional LTM.

Proposal 4: Regarding the early TA acquisition procedure for conditional LTM, UE-based TA measurement could be used.

For the evaluation and execution of conditional LTM, the UE will evaluate the candidate cells based on the conditional LTM configuration and trigger the access to the target cell when the execution conditions are met. Further details will be discussed in the clause 2.2. For the execution of conditional LTM, both RACH-less and RACH-based procedure shall be supported. For the completion of conditional LTM, it is similar to the legacy LTM. The UE completes the conditional LTM procedure by sending the RRCRenconfigurationComplete message to target cell.

Proposal 5: Both RACH-less and RACH-based procedure shall be supported for intra-CU conditional LTM.

Proposal 6: The UE completes the conditional LTM procedure by sending the RRCRenconfigurationComplete message to target cell.

Based on the above analysis, we try to give a potential stage-2 signalling flow for intra-CU conditional LTM in non-DC case as illustrated in Fig.1.

Proposal 7: RAN2 considers the stage-2 signalling flow for intra-CU conditional LTM in non-DC case, as illustrated in Fig.1.

Fig.1 Stage-2 signalling flow for intra-CU conditional LTM in non-DC case

UE evaluated conditions for triggering conditional LTM

In previous RAN2 meetings, the event-triggered L1 measurement reporting was widely discussed and much progress has been made, which can provide guidance to UE evaluated conditions for triggering conditional LTM.

In RAN2#126 meeting, the following beam-level events for L1 LTM measurement reporting are introduced.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For conditional LTM, the triggering event shall be in beam-level as well. For conditional LTM cell switch, it is also very important to avoid “Ping Pong” or unnecessary cell switch, which may reduce the mobility performance and user experience. Therefore, not only the quality of the candidate beam but also the quality of the serving beam shall be considered when evaluating whether to trigger a conditional LTM. Similar to CHO, the following conditional events could be introduced for conditional LTM.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 8: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

For event-triggered L1 measurement reporting, the following agreements were reached on beam of the serving cell and candidate cell used for event evaluation.

Agreements on L1 MR event evaluation

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

Considering the conditional LTM is a kind of cell switch procedure, which is not similar to L1 measurement reporting, to follow the same principle on event evaluation may lead to more frequent cell switch. Thus, the serving beam and candidate beam(s) used for event evaluation in above conditional events might need further discussion. Potential options are listed as below.

- Option 1: all the configured beams of the serving cell or candidate cell;

- Option 2: a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams;

- Option 3: a single beam of the serving cell or candidate cell, e.g. the best beam, the worst beam or any beam.

For Option 1, it seems too strict to trigger conditional LTM only when all the configured candidate beams are good enough while the serving beams are extremely bad. It may take a long time to meet the trigger conditions, which may lead to cell switch failure or reduce the user’s experience. For Option 3, only a single beam is used for evaluating the trigger conditions, which may lead to more frequent or unnecessary conditional LTM cell switch. Option 2 could be a trade-off solution, which can reduce the evaluation time before triggering conditional LTM as well as guarantee the UE to select a target cell with good enough beams. Only when a certain number of beams of the serving cell or conditional candidate cell meet the trigger conditions of conditional events, the conditional LTM execution could be triggered.

Proposal 9: For event evaluation for conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

In CHO, the network can configure up to 2 condition events for a certain candidate cell. When configured, both of the conditional events have to be met simultaneously for triggering CHO. Since the motivation to support conditional LTM is not only to achieve high robustness but also short interruption, there seems no strong need to allow the configuration of up to 2 events for a certain candidate cell.

Proposal 10: For conditional LTM, there is no need to allow the configuration of up to 2 events for a certain candidate cell.

- Conclusion

In this contribution, we discuss the general aspects on conditional Intra-CU LTM in non-DC case. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding conditional LTM preparation in intra-CU case, the interaction between the gNB-CU and gNB-DU follows the same signalling procedure for intra-CU LTM.

Proposal 2: The gNB-CU is responsible for the decision of execution conditions for conditional LTM.

Proposal 3: Whether to have a separate LTM candidate configuration of conditional LTM or to share the LTM candidate configuration of legacy LTM could be further discussed.Proposal 2: For LTM event-triggered L1 measurement reporting, the L1 measurement report could have a flexible length.

Proposal 4: Regarding the early TA acquisition procedure for conditional LTM, UE-based TA measurement could be used.

Proposal 5: Both RACH-less and RACH-based procedure shall be supported for intra-CU conditional LTM.

Proposal 6: The UE completes the conditional LTM procedure by sending the RRCRenconfigurationComplete message to target cell.

Proposal 7: RAN2 considers the stage-2 signalling flow for intra-CU conditional LTM in non-DC case, as illustrated in Fig.1.

Proposal 8: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 9: For event evaluation for conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

Proposal 10: For conditional LTM, there is no need to allow the configuration of up to 2 events for a certain candidate cell.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB