3GPP TSG RAN WG2 Meeting #127bis	R2-2408067

Hefei, Anhui Province, China, Oct 14th – 18th, 2024

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#127 meeting, the following agreements have been made for random access in SBFD, shown as:

- 1. Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. - 2. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. - 3. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. - 4. FFS whether/how early indication is used during a SBFD RA procedure. - 5. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Then in this contribution, we would like to provide our views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

In RAN2#127 meeting, the following working assumptions are made for RA events:

- 1. Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

For legacy NR, there are different CBRA and CFRA events and corresponding PRACH resource indication/configuration signallings as summarized in Table 1.

Table 1. RA events and corresponding PRACH resource indication/configuration signallings

| RA event | CBRA PRACH resource indication/configuration signalling | CFRA PRACH resource indication/configuration signalling |
| --- | --- | --- |
| Initial access from RRC_IDLE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| RRC Connection Re-establishment procedure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronize" | - CBRA resource configured by RACH-ConfigCommon - PDCCH order trigger | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronized" | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| SR failure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Request by RRC upon synchronous reconfiguration (e.g. handover) | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by RACH-ConfigDedicated |
| RRC Connection Resume procedure from RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| To establish time alignment for a secondary TAG | - N/A | - CFRA resource configured by RACH-ConfigCommon - PDCCH order indicating the used PRACH resource and preamble |
| Request for Other SI | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by SI-RequestConfig or SI-RequestConfigRepetition |
| Beam failure recovery | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource, preamble and root sequence configured by BeamFailureRecoveryConfig |
| SDT in RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| Early UL synchronization with an LTM candidate cell | - N/A | - CFRA resource configured by EarlyUL-SyncConfig - PDCCH order trigger |
| RACH-based LTM cell switch | - CBRA resource configured by RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger | - CFRA resource configured by RACH-ConfigDedicated or RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger |

From Table 1, it can be observed that there are multiple events which can trigger random access procedures  and different RA procedures (CBRA or CFRA) may be triggered by different RA events. Furthermore, it can also be found that CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events. Since RAN2 assumes random access procedure in SBFD symbols is supported for all the existing RA events, we will discuss the resource configuration  of CBRA and CFRA separately in section 2.2.

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

## 2.2 	Configuration of RACH resource

In RAN1#118 meeting, it has been agreed that [1]:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration. Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

In RAN1, RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 has been agreed to support. To be specific, RACH configuration Option 1 with Alt 1-1 represents using one single RACH configuration, and only based on the existing parameters of the single RACH configuration. RACH configuration Option 2 with Alt 2-3 represents using two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration, and the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. To this end, one important issue to be discussed by RAN2 is whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported for all PRACH resource configuration signallings.

Proposal 1: RAN2 should discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported for all PRACH resource configuration signallings.

For a comprehensive view, the detailed analysis of the 4-step CBAR and CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is given as following.

### 2.2.1 Configuration for 4-step CBRA

- Option 1 with Alt 1-1

- For 4-step CBRA PRACH resource configured by RACH-ConfigCommon, there are two possible options which depends on whether SBFD is considered as a new feature which is parallel to readcap, smallData, etc. If SBFD is considered as a new feature, the CBRA configuration can be provided with featureCombinationPreamblesList in rach-ConfigCommon. If SBFD is not considered as a new feature, 4-step CBRA configuration shares the existing parameters configured in RACH-ConfigCommon. Besides, whether/which parameter(s) may be reinterpreted by SBFD-aware UEs should be analyzed.

- Option 2 with Alt 2-3

- Similar to Option 1 with Alt 1-1, there are also two options which depends on whether SBFD is considered as a new feature. If SBFD is considered as a new feature, then the CBRA configuration with Option 2 with Alt 2-3 can be provided in featureCombinationPreamblesList in AdditionalRACH-Config-r17. If SBFD is not considered as a new feature, a new IE called rach-ConfigCommonSBFD-r19 can be introduced to indicate the additional 4-step CBRA configuration. An example is given as:

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

[[

rach-ConfigCommonSBFD-r19            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

- While, it still needs to be discussed that whether all parameters in RACH-ConfigCommon IE is needed for rach-ConfigCommonSBFD-r19.

Observation 3: Whether SBFD is considered as a new feature affects the detailed methods of 4-step CBAR configuration.

Proposal 2: RAN2 is asked to discuss whether SBFD is considered as a new feature before determining the solutions to configure 4-step CBRA.

Proposal 3: Based on Option 1 with Alt 1-1, there are two possible solutions to configure 4-step CBRA for SBFD-aware UEs:

- Solution 1: SBFD is considered as a new feature parallel to redcap, smalldata, etc. and 4-step CBRA configuration for SBFD-aware UE is provided with featureCombinationPreamblesList in rach-ConfigCommon.

- Solution 2: SBFD is not considered as a new feature, 4-step CBRA configuration for SBFD-aware UE shares the existing parameters configured in RACH-ConfigCommon.

- FFS other solutions

Proposal 4: Based on Option 2 with Alt 2-3, there are two possible solutions to configure 4-step CBRA for SBFD-aware UEs:

- Solution 1: SBFD is considered as a new feature and 4-step CBRA configuration for SBFD-aware UE is provided with featureCombinationPreamblesList in AdditionalRACH-Config-r17.

- Solution 2: SBFD is not considered as a new feature, a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon, is introduced.

- FFS other solutions

### 2.2.2 Configuration for 4-step CFRA

- Option 1 with Alt 1-1

- Compared to the configuration for 4-step CBRA, which depends on whether SBFD is considered as a new feature, configuration for 4-step CFRA based on Option 1 with Alt 1-1 is much easier. To be specific, 4-step CFRA shares the same existing parameters in resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc. With consideration that there are multiple different resource configuration signallings for the 4-step CFRA configuration triggered by different RAN events, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

- Option 2 with Alt 2-3

- For 4-step CFRA configuration based on Option 2 with Alt 2-3, additional CFRA configuration is needed. An easy solution is introducing a new IE parallel to each legacy IE. For example, a new IE named BeamFailureRecoveryConfigSBFD-r19 can be introduced to provide the additional 4-step CFRA configuration for SBFD-aware UEs to be used when beam failure occurs. While the behavior of SBFD-aware UEs when some parameters in new IE is absent needs further discussion. For example, if ra-Prioritization is absent in BeamFailureRecoveryConfig, legacy UE releases this field. While if ra-Prioritization is absent in BeamFailureRecoveryConfigSBFD-r19, whether the SBFD-aware UE can use the same ra-Prioritization as configured in BeamFailureRecoveryConfig, if available.

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

### 2.2.3 Enabling case of different RACH configuration options

In RAN1#116bis meeting, it has been agreed that UE is not required to support both options. To this end, there may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE only supports Option 2 with Alt 2-3

- Cap 3: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3

If each SBFD-aware UE can support only one RACH configuration Option, that is, Cap 1 or Cap 2, no additional work is required in RAN2. Otherwise, RAN2 should discuss how a SBFD-aware UE identify which RACH configuration Option is enabled because enabling both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 at the same time for a UE is not supported, which is the working assumption of RAN1. From network configuration perspective, there may be two possible solutions. One solution is to  introduce a flag which indicates either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2 with Alt 2-3 is activated at the moment. In fact, the flag mentioned above may not be an explicit flag. For example, if RACH configuration Option 2 is provided in RACH resource configuration signalling, then RACH configuration Option 2 is considered as enabled and RACH configuration Option 1 with Alt 1-1 is disabled. Taking 4-step CBRA configuration as an example, if rach-ConfigCommonSBFD is present, then rach-ConfigCommon configures legacy ROs only, in which legacy ROs denote the ROs in uplink or flexible symbols, which are available for both SBFD-aware and non-SBFD-aware UEs. The other solution is to configure a prioritization rule for SBFD-aware UEs determining whether RACH configuration Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is used. However how to design the prioritization rule which may depends on the use cases needs further discussion.

Observation 4: There may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE only supports Option 2 with Alt 2-3

- Cap 3: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 5: If there are SBFD-aware UE(s) which supports two RACH configuration Options, there are two options:

- Option 1: a flag can be introduced. FFS the flag is explicit or implicit

- Option 2: a prioritization rule can be configured. FFS how to design it

Proposal 7: To indicate which RACH configuration Option is enabled, there are two cases:

- Case 1: If each SBFD-aware UE can support only one RACH configuration Option, no additional work is required.

- Case 2: If there are SBFD-aware UE(s) which supports two RACH configuration Options, there are two options:

- Option 1: a flag can be introduced. FFS the flag is explicit or implicit

- Option 2: a prioritization rule can be configured. FFS how to design it

## 2.3 	RACH procedure

### 2.3.1 Initial PRACH transmission attempt

In RAN1#118 meeting [1], the following agreement is made:

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Despite the agreements are made separately for RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 by RAN1, we think the same solution can be applied to both options with minimum standard effort.

- For Option 2, the resource selection is controlled by gNB and can achieve the gain of offloading PRACH transmission and reducing PRACH collision in best. To be specific, the gNB can configure the specific  prioritization rules based on the motivation of enabling PRACH transmission in SBFD symbols. For example, with the motivation is to improve PRACH capacity and reduce PRACH collision probability, a prioritization rule which implies DL RSRP higher than a threshold can be configured to cell centre SBFD-aware UEs use the additional ROs. On the other hand, If the motivation is to improve the PRACH coverage by using long PRACH format or using PRACH repetition, it is preferred that cell edge SBFD-aware UEs use the additional ROs, thus a prioritization rule which implies DL RSRP lower than a threshold should be configured for SBFD-aware UEs.

- For Option 3, it seems no standard impact since SBFD-aware UEs can select the PRACH resource freely. However, gNB cannot control the PRACH transmission load on legacy-ROs and additional-ROs and cannot change prioritization rules based on on the motivation of enabling PRACH transmission in SBFD symbols, which may degrade RACH performance in SBFD.

Observation 6: For initial PRACH transmission attempt in SBFD, Option 2 can bring better performance than Option 3, since it can achieve the gain of offloading PRACH transmission and reducing PRACH collision.

Proposal 8: For initial PRACH transmission attempt in SBFD, Option 2 is supported, i.e., selecting one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations.

### 2.3.2 PRACH transmission re-attempt

For PRACH transmission re-attempt, it has been agreed by RAN1 that:

Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Similar to initial PRACH transmission attempt, same solution can be applied to RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- For Option 1, it is a simple solution, but may degrade the flexibility for SBFD-aware UE selecting RACH resource. For example, if additional-ROs are selected in initial PRACH transmission by a SBFD-aware UE, then it have to use additional-ROs when conducting PRACH transmission re-attempt even if there is serious CLI in additional-ROs.

- For Option 2, it provides fallback mechanism to further improve the PRACH transmission success probability, similar to the the fallback behaviour from 2-step RA to 4-step RA. For example, considering the fact that the interference level in SBFD symbols is often more serious than that in non-SBFD symbols, a SBFD-aware UE which first selects additional-ROs may fails many times due to the serious interference. For Option 2, if its PRACH transmission counter is larger than a configured value, the SBFD-aware UE can select the legacy-ROs for the subsequent attempt of the random access procedure. However, it needs to be discussed whether both additional-ROs fallback to legacy-ROs and legacy-ROs fallback to additional-ROs are supported.

- For Option 3, it is flexible for UE side, but gNB cannot control the PRACH transmission load on legacy-ROs and additional-ROs because the behavior of SBFD-aware UEs seems to be unpredictable to gNB.

Based on above analysis, Option 2 should be supported from RNA2’s perspective, since it provides fallback mechanism to further improve the PRACH transmission success probability.

Observation 7: For PRACH transmission re-attempt, Option 2 is more flexible for selecting RACH resource than Option 1 and can control the PRACH transmission load on legacy-ROs and additional-ROs better than Option 3.

Proposal 9: For PRACH transmission re-attempt, Option 2 is supported, i.e., first using ROs of the same type as the earlier PRACH transmission for a certain number of times before using other type. FFS additional-ROs fallback to legacy-ROs and/or legacy-ROs fallback to additional-ROs

## 2.4 	Early indication

The gNB can identify a SBFD UE which transmits Msg1 in an additional RO since additional RO is in SBFD symbol. However, considering that legacy ROs are valid for both SBFD-aware UEs and non-SBFD UEs, one issue needs to be discussed by RAN2 is whether to support early indication of SBFD-aware UEs, which can be achieved by either Msg1-based or Msg3-based early indication.

- Msg1-based early indication

If Msg1-based early indication is supported, dedicated preamble(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the preamble received and thus schedule subsequent uplink transmission (e.g. Msg3, etc.) in SBFD symbol(s) for SBFD-aware UE, which brings benefits. However, if some dedicated preambles are allocated to SBFD-aware UEs, accordingly, the number of preambles which can be used by non-SBFD-aware UEs decreases, thus cause the degradation of PRACH capacity of non-SBFD-aware UEs. Considering gNB has already offloaded PRACH transmission in SBFD symbols for SBFD-aware UEs, the PRACH transmission collision is mitigated, the degradation of PRACH capacity of non-SBFD-aware UEs seems to be acceptable.

If Msg1-based early indication is not supported, the PRACH capacity of non-SBFD-aware UEs will not be affected, but gNB can not schedule the subsequent transmission (e.g. Msg3, etc.) in SBFD symbol(s) earlier for SBFD-aware UE.

- Msg3-based early indication

- If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission in SBFD symbol(s) for SBFD-aware UE which brings benefits. Considering using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs. However, compared with Msg1-based early indication, gNB identifies whether the UE is an SBFD-aware UE later, thus can only schedule uplink transmission(s) after Msg3 to transmit on the SBFD symbol(s).

- From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs. For Msg1-based early indication, whether it is supported depends on RAN1’s discussion. While, it should also be noted that if SBFD is considered as a new feature and the RANCH configuration for SBFD-aware UEs is provided with featureCombinationPreamblesList, then Msg1-based early indication can be supported easily.

Observation 8: The gNB can identify a SBFD UE which transmits Msg1 in an additional RO which is in SBFD symbol.

Observation 9: If Msg1-based early indication is supported, gNB can identify a SBFD-aware UE early and thus schedule its subsequent transmission (e.g. Msg2/3/4) in SBFD symbol(s), but may reduce the PRACH capacity of non-SBFD-aware UEs.

Observation 10: For Msg1-based early indication, the degradation of PRACH capacity of non-SBFD-aware UEs seems to be acceptable because gNB has already offloaded PRACH transmission in SBFD symbols for SBFD-aware UEs.

Observation 11: With Msg3-based early indication, there is almost no impact on PRACH capacity of non-SBFD-aware UEs. However, gNB identifies whether the UE is an SBFD-aware UE a little later compared with using Msg1-based early indication.

Observation 12: If SBFD is considered as a new feature and feature combination framework is reused, Msg1-based early indication can be supported naturally.

Proposal 10: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs. Whether supporting Msg1-based early indication depends on RAN1.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings such as SI-RequestConfig, BeamFailureRecoveryConfig etc., for different RA events.

Observation 3: Whether SBFD is considered as a new feature affects the detailed methods of 4-step CBAR configuration.

Observation 4: There may be three types of SBFD-aware UE based on their capabilities, that is:

- Cap 1: SBFD-aware UE only supports Option 1 with Alt 1-1

- Cap 2: SBFD-aware UE only supports Option 2 with Alt 2-3

- Cap 3: SBFD-aware UE supports both Option 1 with Alt 1-1 and Option 2 with Alt 2-3.

- Observation 5: If there are SBFD-aware UE(s) which supports two RACH configuration Options, there are two options:

- Option 1: a flag can be introduced. FFS the flag is explicit or implicit

- Option 2: a prioritization rule can be configured. FFS how to design it

Observation 6: For initial PRACH transmission attempt in SBFD, Option 2 can bring better performance than Option 3, since it can achieve the gain of offloading PRACH transmission and reducing PRACH collision.

Observation 7: For PRACH transmission re-attempt, Option 2 is more flexible for selecting RACH resource than Option 1 and can control the PRACH transmission load on legacy-ROs and additional-ROs better than Option 3.

Observation 8: The gNB can identify a SBFD UE which transmits Msg1 in an additional RO which is in SBFD symbol.

Observation 9: If Msg1-based early indication is supported, gNB can identify a SBFD-aware UE early and thus schedule its subsequent transmission (e.g. Msg2/3/4) in SBFD symbol(s), but may reduce the PRACH capacity of non-SBFD-aware UEs.

Observation 10: For Msg1-based early indication, the degradation of PRACH capacity of non-SBFD-aware UEs seems to be acceptable because gNB has already offloaded PRACH transmission in SBFD symbols for SBFD-aware UEs.

Observation 11: With Msg3-based early indication, there is almost no impact on PRACH capacity of non-SBFD-aware UEs. However, gNB identifies whether the UE is an SBFD-aware UE a little later compared with using Msg1-based early indication.

Observation 12: If SBFD is considered as a new feature and feature combination framework is reused, Msg1-based early indication can be supported naturally.

Proposal 1: RAN2 should discuss whether both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported for all PRACH resource configuration signallings.

Proposal 2: RAN2 is asked to discuss whether SBFD is considered as a new feature before determining the solutions to configure 4-step CBRA.

Proposal 3: Based on Option 1 with Alt 1-1, there are two possible solutions to configure 4-step CBRA for SBFD-aware UEs:

- Solution 1: SBFD is considered as a new feature parallel to redcap, smalldata, etc. and 4-step CBRA configuration for SBFD-aware UE is provided with featureCombinationPreamblesList in rach-ConfigCommon.

- Solution 2: SBFD is not considered as a new feature, 4-step CBRA configuration for SBFD-aware UE shares the existing parameters configured in RACH-ConfigCommon.

- FFS other solutions

Proposal 4: Based on Option 2 with Alt 2-3, there are two possible solutions to configure 4-step CBRA for SBFD-aware UEs:

- Solution 1: SBFD is considered as a new feature and 4-step CBRA configuration for SBFD-aware UE is provided with featureCombinationPreamblesList in AdditionalRACH-Config-r17.

- Solution 2: SBFD is not considered as a new feature, a new IE rach-ConfigCommonSBFD-r19 which parallels to rach-ConfigCommon, is introduced.

- FFS other solutions

Proposal 5: For 4-step CFRA configuration based on Option 1 with Alt 1-1, RAN2 is asked to study whether/which parameter(s) need to be reinterpreted by SBFD-aware UEs in each signalling.

Proposal 6: For 4-step CFRA configuration based on Option 2 with Alt 2-3, a new IE parallel to each legacy IE can be introduced. FFS the behavior of SBFD-aware UEs when some parameters in new IE is absent

Proposal 7: To indicate which RACH configuration Option is enabled, there are two cases:

- Case 1: If each SBFD-aware UE can support only one RACH configuration Option, no additional work is required.

- Case 2: If there are SBFD-aware UE(s) which supports two RACH configuration Options, there are two options:

- Option 1: a flag can be introduced. FFS the flag is explicit or implicit

- Option 2: a prioritization rule can be configured. FFS how to design it

- Proposal 8: For initial PRACH transmission attempt in SBFD, Option 2 is supported, i.e., selecting one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations.

Proposal 9: For PRACH transmission re-attempt, Option 2 is supported, i.e., first using ROs of the same type as the earlier PRACH transmission for a certain number of times before using other type. FFS additional-ROs fallback to legacy-ROs and/or legacy-ROs fallback to additional-ROs

Proposal 10: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs. Whether supporting Msg1-based early indication depends on RAN1.

Reference

- “RAN1 Chair’s Notes”, RAN1#118, Maastricht, NL.