3GPP TSG RAN WG2 Meeting #127bis	R2-2408069

Hefei, Anhui Province, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.5

Source:	CMCC

Title:	Discussions on topology 2 for A-IoT

Document for:	Discussion

Introduction

In RAN2#127 meeting, the following agreements are made on Topology 2 for A-IoT:

- Agreements - 1 RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. - 2 A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. - 3 RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. - 4 RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) - 5 NW is in control of resources to be used by AIoT air interface. - 6 Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Then in this contribution, we would like to provide our views on the remaining issues for topology 2.

Discussion

### 2.1 General aspects

In RAN2#127 meeting, its has been agreed that RAN2 is responsible for the interface between intermediate node and RAN. Further with the consideration that only UE as the intermediate node is in scope of the revised SID [1], NR Uu naturally becomes one of the options. In order to minimize the workload of RAN’s study, we suggest that NR Uu should be reused as much as possible between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements, such as supporting resource allocation for A-IoT air interface, etc.

Proposal 1: NR Uu should be fully reused as the interface between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements.

### 2.2 RRC state of intermediate UE

In terms of the RRC state of intermediate UE, it should be analyzed according to the following two situations:

- Situation 1: the RRC state of the A-IoT enabled UE when initiate

- For sitution 1, the RRC state of A-IoT enbaled UE when initiate, that is, before it is selected as an intermediate, depends on the RRC state of A-IoT enbaled UE can be reached by CN or A-IoT enbaled gNB. To be specific, for the selection of intermediate UE, it may be conducted by A-IoT enabled gNB or CN. To be specific, for the intermediate UE selection dominated by the CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be reached. While, for the intermediate UE selection dominated by gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be reached directly. Note that gNB-dominated selection and CN-dominated selection are not contradictory, that is, they may exist simultaneously. While, note that upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or A-IoT service should be configured to it. Therefore, an direct and efficient solution is to transmit those parameters via dedicated RRC message after intermediate UE transferred to RRC_CONNECTED stste. For A-IoT capable UE in RRC_CONNECTED state, dedicated RRC message can be used to carry such parameter. For A-IoT capable UE in RRC_INACTIVE state, it can be resumed to RRC_CONNECTED by a RAN-initiated paging at first. And for A-IoT capable UE in RRC_IDLE state, it should be transfer to RRC_CONNECTED state by CN-initiated paging at first. For intermediate UEs in RRC_INACTIVE or RRC_IDLE state, dedicated RRC message which contains parameters related to A-IoT air interface resources or subsequent A-IoT procedure can be transmitted to them after they are transferred to RRC_CONNECTED. Based on above analysis, the A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs. While, for A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE, they should transfer to RRC_CONNECTED under network control before initiating A-IoT procedure.

Observation 1: CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Observation 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it. An direct and efficient solution is to transmit those parameters with dedicated RRC message after intermediate UE transferred to RRC_CONNECTED state.

Proposal 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it.

Proposal 3: The A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs.

Proposal 4: If A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE state is selected, it should be transferred to RRC_CONNECTED state before initiating A-IoT procedure.

- Situation 2: the RRC state of the intermediate UE when conducting A-IoT procedure

After an intermediate UE is selected, it conducts the inventory-only or command-only or inventory and command procedure according to the received parameters which indicates the resource allocation of A-IoT air interface and/or the subsequent A-IoT procedure. With the consideration that the feedback of A-IoT procedure should be transmitted to CN or application server and there may be some signallings transmitted from reader or CN to it when conducing A-IoT procedure, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard. To this end we suggest that RAN2 assumes the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure in R19.

Observation 3: With the consideration that the results of A-IoT service should be transmitted to CN or application server and there may be some signallings transmitted from reader or CN to it, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard.

Proposal 5: RAN2 assumes the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure in R19.

### 2.3 Resource allocation for intermediate UE

In RAN2#127 meeting, it has been agree that:

- 5 NW is in control of resources to be used by AIoT air interface.

- Based on legacy NR, dedicated or shared resources may be configured to intermediate UE to be used in A-IoT air interface. For shared resources, e.g. shared resource pool, intermediate UE needs to sense to avoid collision which increases the complexity to the implementation of intermediate UE. Besides, the shared resource pool is also a way of low resource utilization. To this end, shared resources should be deprioritized. For dedicated resources allocation, there may be following two possible options:

- Option 1: The intermediate UE receives the pre-configured A-IoT radio resources from A-IoT enabled gNB.

- For option 1, the intermediate UE does not need to transmit uplink message to request for the resources to be used by A-IoT air interface. For example, a dedicated RRC message which indicates the available A-IoT air interface resources may be transmitted to the intermediate UE in RRC_CONNECTED state if some parameters of A-IoT service (e.g. service type of A-IoT, approximate number of target devices etc.) are visible to the A-IoT enabled gNB.

- Option 2: The intermediate UE requests the A-IoT radio resources from A-IoT enabled gNB.

- For option 2, the intermediate UE need to transmit an assist message to request for the resources to be used by A-IoT air interface. For example, if an intermediate UE is selected by CN trough NAS message directly, thus the parameters related to A-IoT air interface resources allocation is invisible to the A-IoT enabled gNB. To this end, the intermediate UE should request the resources to be used by A-IoT air interface from A-IoT enabled gNB.

- Observation 4: For shared resources, e.g. shared resource pool, intermediate UE needs to sense to avoid collision which increases the complexity to the implementation of intermediate UE. Besides, the shared resource pool is also a way of low resource utilization.

- Proposal 6: Shared resources should be deprioritized for the configuration of resources to be used in A-IoT air interface to intermediate UE.

Proposal 7: For an intermediate UE, the methods to obtain A-IoT radio resources include:

- Option 1: The intermediate UE receives the configuration of A-IoT radio resources from A-IoT enabled gNB without request. However shared resource pool is not prefered.

- Option 2: The intermediate UE requests the A-IoT radio resources from A-IoT enabled gNB on demand.

Apart from this, the resources which is related to A-IoT service to be used in Uu interface should be configured by network as well. Note that the configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports the A-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs each time. Therefore the methods for an itermediate UE reports the results realted to A-IoT procedure need to be studied by RAN2. While for the configuration method of resources in Uu interface, it may depend on the architecture of Topology 2. While no matter which architecture is determined, there are legacy NR solutions which can be considered to reused.

- Observation 5: The configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports theA-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs each time.

Proposal 8: RAN2 should discuss the methods for an itermediate UE reports the results related to A-IoT procedure，i.e., one or multiple reporting.

Proposal 9: For the configuration of the resources which is related to A-IoT service in Uu interface, some legacy NR solutions can be reused. Details depend on the architecture of Topology 2.

### 2.4 Issues for “temporarily” out of connection cases

In RAN2#127 meeting, it has been agreed that:

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Note that for “temporarily” out of coverage cases where the intermediate can not connect to any A-IoT enabled gNB, belongs to Topology 4 which is not in the scope of current SID. For intermediate UE, “temporarily” out of connection cases occurs when its radio link fails or it conducts handover procedure, etc. In this case, the behavior of intermediate UE when it loses connection and after its connection recovers should be discussed.

- The behavior of intermediate UE when it loses connection.

- For the behavior of intermediate UE when it loses connection, there are three options.

- Option 1: Continue conducting the A-IoT related procedure, e.g. inventory-only or command-only or inventory and command procedure.

- Option 2: Suspend the A-IoT related procedure, e.g. inventory-only or command-only or inventory and command procedure..

- Option 3: Terminate the A-IoT related procedure, e.g. inventory-only or command-only or inventory and command procedure..

- Option 1 ensures the continuity of A-IoT procedure, but it may affect the performance resource management for A-IoT air interface, because the resources seem to become uncontrollable during the time when the intermediate UE loses connection, thus may cause serious interference. From our perspective, the corresponding use cases for A-IoT are not time-sensitive, thus the study of Option 1 can be deprioritized. For Option 2, the intermediate UE suspends the A-IoT related procedure in A-IoT air interface during the time when the intermediate UE loses connection. After its connection recoveries, it may resume or terminate the procedure following the indication form network, which will be described in detail in next bullet. For Option 3, the intermediate UE termediates the A-IoT related procedure directly. Based on above analysis, Option 2 and Option 3 is suggested to be studied in RAN2.

- Proposal 10: For the case that intermediate UE out of connection, three optional solutions can be considered:

- Option 1: Continue conducting the A-IoT related procedure.

- Option 2: Suspend the A-IoT related procedure.

- Option 3: Terminate the A-IoT related procedure.

Proposal 11: RAN2 assumes the intermediate UE terminates/suspends the A-IoT related procedure during the time when the intermediate UE out of connection.

- The behavior of intermediate UE after its connection recovers.

For Option 2 mentioned above where the intermediate UE suspends the A-IoT related procedure during the time when the intermediate UE loses connection, the reasonable behavior is to follow the indication of network after its connection recovers. Note that, after the connection of intermediate UE recovered, the resources used in A-IoT air interface may be reconfigured. For example, if a message which indicates the intermediate to continue the A-IoT related procedure with reconfigured resources is received, it resumes the subsequent procedure which has been suspended. Otherwise, it terminate the A-IoT related procedure and release A-IoT air interface resources. For Option 3 mentioned above, there is no need to discuss the behavior of intermediate UE after its connection recivers becaused the A-IoT related procedure has been terminated when the intermediate UE loses connection.

Observation 6: After the connection of intermediate UE recovered, the resources used in A-IoT air interface may be reconfigured.

Proposal 12: If the intermediate UE suspends the the A-IoT related procedure when it loses connection, it should follow the indication from network to determine either terminate or resume the subsequent procedure after its connection recovers.

### 2.5 Architecture and signalling/data transmission procedure

In 3GPP TR 23.700, some architectures and solutions for Topology 2 has been captured by SA2. In solution #33, NR Uu RRC layer is used to forward the NAS messages/data between intermediate UE and CN, which is called RRC-based solution in this paper. In solution #4/11/12/21/30, the NAS layer is the end-to-end layer between intermediate UE and CN, which is called NAS-based solution in this paper. In solution #3/4/22/32, PDU session-based solution is given. From our point of view, the architecture and signalling/data transmission procedure between intermediate UE and A-IoT enabled gNB are analyzed as following:

#### 2.5.1 RRC-based solution

For RRC-based solution, the signalling or data interacted between A-IoT enabled gNB and intermediate UE is RRC message. The A-IoT enabled gNB generates RRC messages based on the parameters provided by the CN when transmitting downlink signalling/data to intermediate UE. To this end, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from CN can be visible to A-IoT enabled gNB directly. The detailed protocol stack and corresponding procedure can be given in Figure 1 and Figure 2, respectively.

Figure 1: Protocol stack of Topology 2 with RRC-based solution

Figure 2: Procedure of RRC-based solution

1. The AIoT CN sends an Inventory request message to the AIoT-enabled gNB via NG/XX-AP.

- The AIoT-enabled gNB sends an Inventory response message to the AIoT CN via NG/XX-AP.

- The A-IoT enabled gNB generates and transmits a RRC message which contains inventory or command to intermediate UE.

- The intermediate UE transmits an A-IoT paging to A-IoT device(s) and conduct the corresponding procedure indicated in the received RRC message.

- A-IoT device transmits its response to intermediate UE.

- The intermediate UE transmits a RRC message with inventory report or command report to A-IoT enabled gNB.

- The A-IoT enabled gNB transmits a message with inventory report or command report to A-IoT CN in NG/XX-AP interface.

Note 1: The details of step 1, 2 and 7 should be discussed and determined by RAN3.

Note2: The configuration of intermediate UE for A-IoT air interface resources can be transmitted to intermediate without request before step 4, e.g. in step 3.

Observation 7: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Observation 8: For RRC-based solution, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from A-IoT CN can be visible to A-IoT enabled gNB directly.

Proposal 13: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 14: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

#### 2.5.2 NAS-based solution

In this case, A-IoT CN interacts with intermediate UE using NAS message directly. Considering that the NAS message is transparent for A-IoT enabled gNB, the intermediate UE need to request the resources used in A-IoT air interface from A-IoT enabled gNB, thus increases the signalling overhead compared to RRC-based solution. The protocol stack and corresponding procedure can be given in Figure 3 and Figure 4, respectively.

Figure 3: Protocol stack of Topology 2 with NAS-based solution

Figure 4: Procedure of NAS-based solution

- 1. The AMF sends a NAS message which contains inventory request or command request to intermediate UE.

- 2. The intermediate UE sends an NAS message contains inventory response or command response message to the AMF.

- 3. The intermediate UE sends a message to request the A-IoT air interface resources from A-IoT enabled gNB.

- 4. The A-IoT CN sends the configuration message to intermediate UE.

- 5. The intermediate UE sends the A-IoT paging to A-IoT device(s) and conducts the corresponding procedure indicated in the received inventory request or command request.

- 6. A-IoT device sends its response to intermediate UE.

- 7. The intermediate UE sends a NAS message which contains the inventory report or command report to AMF.

Note 1: The details of step 1, 2 and 7 may also be discussed and determined by RAN3/SA2.

Observation 9: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 10: For NAS-based solution, the intermediate UE may need to request the A-IoT radio resources from gNB when no available A-IoT air interface resources allocated, thus increases the signalling overhead compared to RRC-based solution.

Proposal 15: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 16: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

#### 2.5.3 PDU session-based solution

In this case, CN interacts with intermediate UE using user plane PDU transmitted in PDU session. Note that for PDU session-based solution, signalling transmission via control plane may also be needed because A-IoT air interface resources should be configured by A-IoT enabled gNB. The protocol stack and corresponding procedure can be given in Figure 5 and Figure 6, respectively.

Figure 5: Protocol stack of Topology 2 with PDU session-based solution

Figure 6: Procedure of PDU session-based solution

- 0.The PDU session is successfully established between Intermediate UE and UPF.

1. The UPF sends a inventory request or command request which a PDU session to intermediate UE.

2. The intermediate UE sends a inventory response or command response in a PDU session to UPF.

- 3. The intermediate UE sends a message to request the A-IoT air interface resources from A-IoT enabled gNB.

4. The A-IoT CN sends the configuration message to intermediate UE.

5. The intermediate UE sends the A-IoT paging to A-IoT device(s) and conducts the corresponding procedure indicated in the received inventory request or command request.

3. A-IoT device(s) sends its response to the intermediate UE.

4. The intermediate UE sends the inventory report or command report in a PDU session to UPF.

Note 1: The details of step 0, 1, 2 and 7 may also be discussed and determined by RAN3/SA2.

Note 2: signalling transmission via control plane may also be needed because A-IoT enabled gNB controls the resources to be used in A-IoT air interface which is not depicted in Figure 6.

Observation 11: For PDU session-based solution, CN interacts with intermediate UE using user plane PDU transmitted in PDU session.

Observation 12: For PDU session-based solution, signalling transmission via control plane may also be needed when A-IoT enabled gNB controls resource allocation.

Proposal 17: For Topology 2 protocol stack, RAN2 can capture Figure 5 as the baseline of the PDU session-based solution.

Proposal 18: RAN2 can capture the procedure of PDU session-based solution shown in Figure 6 as a baseline.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some considerations for topology 2 as following:

Observation 1: CN or A-IoT enabled gNB initiated intermediate UE selection are both feasible.

Observation 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it. An direct and efficient solution is to transmit those parameters with dedicated RRC message after intermediate UE transferred to RRC_CONNECTED state.

Observation 3: With the consideration that the results of A-IoT service should be transmitted to CN or application server and there may be some signallings transmitted from reader or CN to it, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard.

- Observation 4: For shared resources, e.g. shared resource pool, intermediate UE needs to sense to avoid collision which increases the complexity to the implementation of intermediate UE. Besides, the shared resource pool is also a way of low resource utilization.

- Observation 5: The configuration of A-IoT related resources in Uu interface is closely related to how an intermediate UE reports theA-IoT procedure feedback to network, e.g. one device ID each time or multiple device IDs each time.

Observation 6: After the connection of intermediate UE recovered, the resources used in A-IoT air interface may be reconfigured.

Observation 7: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Observation 8: For RRC-based solution, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since some parameters from A-IoT CN can be visible to A-IoT enabled gNB directly.

Observation 9: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 10: For NAS-based solution, the intermediate UE may need to request the A-IoT radio resources from gNB when no available A-IoT air interface resources allocated, thus increases the signalling overhead compared to RRC-based solution.

Observation 11: For PDU session-based solution, CN interacts with intermediate UE using user plane PDU transmitted in PDU session.

Observation 12: For PDU session-based solution, signalling transmission via control plane may also be needed when A-IoT enabled gNB controls resource allocation.

Proposal 1: NR Uu should be fully reused as the interface between A-IoT enabled UE and A-IoT enabled gNB with necessary enhancements.

Proposal 2: Upon one A-IoT enabled UE is selected as an intermediate UE, some parameters which are related to A-IoT air interface resources or subsequent A-IoT procedure should be transmitted to it.

Proposal 3: The A-IoT capable UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as intermediate UEs.

Proposal 4: If A-IoT capable UEs in RRC_IDLE or RRC_INACTIVE state is selected, it should be transferred to RRC_CONNECTED state before initiating A-IoT procedure.

Proposal 5: RAN2 assumes the intermediate UE in RRC_CONNECTED state when conducting A-IoT procedure in R19.

- Proposal 6: Shared resources should be deprioritized for the configuration of resources to be used in A-IoT air interface to intermediate UE.

Proposal 7: For an intermediate UE, the methods to obtain A-IoT radio resources include:

- Option 1: The intermediate UE receives the configuration of A-IoT radio resources from A-IoT enabled gNB without request. However shared resource pool is not prefered.

- Option 2: The intermediate UE requests the A-IoT radio resources from A-IoT enabled gNB on demand.

Proposal 8: RAN2 should discuss the methods for an itermediate UE reports the results related to A-IoT procedure，i.e., one or multiple reporting.

Proposal 9: For the configuration of the resources which is related to A-IoT service in Uu interface, some legacy NR solutions can be reused. Details depend on the architecture of Topology 2.

- Proposal 10: For the case that intermediate UE out of connection, three optional solutions can be considered:

- Option 1: Continue conducting the A-IoT related procedure.

- Option 2: Suspend the A-IoT related procedure.

- Option 3: Terminate the A-IoT related procedure.

Proposal 11: RAN2 assumes the intermediate UE terminates/suspends the A-IoT related procedure during the time when the intermediate UE out of connection.

Proposal 12: If the intermediate UE suspends the the A-IoT related procedure when it loses connection, it should follow the indication from network to determine either terminate or resume the subsequent procedure after its connection recovers.

Proposal 13: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 14: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

Proposal 15: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 16: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

Proposal 17: For Topology 2 protocol stack, RAN2 can capture Figure 5 as the baseline of the PDU session-based solution.

Proposal 18: RAN2 can capture the procedure of PDU session-based solution shown in Figure 6 as a baseline.

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, Maastricht, Netherlands, CMCC, Huawei, T-Mobile USA.