3GPP TSG-RAN WG2 Meeting #127bis		R2-2408071

Hefei, China, Oct 14th – 18th, 2024

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda item:	8.4.3

Document for:	Discussion

- Introduction

At the RAN2#127 meeting, we reached the following agreements[1]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the LR measurement results.

- Discussion

In the LP-WUS discussion of RAN4, they listed several cases for measurement offloading/relaxation. In order to avoid duplication of work and conduct efficient discussions within a limited time, RAN2 should prioritize research based on the cases approved by RAN4.

Proposal 1: RAN2 should prioritize research based on the cases approved by RAN4.

2.1 Measurement relaxation/offloading of serving cell

Since RRM measurements in IDLE/INACTIVE state are mainly used for cell (re)selection, MR measurement relaxation/offloading of serving cell should have a premise that it cannot affect the cell (re)selection performance of UE.

Proposal 2: Serving cell measurement relaxation/offloading of MR cannot affect the performance of cell (re)selection by UE.

Measurement offloading

According to the conclusions of RAN2#127 and RAN4#111, measurement offloading is supported:

RAN4#111  Agreement

- Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN2#126  Agreement

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

In the 'Fully offloading case' agreed by RAN4, MR serving and neighboring cell measurement are stopped. In our understanding, in order not to affect the existing cell (re)selection process in TS 38.304, this means that the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements. Otherwise, the MR cannot stop measuring neighboring cells. As shown in Figure 1.

Figure 1 measurement offloading to LR

Proposal 3: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Considering that the latency of enabling MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 4: UEs with high-mobility do not perform serving cell measurement offloading.

When the UE stops measurement offloading and turns on MR again, the delay between these two actions can be as high as hundreds of milliseconds, which is unprecedented. So RAN2 can discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR. For example, LR will not completely stop measuring until MR is turned back on

Proposal 5: RAN2 discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR.

measurement relaxation

According to the conclusions of RAN4#112, RRM measurement relaxed is supported.

RAN4#112  Agreement

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Similarly as measurement offloading, an RSRP threshold can be predefined. In addition, since the MR is not turned off, the mobility of the UE may not need to be considered in this case. Since the MR in this case will perform relaxation measurements on the serving cell and the neighboring cell, the threshold for measurement relaxation should be less than the threshold for the UE to start neighboring cell measurement. As shown in Figure 2.

Figure 2 measurement relaxation

In addition, since the types and accuracy of MR and LR may be different, the measurement results of MR and LR should not be combined, but should be processed as independent measurement quantities. And considering that MR is crucial to the measurement of neighboring cells in cell (re)selection, the entry/exit conditions of measurement relaxation should be mainly based on the measurement quality of MR.

Proposal 6: For serving cell measurement relaxation:

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the MR measurement results.

Proposal 7: The threshold for measurement relaxation is less than the threshold for starting neighboring cell measurements.

Proposal 8: Since the types and accuracy of MR and LR may be different, the measurement results of MR and LR can not be combined.

2.2 Measurement relaxation of neighboring cell

In last meeting, we agreed that the RRM measurement of the neighboring cell can only be performed by MR. Therefore, the neighbor cell measurement relaxation mechanism in Release 16 can be reused, which includes two scenarios: not at cell edge and low mobility, respectively.

According to TS 38.133, if MR performs serving cell measurement offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism for equal or lower priority neighboring cells because the measurement will not be turned on at this time. But for higher priority cells, the corresponding protocol in TS 38.133 is as follows:

TS 38.133  Measurements for UE fulfilling low mobility and not-at-cell edge criterion

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-frequency layers of higher, equal or lower priority at least every 1 hour.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=60.

Therefore, for not-at-cell edge criterion, if the offloading threshold is greater than SnonIntraSearchP, the UE can implicitly determine whether the R16 measurement relaxation condition is met based on whether the serving cell performs measurement offloading. However, when the UE performs serving cell measurement offloading, it needs to use the LR measurement results to evaluate the low mobility criterion of the higher priority cells.

Observation 1: For equal or lower priority neighboring cells, if MR performs serving cell offloading to LR and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism.

Observation 2: For higher priority neighboring cells, if MR performs serving cell offloading to LR and the offloading threshold is greater than SnonIntraSearchP, it will not affect the not-at-cell edge criterion of the existing R16 measurement relaxation mechanism, but the LR measurement results need to be used to evaluate the low mobility criterion.

If the threshold does not meet the above conditions, or MR and LR are used together for serving cell measurement, it will affect the existing R16 measurement relaxation mechanism, as shown in Figure 3. That is, when evaluating the relaxation conditions of neighboring cell measurements in R16, it may be necessary to use the measurement results of LR.

Figure 3 SnonIntraSearchP>Threshold>SSearchThresholdP

In this case, all higher, equal or lower priority neighboring cells need to use LR measurements to evaluate the R16 measurement relaxation criterion.

Observation 3: If the serving cell measurement offloading threshold is less than SnonIntraSearchP, or both MR and LR doing measurement, all higher, equal or lower priority neighboring cells need to use LR measurement results to evaluate the R16 measurement relaxation criterion.

Proposal 9: RAN2 discusses support for using LR-based serving cell measurement results to evaluate R16 measurement relaxation criterion.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are the observations and proposals.

Observations:

Observation 1: For equal or lower priority neighboring cells, if MR performs serving cell offloading to LR and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism.

Observation 2: For higher priority neighboring cells, if MR performs serving cell offloading to LR and the offloading threshold is greater than SnonIntraSearchP, it will not affect the not-at-cell edge criterion of the existing R16 measurement relaxation mechanism, but the LR measurement results need to be used to evaluate the low mobility criterion.

Observation 3: If the serving cell measurement offloading threshold is less than SnonIntraSearchP, or both MR and LR doing measurement, all higher, equal or lower priority neighboring cells need to use LR measurement results to evaluate the R16 measurement relaxation criterion.

Proposals:

Proposal 1: RAN2 should prioritize research based on the cases approved by RAN4.

Proposal 2: Serving cell measurement relaxation/offloading of MR cannot affect the performance of cell (re)selection by UE.

Proposal 3: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Proposal 4: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 5: RAN2 discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR.

Proposal 6: For serving cell measurement relaxation:

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the MR measurement results.

Proposal 7: The threshold for measurement relaxation is less than the threshold for starting neighboring cell measurements.

Proposal 8: Since the types and accuracy of MR and LR may be different, the measurement results of MR and LR can not be combined.

Proposal 9: RAN2 discusses support for using LR-based serving cell measurement results to evaluate R16 measurement relaxation criterion.

- References

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD