3GPP TSG-RAN WG2 Meeting #127bis        	                       R2-2408077

Hefei, China, October 14th-18th, 2024

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the NW side data collection and achieved the following agreements.

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

This contribution will discuss the remaining issues for NW side data collection.

- Discussion

- 2.1 NW side data collection for beam management

- Issue#1: FFS on multiple configurations and dynamic activation/deactivation

- In legacy CSI measurement/report mechanism, there can be multiple CSI configurations provided to the UE, and the UE would report multiple CSI reports to the network. For training data collection for beam management, the similar mechanism can be reused.

- Proposal 1：There can be multiple measurement configurations provided to the UE.

- In current immediate MDT mechanism, the UE follows the RRM measurement behavior, i.e., the UE starts to collect measurement data upon reception of the measurement configuration. We think the motivation and gain of introducing dynamic activation/deactivation is unclear, and prefer that the UE starts the measurement immediately upon reception of the measurement configuration.

- Proposal 2: The UE starts the data collection immediately upon reception of the measurement configuration. The dynamic activation/deactivation is not support.

- Issue#2: FFS on memory size

- RAN2 agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. In legacy logged MDT and QoE measurement, the minimum AS layer memory size is 64KB [2]. We prefer to follow the legacy value for logging training data.

- TS 38.306, Section 4.2.18 Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. - TS 38.306, Section 6 Features; Condition. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

- Proposal 3: The minimum AS layer memory size for training data logging is 64KB, which is the same as the existing logged MDT and QoE measurement.

- Issue#3: FFS on event based/on-demand reporting

- In last meeting, RAN2 agreed that measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- In legacy logged MDT, when UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. If the power state of the UE is low, we think that the UE can stop autonomously, but it needs to report to the network, otherwise, there is misunderstanding between UE and NW.

- Proposal 4: If the power state of the UE is low, the UE can stop measurement autonomously.

- In legacy logged MDT mechanism, the UE sends availability indication to the network, and the network may decide to retrieve the logged measurements based on this indication via UE Information procedure.

- Figure 1. UE information procedure

- RAN2 has agreed to support the periodical reporting and on-demand reporting, but has no consensus on event based reporting. In our understanding, when the event is satisfied if event based reporting is supported, the UE will send BSR to the network for UL transmission scheduling. However, the network cannot be aware that the data is for training which is usually not urgent, and the transmission of logged data may cause congestion in the network. If the availability indicator for both AS full buffer and low power state is introduced to indicate the availability of logged data for training, the network could know the logging in UE side is stopped and determine when to request the data considering the current channel condition which can improve the flexibility of network scheduling.

- Therefore, we think it is no need to introduce event based reporting, e.g. AS buffer, power state event based reporting. We support to use the availability indicator to indicate the availability of logged data for training, e.g. when the AS buffer is full or the power state is low.

- Proposal 5: Event based reporting is not supported.

- Proposal 6: Introduce availability indicator to indicate the availability of logged data for training, e.g. when the AS buffer is full or the power state is low.

- Issue#4: FFS on on-demand request from NW

- In our understanding, on-demand request from the network means that the network can request the UE to report collected data at any time which is up to network implementation. And this is not associated with the availability indicator. The legacy UE information procedure can be reused for on-demand request from the network.

- Proposal 7: The legacy UEInformationRequest/UEInformationResponse message can be reused for on-demand request from the network.

- Issue#5: FFS on SRB

- For NW side data collection, the data for training is not urgent, thus there is no need to use the high priority SRB for transmission. As specified in TS 38.331[3], logged MDT measurements are sent on SRB2 in RRC_CONNECTED state. In last meeting, some companies proposed to use lower priority SRB to transmit the logged data. However, we also think that periodic reporting is useless if the low priority SRB is used. Therefore, we prefer to reuse SRB2 to send the logged data for training.

- Proposal 8: SRB2 is used to transmit the logged data for training.

- Issue#6: FFS on logged time

- In legacy logged MDT, the LoggingDuration and LoggingInterval will be provided to the UE, where LoggingDuration indicates the duration for which UE is requested to perform measurement logging, and LoggingInterval indicates the periodicity for logging measurement results.

- In previous meeting, RAN2 agreed the periodical reporting for NW side data collection. Therefore, we understand that there can be multiple reports within the logged duration, and each report can include multiple logged L1 measurements. For example, the logging interval is 80ms, the reporting period is 240ms which includes 3 logged measurement results, and the logged duration is 960ms which includes 4 reports.

- Figure 2. The relationship between logging interval, reporting period and logged duration

- For temporal beam prediction, the time info is necessary for model training. In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [3]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [3]) which indicates the time of logging measurement results would be included. The similar principle can be reused for training data reporting to reduce the signaling overhead.

- Proposal 9: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logging measurement results, similar as logged MDT.

- 2.2 NW side data collection for Positioning

- 2.2.1 Positioning with gNB-sided model (case 3a)

- Figure 3. Positioning with gNB-sided model (case 3a)

- During the R18 study, the entity for gNB-sided model training can reside at gNB or OAM. Based on the following RAN1 agreement, the label and its related data (e.g., time stamp) can be generated by at least LMF. Thus, the NRPPa should be enhanced to transmit the label and related data from LMF to gNB for model training.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

- Proposal 10: For positioning with gNB-sided model (case 3a), the enhancement on NRPPa maybe needed to transmit the data (e.g. the label and its related data) from LMF to gNB for model training, the details should be discussed in RAN3.

- Based on the following RAN1 agreement, the measurement for case 3a is based on the existing UL SRS and no new SRS will be introduced. Therefore, the legacy UL SRS transmitting and measurement will be reused, which has no additional impacts on Uu interface.

RAN1#116 Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE.

- Proposal 11: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface.

- 2.2.2 Positioning with LMF-sided model (case 3b)

- During the R18 study, the entity for LMF-sided model training resides at LMF and the LMF is the termination point for training data. Therefore the LMF-centric data collection should be considered, i.e. LMF configures the UE/gNB to initiate/terminate the data collection procedure.

- Figure 4. Positioning with LMF-sided model (case 3b)

For the content of collected data for LMF-sided model training, at least measurements (corresponding to model input, e.g. timing, power, and/or phase info) and location coordinates as model output are needed based on RAN1 reply LS [4]. During the R19 discussion, RAN1 has achieved the following agreements on the collected data for LMF-sided model.

RAN1#116 Agreement - For AI/ML based positioning case 2b and 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time RAN1#116bis Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

- In general, the training data can be generated by gNB and terminated at LMF. Hence, it is straight forward to extend the NRPPa protocol to collect data for LMF-sided model training which should be discussed in RAN3.

- Proposal 12: For positioning with LMF-side model (case 3b), it is up to RAN3 whether to extend the NRPPa protocol to collect data from gNB to LMF for model training.

# 3	Conclusion

Here are the proposals for NW side data collection.

Beam management

- Proposal 1：There can be multiple measurement configurations provided to the UE.

- Proposal 2: The UE starts the data collection immediately upon reception of the measurement configuration. The dynamic activation/deactivation is not support.

- Proposal 3: The minimum AS layer memory size for training data logging is 64KB, which is the same as the existing logged MDT and QoE measurement.

- Proposal 4: If the power state of the UE is low, the UE can stop measurement autonomously.

- Proposal 5: Event based reporting is not supported.

- Proposal 6: Introduce availability indicator to indicate the availability of logged data for training, e.g. when the AS buffer is full or the power state is low.

- Proposal 7: The legacy UEInformationRequest/UEInformationResponse message can be reused for on-demand request from the network.

- Proposal 8: SRB2 is used to transmit the logged data for training.

- Proposal 9: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logging measurement results, similar as logged MDT.

Positioning

- Proposal 10: For positioning with gNB-sided model (case 3a), the enhancement on NRPPa maybe needed to transmit the data (e.g. the label and its related data) from LMF to gNB for model training, the details should be discussed in RAN3.

- Proposal 11: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface.

- Proposal 12: For positioning with LMF-side model (case 3b), it is up to RAN3 whether to extend the NRPPa protocol to collect data from gNB to LMF for model training.

# 4	Reference

- Draft_R2_127_Meeting_Report_v1

- TS 38.306 NR; User Equipment (UE) radio access capabilities

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- R2-2311720 Reply LS on Data Collection Requirements and Assumptions (R1-2310681)