3GPP TSG RAN WG2 Meeting #127bis	R2-2408080

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.8.4

Source:	CMCC

Title:	Discussion on MBS broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we would like to provide some further considerations on the MBS broadcast service for NTN system.

Discussion

How to provide service area information

- In last meeting, we discussed signaling details on how to provide service area information online and offline but without convergence (offline [2]) summary: SIB20 vs MCCH: 7:6. The main concerns of SIB20 are signaling overhead and extra modification of SIB20 caused by service area info. update. And concern on using MCCH is UE power consumption due to always monitor MCCH update.

- Per the current MBS broadcast mechanism, MCCH carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSBroadcastConfiguration message may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. The configuration information required by the UE to receive MCCH is provided in SIB1 and SIB20.

- From our perspective, the main advantage of SIB20 is power saving that UE could determine whether to monitor the MCCH based on the service area information. However, this solution would have impact on all UEs due to service area info. update, which may lead to more signaling overhead.

- Then for MCCH, MBSBroadcastConfiguration including mbs-SessionInfoList which provides the configuration of each MBS session provided by MBS broadcast in the current cell, considering we have another agreement that when intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session, therefore, using MBSBroadcastConfiguration could provide both service area information and association between service area and MBS session. Specifically, introduce intended service area ID and provide association (e.g. one to one, one to multiple, multiple to one) between intended service area ID and MBS session ID.

- As in current TS 38.331, the MCCH information validity and notification of changes are defined as following:

5.9.1.3 MCCH information validity and notification of changes Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). When the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH which schedules the MCCH in every repetition in that modification period. Upon receiving a change notification, a UE receiving or interested to receive MBS services transmitted using MBS broadcast acquires the new MCCH information starting from the same slot. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information. The notification is transmitted with a 2-bit bitmap, see TS 38.212 [17] clause 7.3.1.5.1. The MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBS service(s). The LSB in the 2-bit bitmap, when set to '1', indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g. modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighbouring cell information modification.

- Then for MCCH update due to service area information changing, considering that we would specify the association between the intended service area and MBS session, the MCCH update mechanism could follow legacy solution mentioned above.

- Proposal 1: It is proposed to include service area information in MBSBroadcastConfiguration

- Proposal 2: Introduce intended service area ID and provide association (e.g. one to one, one to multiple, multiple to one) between intended service area ID and MBS session ID.

- Proposal 3: For MCCH update due to service area information changing, legacy mechanism could be the baseline.

- For ETWS, we have agreed to send an LS [3] to SA1 to check whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification. Therefore we could wait the feedback from SA1 for provision service area information for ETWS.

Proposal 4: Wait the feedback from SA1 for provision service area information for ETWS.

Signaling format of service area

Regarding the service area indication, we agreed that the intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

- For the former solution, similar to TN coverage information introduced in NTN R18 to assist TN neighbour cell measurements, we could consider to introduce a new IE (e.g. IntendedServiceAreaInfo-r19) as follows to provide intended service area ID, service area reference location and distance radius information to UE. Then UE receive the broadcast service based on whether in the service area or whether interested in the service.

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Proposal 5: For the referenceLocation + radius solution, the following new IE format could be discussed:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Then for the latter solution (polygon), it is already captured in section 5.4 of TS 23.032.

- 5.4 Polygon A polygon is an arbitrary shape described by an ordered series of points (in the example pictured in the drawing, A to E). The minimum number of points allowed is 3, and the maximum number of points allowed is 15. The points shall be connected in the order that they are given. A connecting line is defined as the line over the ellipsoid joining the two points and of minimum distance (geodesic). The last point is connected to the first. The list of points shall respect a number of conditions: - a connecting line shall not cross another connecting line; - two successive points must not be diametrically opposed on the ellipsoid. The described area is situated to the right of the lines with the downward direction being toward the Earth's centre and the forward direction being from a point to the next. NOTE: This definition does not permit connecting lines greater than roughly 20 000 km. If such a need arises, the polygon can be described by adding an intermediate point. Computation of geodesic lines is not simple. Approximations leading to a maximum distance between the computed line and the geodesic line of less than 3 metres are acceptable. Figure 3: Description of a Polygon

To support both the solutions, a “CHOICE” signaling structure could be considered. And based on NW configuration to provide the intended service area information

Proposal 6: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

Country information

Per the discussion, some companies indicate that one intended service area may cross several country border, different country information need to be taken into account. However, the existing TMGI (Temporary Mobile Group Identity) used within MBMS to uniquely identify Multicast and Broadcast bearer services and the structure of TMGI is shown in the following figure:

Figure 15.2.1: Structure of TMGI [4]

And the SAI is constituted by Service Area Code (SAC) together with the PLMN-Id and the LAC (Location Area Code):

-	SAI = PLMN-Id || LAC || SAC [4]

Based on above, current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to represent different country and no further enhancements is needed.

Proposal 7: Current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to cover different country information.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following proposals:

- Proposal 1: It is proposed to include service area information in MBSBroadcastConfiguration

- Proposal 2: Introduce intended service area ID and provide association (e.g. one to one, one to multiple, multiple to one) between intended service area ID and MBS session ID.

- Proposal 3: For MCCH update due to service area information changing, legacy mechanism could be the baseline.

Proposal 4: Wait the feedback from SA1 for provision service area information for ETWS.

Proposal 5: For the referenceLocation + radius solution, the following new IE format could be discussed:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Proposal 6: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

Proposal 7: Current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to cover different country information.

Reference

- R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM

- R2-2407619_Summary of [AT127][307][R19 NR NTN] (Apple)

- R2-2407823	LS on requirements for ETWS primary notification	Ericsson	LS 	Rel-19	NR_NTN_Ph3-Core	To:SA1

- 3GPP TS 23.003 V18.4.0 Numbering, addressing and identification; (Release 18)