3GPP TSG RAN WG2 Meeting #127bis	R2-2408081

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.8.6

Source:	CMCC

Title:	Discussion on left issues of LTE to NR mobility

Document for:	Discussion, Discussion

Introduction

- There were some discussions on LTE to NR NTN mobility in last RAN2 meeting, and agreements were reached as following [1]:

Agreements:

1.	Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

2.	The ntn-PolarizationalDL is optional.

3.	Reuse the SatelliteId-r18 to identify either an NR satellite or an IoT NTN satellite

4.	Consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells

5.	maxSat-r17 (4) is reused for the maximum number of NR satellites.

6.	RAN2 will not do further work to introduce multiple SMTCs in LTE

7.	NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

8.	RAN2 confirms that measurements of NR NTN cells for a UE in E-UTRAN TN RRC_INACTIVE are supported, with the understanding that UE moves to RRC idle upon selecting the NR NTN cell

9.	Introduce a new UE capability without signalling for LTE TN to NR NTN mobility.

In this contribution, we would like to provide some further discussion on the left issues of cell reselection enhancements from E-UTRAN TN to NR-NTN.

Discussion

Stage 2 description

In last meeting, a baseline content of stage 2 running CR [2] is endorsed and needed further check in this meeting whether to put the change elsewhere. Regarding current description structure, a new separate inter-RAT cell reseletion section is added as following:

From our perspective, the current structure is acceptable to us. However, another alternative could also be considered, which is to put the change above in 10.2.1 cell reselection directly.

Proposal 1: Discuss between current description structure in [2] and put the change above in 10.2.1 cell reselection directly.

Proposal 2: If the latter solution in proposal 1 is agreed, suggest RAN2 to adopt the TP in annex.

Stage 3 aspects

For stage 3 aspects, there are two remaining issues as follows:

R2-2407812 Summary of [AT127][305][LTE_TN_NR_NTN_mob] RRC CR CATT disc Rel-19 LTE_TN_NR_NTN_mob Proposal 1: RAN2 agrees signaling Alternative 1 to indicate ephemeris data for a satellite supporting both NR NTN and IoT NTN, i.e. introducing a Satellite reference ID that refers to the ephemeris data of an IoT NTN satellite in the existing list neighSatelliteInfoList-r18, and using the "CHOICE" signaling structure including this Satellite reference ID and explicit ephemeris data configuration as the two choices. Come back in the next meeting Proposal 2: RAN2 sticks to the agreement that only duplication of ephemeris data needs to be avoided in the case of same satellite supporting both NR NTN and IoT NTN. Come back in the next meeting

For the first issue, two alternatives signaling structure were provided in [3]. During the offline discussion, the majority (9 out of 12) prefer alt A, i.e. introducing a Satellite reference ID (satelliteRefId) to support a satellite providing both IoT-NTN and NR NTN service case. Opponents indicated that for a same satellite, just use the existing satelliteId is sufficient and beneficial for signaling overhead reduction. From our point of view, considering the case that a same satellite supporting both IoT-NTN and NR NTN simultaneously, alt A is more cleaner than alt B, using a satellite reference ID to distinguish different NTN service case. However, compared to a "CHOICE" signaling structure to include this Satellite reference ID and explicit ephemeris data configuration as the two choices, maybe using a "SEQUENCE" signaling structure (as shown below) is better to apply to the scenario that a same satellite providing both IoT-NTN and NR NTN services.

NeighSatelliteInfoNR-r19::=	SEQUENCE {

satelliteId-r19				SatelliteId-r18,

ephemerisInfo-r19				SEQUENCE {

ephemerisInfoNR-r19				CHOICE {

stateVectors-r19				EphemerisStateVectors-r17,

orbitalParameters-r19			EphemerisOrbitalParameters-r17

},

satelliteRefId			SatelliteId-r18

},

nta-CommonParametersNR-r19		SEQUENCE {

nta-CommonNR-r19			    INTEGER (0.. 66485757)     OPTIONAL,	-- Need OP

nta-CommonDriftNR-r19			INTEGER (-257303..257303)	OPTIONAL,	-- Need OP

nta-CommonDriftVariationNR-r19	INTEGER (0..28949)			OPTIONAL	-- Need OP

},

epochTime-r19					SEQUENCE {

startSFN-r19					INTEGER (0..1023),

startSubFrame-r19				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r19						INTEGER (1..512)				OPTIONAL,	-- Need OP

ntn-PolarizationDL-r19			ENUMERATED {rhcp,lhcp,linear}	OPTIONAL	-- Need OR

}

Proposal 3: Suggest modified Alt A with a "SEQUENCE" instead of "CHOICE"signaling structure to include the Satellite reference ID and explicit ephemeris data configuration for a same satellite supporting both NR NTN and IoT NTN.

For the second issue, companies raised that other parameters (e.g. t-ServiceStartNeigh, K-Mac) than ephemeris could also be reused. Currently, we just agreed to avoid repeating the ephemeris for a satellite providing both IoT NTN and NR NTN services. Therefore reusing other information is not considered unless necessary.

4.	Consider a solution that avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells

Proposal 4: Confirm that only avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on mobility enhancements for E-UTRAN TN to NR-NTN and have the following proposals:

Proposal 1: Discuss between current description structure in [2] and put the change above in 10.2.1 cell reselection directly.

Proposal 2: If the latter solution in proposal 1 is agreed, suggest RAN2 to adopt the TP in annex.

Proposal 3: Suggest modified Alt A with a "SEQUENCE" instead of "CHOICE"signaling structure to include the Satellite reference ID and explicit ephemeris data configuration for a same satellite supporting both NR NTN and IoT NTN.

Proposal 4: Confirm that only avoids repeating the ephemeris for a satellite which provides both IoT NTN and NR NTN cells.

Reference

- R2-127 NR-NTN-IoT-NTN (Sergio)_ EOM

- R2-2407616	Stage 2 Running CR for E-UTRAN to NR NTN mobility	Samsung	draftCR	Rel-19	36.300	18.2.0	LTE_TN_NR_NTN_mob-Core

- R2-2407812	Summary of [AT127][305][LTE_TN_NR_NTN_mob] RRC CR	CATT	disc	Rel-19		LTE_TN_NR_NTN_mob

- R2-2407617	Introduction of LTE TN to NR NTN IDLE mode mobility (Option 2)	CATT	draftCR	Rel-19	36.331	18.2.0	B	LTE_TN_NR_NTN_mob

Annex

- TP for stage 2:

CHANGE BEGIN

  - 10.2	Inter RAT

  - 10.2.0	General

Service-based redirection between GERAN / UTRAN / NR and E-UTRAN is supported in both directions. This should not require inter-RAT reporting in RRC CONNECTION REQUEST.

  - 10.2.1	Cell reselection

A UE in RRC_IDLE performs cell reselection. The principles of this procedure are as follows:

-	The UE makes measurements of attributes of the serving and neighbour cells to enable the reselection process:

-	For a UE to search and measure neighbouring GERAN cells, the ARFCNs of the BCCH carriers need to be indicated in the serving cell system information (i.e., an NCL). The NCL does not contain BSICs or cell specific offsets and Qrxlevmin is given per frequency band.

-	For a UE to search and measure neighbouring UTRAN cells, the serving cell can indicate an NCL containing a list of carrier frequencies and scrambling codes.

-	For a UE to search and measure neighbouring NR cells, the serving cell can indicate the measured RS types and parameters for cell quality derivation.

-	Measurements may be omitted if the serving cell attribute fulfils particular search or measurement criteria.

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	Inter-RAT reselection is based on absolute priorities where UE tries to camp on highest priority RAT available. Absolute priorities for inter-RAT reselection are provided only by the RPLMN and valid only within the RPLMN; priorities are given by the system information and valid for all UEs in a cell, specific priorities per UE can be signalled in the RRC Connection Release message. A validity time can be associated with UE specific priorities.

-	It should be possible to prevent the UE from reselecting to specific detected neighbouring cells;

-	The UE is allowed to "leave" the source E-UTRAN cell to read the target GERAN cell broadcast, in order to determine its "suitability", prior to completing the cell reselection;

-	Cell reselection can be speed dependent (speed detection based on UTRAN solution);

Cell access restrictions apply as for UTRAN, which consist of access class (AC) barring and cell reservation (e.g. for cells "reserved for operator use") applicable for mobiles in RRC_IDLE mode.

When performing cell reselection while the UE is camped on another RAT, the principles of this procedure are as follows:

-	The UE measures attributes of the E-UTRA neighbouring cells:

-	Only the carrier frequencies need to be indicated to enable the UE to search and measure E-UTRA neighbouring cells;

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	For E-UTRA neighbouring cells, there is no need to indicate cell-specific cell reselection parameters i.e. these parameters are common to all neighbouring cells on an E-UTRA frequency;

-	Cell reselection parameters are applicable to all UEs in a cell, but it is possible to configure specific reselection parameters per UE group or per UE.

-	It should be possible to prevent the UE from reselecting to specific detected neighbouring cells.

E-UTRAN supports inter-RAT cell reselection in RRC_IDLE and RRC_INACTIVE to NR NTN. E-UTRAN provides NR NTN payload assistance information to assist UEs in searching and measuring NR NTN cells.

END OF CHANGE