3GPP TSG RAN WG2 Meeting #127bis	R2-2408083

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.9.4

Source:	CMCC

Title:	Discussion on broadcast of PWS messages for NB-IoT

Document for:	Discussion, Decision

Introduction

In RAN #105, PWS for NB-IoT has been discussed and supported finally, then the WID scope has been updated to cover the objective [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we would like to provide some discussion on broadcast of PWS messages for NB-IoT NTN system.

Discussion

ETWS (Earthquake and Tsunami Warning System)

As we known, in R19 NR NTN, an LS about whether support geofence of ETWS emergency messages to an area is sent to SA1 in last RAN2 meeting. Then regarding the intended service area for ETWS, we could just wait for NR NTN agreements.

Proposal 1: It is proposed to wait for NR NTN agreements on intended service area for ETWS.

As in current TS 36.331, the IE SystemInformationBlockType10 contains an ETWS primary notification and SystemInformationBlockType11 contains an ETWS secondary notification. For NB-IoT, we could consider to introduce two new SystemInformationBlockTypexx-NB to include the ETWS primary notification and ETWS secondary notification respectively.

Proposal 2: Similar to existing TS 36.331, introduce two new SystemInformationBlockTypexx-NB to include the ETWS primary notification and ETWS secondary notification for NB-IoT respectively.

CMAS (Commercial Mobile Alert System)

The IE SystemInformationBlockType12 contains a CMAS notification. And CMAS already supports the geo-fencing message with the warningAreaCoordinatesSegment filed in SystemInformationBlockType12, which could be acted as the service area. Therefore, a new SystemInformationBlockTypexx-NB could also be defined to include the CMAS notification for NB-IoT and no additional service area indication is needed.

Proposal 3: Introduce a new SystemInformationBlockTypexx-NB to include the CMAS notification for NB-IoT and no additional service area indication is needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on Discussion on broadcast of PWS messages for NB-IoT and have the following observations and proposals:

Proposal 1: It is proposed to wait for NR NTN agreements on intended service area for ETWS.

Proposal 2: Similar to existing TS 36.331, introduce two new SystemInformationBlockTypexx-NB to include the ETWS primary notification and ETWS secondary notification for NB-IoT respectively.

Proposal 3: Introduce a new SystemInformationBlockTypexx-NB to include the CMAS notification for NB-IoT and no additional service area indication is needed.

Reference

- RP-242397 WID Rev R19 IOT-NTN cl

- 3GPP TS 36.300 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)