3GPP TSG-RAN WG2 Meeting #127bis	R2-2408088

# Hefei, China, Oct 14th – 18th, 2024

# Agenda item:	8.6.4

# Source:	CMCC

# Title:	Discussion on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#105 meeting, the objective related to conditional LTM of Rel-19 Mobility enhancements was agreed, following is the related description [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we provides our view on conditional LTM.

- Discussion

In Rel-16, conditional handover was first introduced in NR to improve handover reliability and robustness. In TS 38.300 [2], CHO is defined as following:

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and stops evaluating the execution condition(s) once a handover is executed.

When it comes to conditional LTM, the similar definition can be described: a conditional LTM is defined as a LTM that is executed by the UE when or more LTM execution conditions are met.

Proposal 1: The definition of conditional LTM is as following:

A conditional LTM is defined as a LTM that is executed by the UE when or more LTM execution conditions are met.

Based on the definition above, the main difference of CHO compared to legacy HO is that there’s no L3 HO command compared for CHO, the mobility procedure is triggered by UE itself. When it comes to conditional LTM, the HO command is substituted by L2 LTM Cell Switch command. However, except for indicating the target cell of LTM, the L2 LTM cell switch command  also be used to indicate TA value for RACH-less LTM or RACH resource for CBRA.

It’s not clear whether early TA acquisition is supported for conditional LTM. There’s two methods adopted for early TA acquisition: PDCCH-order based  early TA acquisition and UE-based TA measurement.

If early TA acquisition is supported, LTM cell switch command cannot be omitted for PDCCH-order based TA acquisition. While for UE-based TA acquisition, the LTM cell switch command can be omitted. But the accuracy of UE-based TA acquisition may not be as good as PDCCH-order based TA measurement.

Observation 1: For PDCCH-ordered early TA acquisition for LTM, LTM cell switch command is always needed to indicate the TA value to the UE.

Besides, in both Rel-18 and Rel-19 LTM discussion, both RAR for serving cell and RAR from candidate cell were considered. Both two RAR-based options can avoid LTM cell switch command for TA indication, but PDCCH order cannot be avoided as current PDCCH-order based TA acquisition. And based on the discussion in Rel-18, RAR-based TA acquisition from candidate cell may result in UE’s serving cell reception interruption.

Observation 2: RAR-based TA acquisition can work without network cell switch command, but PDCCH order to trigger TA acquisition is still needed.

Observation 3: RAR-based TA acquisition from candidate cell may result in UE’s serving cell reception interruption.

In current non-conditional LTM, the validity of TA can be assured by the network, i.e., the time to send PDCCH to trigger TA acquisition may be close to the time of sending LTM cell switch command, and PDCCH order and LTM cell switch command aim to the same candidate cell. But when it comes to conditional LTM with PDCCH order based TA acquisition, no matter RAR-based or not, it cannot be assured that the candidate cell with TA acquired is the one satisfying the related execution condition. Furthermore, the TA may be acquired too early than execution condition is satisfied.

Observation 4: For PDCCH-order based TA acquisition with/without RAR, it cannot assure the acquired TA is for the proper candidate cell and whether it is still valid when the related cell’s execution condition is satisfied.

To assure a valid TA with a proper candidate for PDCCH-order based TA acquisition, a condition may be used to trigger network’s PDCCH order. It can be another condition except for conditional LTM execution, or it can be the only condition, when it is satisfied, the UE acquires TA and completes the follow-up steps to access to the candidate cell. However, this may may introduce extra interaction between the UE and the network.

Proposal 2: RAN2 is proposed to discuss whether early TA acquisition is supported for conditional LTM.

Proposal 3: If RAR-based TA acquisition with PDCCH order is adopted for conditional LTM, a condition can be used to trigger network’s PDCCH order.

If RAR-based TA acquisition is not triggered by network’s PDCCH order, UE may trigger RAR acquisition when the execution condition is satisfied and then completes the follow-up steps to access to the candidate cell. In other words, RACH-based conditional LTM is supported.

Proposal 4: RACH-based conditional LTM is supported.

In current CHO procedure, the UE releases stored CHO configurations after successful completion of RRC handover procedure [2]. While for LTM, the UE doesn’t release LTM configuration after LTM cell switch completion unless there’s network indication to release. This is to support subsequent LTM. Similarly, to support subsequent conditional LTM, the configuration cannot be released after conditional LTM cell switch completion.

Proposal 5:To support subsequent conditional LTM, the related configuration is not released after conditional LTM cell switch completion.

The CHO configuration contains the configuration of CHO candidate cell(s) generated by the candidate gNB(s) and execution condition(s) generated by the source gNB. Similarly, the conditional LTM configuration contains the configuration of conditional LTM candidate cells generated by the candidate gNB(s) and execution condition(s). For the conditional LTM candidate cells, in our view it can be the same with legacy LTM candidate cells or not. As to the execution conditions, both L1 based trigger condition and L3 based trigger condition can be considered, since that L1 measurement capability and LTM capability are decouple. For the UE with L1 measurement capability, L1 based trigger condition is configured, while for the UE not supporting L1 measurement, L3 based trigger condition is configured. As to the L1 based trigger condition, it can reuse the design for L1 based measurement events, while for L3 based trigger conditions, the trigger conditions for CHO can be reused.

Proposal 6:  The candidate cells of conditional LTM can be the same with legacy LTM candidate cells or not.

Proposal 7: Both L1 based trigger condition and L3 based trigger condition can be considered in conditional LTM.

- For the UE supporting L1 measurement, L1 based trigger condition is configured;

- For the UE not supporting L1 measurement, trigger conditions of CHO can be reused for conditional LTM trigger condition.

Proposal 8: L1 based measurement events design can be reused for L1 based trigger condition for conditional LTM.

For CHO, Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell. We think similar principle can be applied in conditional LTM.

Proposal 9: Only one single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

Besides, for CHO An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5, as defined in TS 38.331). However, the events for CHO are cell-level measurement results. While for the beam level events for conditional LTM, whether it needs more trigger conditions can be further discussed.

Proposal 10: For the beam level events for conditional LTM, whether it needs more trigger conditions can be further discussed.

What’s more, to support subsequent conditional LTM, it should be made clear which node to generate execution condition. Similar issue was discussed in Rel-18 S-CPAC, following is the related description in [3]:

…… The following principles apply to subsequent CPAC: - For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC). - For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC. - For both MN and SN initiated inter-SN subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s). ……

Observation 5: the initial execution condition of S-CPAC is generated by the initiating node, while the execution condition for the following S-CPAC is generated by the candidate SNs.

Based on the observation on S-CPAC, for subsequent conditional LTM, the initial execution condition of subsequent conditional LTM is generated by the serving cell, while the execution conditions for the following subsequent conditional LTM are generated by candidate cells.

Proposal 11: The initial execution condition of subsequent conditional LTM is generated by the serving gNB, while the execution conditions for the following subsequent conditional LTM are generated by candidate gNBs.

Based on [4], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 6: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command or LTM cell switch command.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 12: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 13: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- Conclusion

In this contribution, we discuss conditional LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: For PDCCH-ordered early TA acquisition for LTM, LTM cell switch command is always needed to indicate the TA value to the UE.

Observation 2: RAR-based TA acquisition can work without network cell switch command, but PDCCH order to trigger TA acquisition is still needed.

Observation 3: RAR-based TA acquisition from candidate cell may result in UE’s serving cell reception interruption.

Observation 4: For PDCCH-order based TA acquisition with/without RAR, it cannot assure the acquired TA is for the proper candidate cell and whether it is still valid when the related cell’s execution condition is satisfied.

Observation 5: the initial execution condition of S-CPAC is generated by the initiating node, while the execution condition for the following S-CPAC is generated by the candidate SNs.

Observation 6: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1: The definition of conditional LTM is as following:

A conditional LTM is defined as a LTM that is executed by the UE when or more LTM execution conditions are met.

Proposal 2: RAN2 is proposed to discuss whether early TA acquisition is supported for conditional LTM.

Proposal 3: If RAR-based TA acquisition with PDCCH order is adopted for conditional LTM, a condition can be used to trigger network’s PDCCH order.

Proposal 4: RACH-based conditional LTM is supported.

Proposal 5:To support subsequent conditional LTM, the related configuration is not released after conditional LTM cell switch completion.

Proposal 6:  The candidate cells of conditional LTM can be the same with legacy LTM candidate cells or not.

Proposal 7: Both L1 based trigger condition and L3 based trigger condition can be considered in conditional LTM.

- For the UE supporting L1 measurement, L1 based trigger condition is configured;

- For the UE not supporting L1 measurement, trigger conditions of CHO can be reused for conditional LTM trigger condition.

Proposal 8: L1 based measurement events design can be reused for L1 based trigger condition for conditional LTM.

Proposal 9: Only one single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

Proposal 10: For the beam level events for conditional LTM, whether it needs more trigger conditions can be further discussed.

Proposal 11: The initial execution condition of subsequent conditional LTM is generated by the serving gNB, while the execution conditions for the following subsequent conditional LTM are generated by candidate gNBs.

Proposal 12: If L3 HO command/LTM cell switch command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command or LTM cell switch command.

Proposal 13: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

- References

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4;

- TS 38.300; NR; NR and NG-RAN Overall Description; Stage 2 (Release 18);

- TS 37.340 Multi-connectivity; Stage 2; (Release 18);

- S 38.473 F1 application protocol (F1AP) (Release 18).