3GPP TSG-RAN2 Meeting #127bis			R2-22408095

Hefei, China, 14th-18th, Oct.

Agenda item:	8.7.4.2

Title:	Consideration on DSR Enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 discusses DSR enhancement, leaving several FFS, as outlined below [1].

For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

This contribution will discuss some related issue on DSR enhancement.

Discussion

In the last meeting, RAN2 discusses whether to modify the definition of delay-critical data to accommodate DSR with multiple reporting thresholds. Considering the impact on TS38.323 and the inherent nature of delay-critical data, we believe it is unnecessary to modify delay-critical data definition to avoid ambiguity. If modifications are deemed necessary, it would be sufficient to include some adjustments in the DSR-related sections of TS38. 321.

Proposal 1: Multiple reporting thresholds for DSR has no impact on delay-critical data definition, it may have some impacts on DSR MAC CE definition in TS38.321, which can be discussed in stage-3.

  - Non-delay-critical data in DSR

Based on our previous discussions and consensus, non-delay-critical data primarily consists of two kinds of data. The first one includes data that does not have a configured discardTimer, such as certain SDU from SRB. The other one includes data that is configured with either a discardTimer or discardTimerForLowImportance, with remaining time that is less than the remainingTimeThreshold. It is important to note that discardTimerForLowImportance is not currently capture in the definition of delay-critical data at this stage, which we believe may require further refinement.

Observation 1a: Non-delay-critical data consist of delay-critical PDCP SDU with sufficient remaining time and PDCP SDU without discard timer.

First, it is important to clarify that the delay-critical PDCP SDUs in TS38.323 include restrictions on remaining time. If the remaining time for a PDCP SDU is still sufficient, according to the current protocol, this SDU should be classified as non-delay-critical data. Since the remainingTimeThreshold is configured by the gNB, and there can be multiple reporting thresholds, the gNB can autonomously determine the range of remaining time for PDCP SDUs it wishes to monitor. Therefore, the necessity of reporting non-delay-critical data outside the reporting threshold in DSR is questionable.

For PDCP SDUs with a configured discardTimer, the gNB can always derive their size by comparing the total buffer size reported in BSR with the size of the delay-critical data reported in DSR. A straightforward approach is to set a sufficiently large DSR reporting threshold that encompasses all PDCP SDUs with configured discardTimer. Given the frame rates of modern codec, this value is unlikely to be excessively large. For PDCP SDUs without discardTimer, gNB can infer their sizes in conjunction with both BSR and DSR.

Observation 1b: For delay-critical data with sufficient remaining time, gNB should be aware of their size with a proper multiple reporting thresholds configuration. For delay-critical data without discardTimer, gNB can calculate their buffer size with DSR and BSR.

Therefore, it is evident that, regardless of the type of non-delay-critical data, with the introduction of multiple reporting thresholds in DSR, the gNB can indirectly or directly ascertain their buffer sizes. Thus, we believe that if multiple remaining time thresholds for DSR reporting are configured, there is no necessity to report information on non-delay-critical data in DSR.

Proposal 2: With introduction of multiple DSR reporting thresholds, there is no need to report non-delay-critical data in DSR.

Additionally, it must be noted that RAN2 needs to consider the overhead implications of DSR. Given that the length of DSR is already quite substantial, attention should be paid to the effective utilization of the PDU.

  - Reporting and triggering threshold for DSR

In the last meeting, it was agreed that the network can configure a single triggering threshold along with multiple reporting thresholds for DSR. Based on current observations, configuring reporting thresholds that are either longer or shorter than the triggering threshold is beneficial. Reporting thresholds above the triggering threshold can provide additional information about non-delay-critical data, while those below the triggering threshold can help the gNB understand how much data is going to exhaust their delay budget and should be sent immediately. Therefore, it is recommended to set the triggering threshold within the range of the reporting thresholds to acquire more information. However, it should ultimately be a gNB implementation issue.

Proposal 3: It's suggested to configure DSR triggering threshold within the range of multiple DSR reporting thresholds, but it up to gNB implementation.

However, regarding the triggering threshold, it is believed that it is unnecessary to set a too small value. For instance, it should be at least greater than the duration of two consecutive PUSCH transmission opportunities, especially in TDD scenarios. Nonetheless, this can be determined by gNB configuration and is unlikely to have significant specification impact.

  - PSI in DSR

Another issue is whether to include PSI in DSR. According to the current TS38.321 and TS38.323, DSR includes only delay-critical PDCP SDUs.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

The necessity of reporting PSI for low-importance data remains to be clarified and questionable, and the signaling overhead introduced by the additional multiple pair reporting in DSR should not be overlooked. Furthermore, if only a 1-bit report is added, it is very question questionable on how many information it can provide and its granularity with just a 1-bit indication, as there may be multiple PSI levels within the LCH.

However, increasing the bit field would further escalate the DSR’s overhead. Therefore, we recommend modifying the definition of delay-critical PDCP SDUs to include low-PSI but delay-critical data, as follows:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. If discardTimerForLowImportance is configured and PSI based discard is activated, a PDCP SDU belonging to a low importance PDU Set of which at least one PDCP SDU has the remaining time till discardTimerForLowImportance expiry is less than the remainingTimeThreshold.

This way, low-PSI but delay-critical data can also be reported in DSR, thereby preventing the issue of the gNB being unaware of low-importance data.

Proposal 4: Update the definition of delay-critical PDCP SDU in TS38.323 to capture low importance PDCP SDU, i.e., SDU from PDU set configured with discardTimerForLowImportance.

Conclusion

This contribution discusses the DSR enhancement for XR, as follows.

Definition:

Proposal 1: Multiple reporting thresholds for DSR has no impact on delay-critical data definition, it may have some impacts on DSR MAC CE definition in TS38.321, which can be discussed in stage-3.

Non-delay-critical data in DSR:

Observation 1a: Non-delay-critical data consist of delay-critical PDCP SDU with sufficient remaining time and PDCP SDU without discard timer.

Observation 1b: For delay-critical data with sufficient remaining time, gNB should be aware of their size with a proper multiple reporting thresholds configuration. For delay-critical data without discardTimer, gNB can calculate their buffer size with DSR and BSR.

Proposal 2: With introduction of multiple DSR reporting thresholds, there is no need to report non-delay-critical data in DSR.

Thresholds configuration constraints:

Proposal 3: It's suggested to configure DSR triggering threshold within the range of multiple DSR reporting thresholds, but it up to gNB implementation.

PSI in DSR:

Proposal 4: Update the definition of delay-critical PDCP SDU in TS38.323 to capture low importance PDCP SDU, i.e., SDU from PDU set configured with discardTimerForLowImportance.

References

[1]	RP-242390, Release 19 XR enhancements - Way forward on WID scope revision, Nokia (Moderator)