3GPP TSG-RAN2 Meeting #127bis			R2-2408096

Hefei, China, 14th-18th, Oct.

Agenda item:	8.7.6

Title:	Consideration on Rate Control Enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

In RAN#105, uplink congestion control signaling is captured as an objective for XR ph3 WID, as follows, [1].

- Specify uplink congestion signaling [RAN2]: - Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

This contribution will discuss potential enhancement on this new objective.

Discussion

Generally, congestion control consists of congestion detection, congestion feedback to the source, source rate adaptation, and recovery after congestion. As RAN agrees to specify uplink congestion signaling, it is recommended to study several potential directions: uplink congestion detection, congestion feedback to the source (UE), and recovery after congestion. Source rate adaptation in the uplink is primarily determined by the implementation at the UE application layer, and thus will not be discussed here.

  - Uplink Congestion detection

It is clear that uplink congestion detection can be implemented by either the UE or the gNB; however, it is believed that it is more appropriate for the gNB to detect uplink congestion. This approach reduces the implementation complexity for the UE, and if the UE were to perform congestion detection, it would need to notify the gNB in uplink of the congestion once it has occurred. This can be challenging since the channel capacity or conditions may already be insufficient for the UE to send a congestion indication.

Observation 1: Both UE and gNB can detect uplink congestion, but it could be complicated and inappropriate for UE to detect or report it with regarding to implementation complexity and potential deteriorated uplink channel.

The gNB can detect congestion by analysing the allocated resource in UL Grant already and will be sent to the UE and the BSR reported by the UE. Specifically, if the gNB observes a reduction in available radio resources allocated to the UE, or channel quality deteriorates based on UL reference signal measurement, or consistent increase of UE's buffer queue, gNB can infer that congestion has occurred. But uultimately, uplink congestion detection is a gNB implementation issue.

Proposal 1: gNB should detect uplink congestion based on its implementation.

UE lacks visibility into the allocation of radio resources, which can lead to insufficient or excessive adjustments. Therefore, it is advisable to prioritize the research of gNB-based solutions for this purpose.

  - Congestion feedback to UE in downlink

Upon detecting uplink congestion, gNB can indicate to UE that congestion has occurred and start rate control or rate adaptation. An explicit downlink congestion indication is more appropriate, as it can provide additional information compared to implicit one. Furthermore, since the WID emphasizes that uplink congestion control primarily occurs at the MAC layer, using a MAC CE is more suitable.

In VoNR, the Recommended Bit Rate and Recommended Bit Rate Query MAC CE has been introduced to facilitate rate adaptation for EVS encoding. However, the rate range is limited to 0 to 1.56 Mbit/s (8000 * 200). Additionally, uplink congestion control does not require UL/DL indications. Therefore, reusing the Recommended Bit Rate MAC CE is not advisable.

Proposal 2a: Introduce an explicit congestion indication MAC CE to indicate uplink congestion to UE.

Considering that the WID has already mandated congestion indications for each QoS flow/per DRB, it is essential to indicate which DRBs/LCHs or QoS flows are experiencing congestion. Additionally, since the UE lacks visibility into the gNB's overall uplink resource allocation, the gNB can indicate the corresponding (maximum) date rate for specific DRB/LCH/QoS flow. This would help prevent the UE application layer from making excessive adjustments due to blindly reducing the congestion window.

UE detects congestion through packet loss in general TCP congestion control. However, it is believed that efforts should be made to avoid packet loss even in congestion, especially for data of high importance.

Proposal 2b: The congestion indication MAC CE should capture at least data rate and DRB/LCH identification.

With the new MAC CE, UE can make supervised adjustments to the data rate of each DRB, resulting in faster and more effective rate adjustments.

  - Recovery after congestion

In traditional TCP congestion control, after congestion mitigation, the sender typically increases its rate gradually or slowly, known as Additive Increase, Multiplicative Decrease (AIMD), to avoid causing further congestion. However, this additive increase can lead to the TCP slow start problem, where the sender requires a significant amount of time to return to a "normal" transmission rate.

Observation 2: With fast recovery, UE could get a proper and potentially higher data rate after congestion than traditional TCP slow start.

With regarding to gNB and UE, gNB can provide an approximate rate range that avoids congestion while ensuring a rapid start for the sender to enhance user experience. Therefore, it is recommended to introduce an indication to inform the UE that congestion has been alleviated.

Proposal 3a: Introduce a congestion relief MAC CE to indicate uplink congestion is alleviated to UE.

Similar to the congestion indication MAC CE, the congestion relief MAC CE should at least include DRB/LCH identification and a maybe maximum data rate. The gNB can generate a rate that avoids causing further congestion based on the air interface conditions and the BSR reported by other UEs. The UE can then select a transmission data rate that does not exceed this maximum data rate, ensuring a rapid recovery while preventing the reoccurrence of congestion.

Proposal 3b: The congestion relief MAC CE should optionally capture at least maximum data rate and DRB/LCH identification for fast recovery.

Of course, if the gNB does not provide the relevant information, the UE can also gradually recover its rate according to the traditional TCP slow start strategy.

Conclusion

This contribution discusses the congestion control for XR, as follows.

UL congestion detection:

Observation 1: Both UE and gNB can detect uplink congestion, but it could be complicated and inappropriate for UE to detect or report it with regarding to implementation complexity and potential deteriorated uplink channel.

Proposal 1: gNB should detect uplink congestion based on its implementation.

Congestion indication to UE:

Proposal 2a: Introduce an explicit congestion indication MAC CE to indicate uplink congestion to UE.

Proposal 2b: The congestion indication MAC CE should capture at least data rate and DRB/LCH identification.

Fast recovery:

Observation 2: With fast recovery, UE could get a proper and potentially higher data rate after congestion than traditional TCP slow start.

Proposal 3a: Introduce a congestion relief MAC CE to indicate uplink congestion is alleviated to UE.

Proposal 3b: The congestion relief MAC CE should optionally capture at least maximum data rate and DRB/LCH identification for fast recovery.

References

[1]	RP-242390, Release 19 XR enhancements - Way forward on WID scope revision, Nokia (Moderator)