3GPP TSG-RAN WG2 Meeting #127bis                                      	         R2-2408097

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.8.2

Title:	RAN2 Impact on DL coverage enhancements

Source:	CMCC

Document for:	Discussion

- Introduction

In previous RAN1 meetings, the study of downlink coverage enhancement in NR NTN had been discussed, focusing on the evaluation methodology, simulation assumptions and link budget analysis for DL coverage performance.

And in last RAN1 meeting, the following agreements has acheived:

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

And RAN1 has sent an reply LS[2] to RAN2 regarding the DL coverage enhancements in NTN scenarios. In the LS, RAN1 provides answers to the questions on DL coverage enhancements related to SSB, common control signaling, UL beams and beam status. As the progress in RAN1 is not so solid, the reply LS only provided limited information to progress RAN2 progress.

In this contribution, we will further provide our views on on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

- Discussion

According to the WID objective, Rel-18 network energy saving techniques are expected to be reused as baseline for DL system level enhancement:

- Rel-18 network energy saving techniques should be considered as baseline in the system level study

15.4.2.3 Cell DTX/DRX To facilitate reducing gNB downlink transmission/uplink reception active time, UE can be configured with a periodic cell DTX/DRX pattern (i.e. active and non-active periods). The pattern configuration for cell DTX/DRX is common for the UEs configured with this feature in the cell. The cell DTX and cell DRX patterns can be configured and activated separately. A maximum of two cell DTX/DRX patterns can be configured per MAC entity for different serving cells. When cell DTX is configured and activated for the concerned cell, the UE may not monitor PDCCH in selected cases or does not monitor SPS occasions during cell DTX non-active duration. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or does not transmit a SR during cell DRX non-active duration. This feature is only applicable to UEs in RRC_CONNECTED state and it does not impact Random Access procedure, SSB transmission, paging, and system information broadcasting. Cell DTX/DRX can be activated/deactivated by RRC signalling or L1 group common signalling. Cell DTX/DRX is characterized by the following: - active duration: duration that the UE waits for to receive PDCCHs or SPS occasions, and transmit SR or CG. In this duration, the gNB transmission/reception of PDCCH, SPS, SR, CG, periodic and semi-persistent CSI report are not impacted for the purpose of network energy saving; - cycle: specifies the periodic repetition of the active-duration followed by a period of non-active duration. Active duration and cycle parameters are common between cell DTX and cell DRX, when both are configured;

As specified above, current cell DTX/DRX patterns in Rel-18 NES is common for the UEs configured with this feature in the cell. For Rel-18 NES, a serving cell can be configured with either DTX or DTX+DRX. For cell DTX/DRX, each Serving Cell may be configured by RRC with a periodic cell pattern with only two status, i.e., Active and Inactive Periods.

However, according to the agreed assumption in RAN1, it is mentioned that a satellite may have large satellite footprint and limited feeder link bandwidth, especially in Set1-2 in RAN1 agreement. For Set1-2, the number of active beams is just 16, which is constrained by satellite payload capability, it is not be able to provide NR service for all beam footprints, the total number of the beam footprints is 1058. As shown in the table, without system level coverage enhancement, the effective coverage providing communication service/common signaling can only be transmitted for 1.5% of beam footprints within a satellite footprint.

As indicated in RAN1 LS as follows:

- Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

This means there is no term of N1, N2, N3 beam introduced in the spec. And a UE in a satellite cell is expected to be transited among only the two status for beam footprints, where beam status is to switch among ON and deactive during the beam hopping procedure. Hnece,  the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

Proposal 1:the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

Table 1  Comparison of UE behavior between DTX/DRX and Beam hopping

| Status | R19 beam hopping | R18 NES | NOTE |
| --- | --- | --- | --- |
| Active status | Regular UE behaviors | Regular UE behaviors |  |
| Cell DTX non-active status | Nothing to be received/transmitted | Not monitor PDCCH in certain cases Not monitor SPS No impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission | As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission; the UE behavior is different in the non-actvie status. |
| Cell DRX non-active status | Nothing to be received/transmitted | Not transmit on CG resources No SR transmission No periodic/semi-persistent CSI reporting No periodic/semi-persistent SRS No impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission |  |

Proposal 2: As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing thecurrent periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

Furthermore, in the RAN1 LS, it is replied as follows:

- Currently RAN1 does not have any discussion on the enhancements to the common control channel. But if the extension of SSB periodicity is supported, the enhancements to the corresponding common control channel/signals may be needed.

And in last RAN1 meeting, it also agreed that:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

Therefore, as the SSBs/SIBs can be transmitted more sparsely than current SSB pattern for different beam footprints to support more served beams. This would extend the SSB burst periodicity for each satellite beam footprint. For example, as shown in the above table for Set1-2, since the number of active beams is just 16, and the total number of the beam footprints is 1058, the value of PSSB can be derived, i.e., PSSB>=340ms. Considering the PSSB should be set as the powers of 2, the candidate PSSB value can be 320ms for 94% coverage ratio, or 640ms for 100% coverage ratio.

Figure 1 Illustration of extension the default PSSB/actual Tx PSSBfor cell search/detection in NTN system

Similarly, on top of default PSSB for cell detection/search, the actual transmission PSSB broadcasted in SIB1, periodicity of other common control channels (e.g. CORESET0/SIB1, SIB19) demands the value extension as well.

Obviously, the impact on system information needs to be considered as well, e.g., the enhanced SSB/SMTC pattern configuration for the dwell time and revisit time interval for each beam illumination across the coverage and the corresponding SI window design.

Proposal 3: it is propsed to design the DTX/DRX configuration for common control channels (e.g.SSB, CORESET0/SIB1, SIB19), based on RAN1 LS.

- Summary

Proposal 1:the current periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES for reducing gNB downlink transmission/uplink reception active time seems can be utilized for the DL system level enhancement (i.e., beam hopping ) use case.

Proposal 2: As for NES, no impact to idle UE, e.g., RACH, paging, SIBs and SSB transmission, it is proposed to define different UE behavior is different in the non-actvie status when reusing thecurrent periodic cell DTX/DRX pattern (i.e. active and non-active periods) in Rel-18 NES.

Proposal 3: it is propsed to design the DTX/DRX configuration for common control channels (e.g.SSB, CORESET0/SIB1, SIB19), based on RAN1 LS.

- References

- Chairman's Notes RAN1#116

- R2-2407919 Reply LS on DL coverage enhancements