# 3GPP TSG-WG2 Meeting #127bis                                             R2-2408098

Hefei, China, 14th – 18th October, 2024

Source: 			CMCC

Title:  		    Discussion on LCM for positioning

Agenda item:	8.1.2.3

Document for: 	Discussion

Introduction

With the Post126 #32 email discussion and RAN2#127 meeting, RAN2 discussed the definitions and procedures for applicable functionality. The following agreements have been reached.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- Step 3: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- Step 4: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

The email discussion also included the positioning use case, however, the positioning part had not been discussed for the limit of online time. In the contribution, we will focus on the LCM for positioning use case.

Discussion

In the RAN2#125Bis meeting, it has concluded that both proactive and reactive reporting are applicable to positioning use case. For proactive reporting of UE sided applicable functionality, LPP message can be used. For reactive reporting of UE-sided applicable functionality, LMF can configure the AI/ML functionalities via LPP.

Considering the difference between proactive and reactive reporting, for example the trigger of applicable functionality reporting, and the timing of the inference configuration of functionalities provided, the applicable functionality for positioning use case can be clarified as, in proactive reporting, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling, LMF provides inference configuration of functionalities via LPP signaling after LMF knowing applicable functionalities; in reactive reporting, applicable functionality reporting is provided as a response to the LMF request, whether LMF provides inference configurations of functionalities via LPP signaling before LMF receiving applicable functionalities and the UE behaviors when receive the inference configuration in Step 3, wait for RAN1 feedback.

Figure 1: Procedure of LCM for positioning

Proposal 1: In proactive reporting, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling, LMF provides inference configurations of functionalities via LPP signaling after LMF knowing applicable functionalities.

Proposal 2: In reactive reporting, applicable functionality reporting is provided as a response to the LMF request, whether LMF can provide inference configurations of functionalities via LPP signaling before LMF receiving applicable functionalities and the UE behaviors when receive the inference configuration in Step 3, can waiting for RAN1 feedback.

In RAN1#118, it has been agreed that for training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

For option A, UE-side assistance information can be transmitted via the existing signaling UEPositioningAssistanceInformation.

For the LMF decision in option A and option B, existing PRS reporting configuration can be reused.

Proposal 3-1: For option A, UE-side assistance information can be transmitted via the existing signaling UEPositioningAssistanceInformation.

Proposal 3-2: For the LMF decision in option A and option B, existing PRS reporting configuration can be reused.

With RAN1#116bis and RAN1#117, we have the following work assumptions and agreement:

Work assumptions: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Agreement: For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least:

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

For the case that the ground truth label is generated by LMF, LMF generates the ground truth label based on some measurement transmitted by UE and TRP’s location coordinate. LMF provides the corresponding ground truth label and its related data (e.g. time stamp) to UE for model training. The existing signaling ProvideAssistanceData can be reused.

Proposal 4: ProvideAssistanceData can be reused to provides the ground truth label and its related data (e.g. time stamp) to UE by LMF for model training.

Conclusion

According to the above discussion, the following observations and proposals are given:

Proposal 1: In proactive reporting, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling, LMF provides inference configurations of functionalities via LPP signaling after LMF knowing applicable functionalities.

Proposal 2: In reactive reporting, applicable functionality reporting is provided as a response to the LMF request, whether LMF can provide inference configurations of functionalities via LPP signaling before LMF receiving applicable functionalities and the UE behaviors when receive the inference configuration in Step 3, can waiting for RAN1 feedback.

Proposal 3-1： For option A, UE-side assistance information can be transmitted via the existing signaling UEPositioningAssistanceInformation.

Proposal 3-2: For the LMF decision in option A and option B, existing PRS reporting configuration can be reused.

Proposal 4: ProvideAssistanceData can be reused to provides the ground truth label and its related data (e.g. time stamp) to UE by LMF for model training.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR