# 3GPP TSG-WG2 Meeting #127                                             R2-2408099

HeFei, China, 14th – 18th October, 2024

Source: 			CMCC

Title:  		    Discussion on functionality for A-IoT

Agenda item:	8.2.2

Document for: 	Discussion

Introduction

With the past RAN2#127 meeting, we mainly focus functionality discussion on segmentation, energy state report, visibility information and message size reporting. Some progresses have been made and further discussion are still needed. In addition, some other functionalities also captured in this contribution.

Discussion

Information visibility to network/reader

RAN 2 have concluded that the service type of A-IoT message, the approximate number of target devices and targeted for one or more devices are useful information for reader to obtained from the CN.

At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

For A-IoT radio between device and reader, it is the network to allocate RA resource and data transmission resource to A-IoT radio. Random access and data transmission procedure are both fully controlled by network. For different kinds of procedure, the resource allocation can be different. For example, if inventory procedure is triggered, the expected data are RN16 and device ID, the D2R data can be almost 16 or 96bits from high layer perspective; if read command is triggered, the acknowledgement can be 1000bits at most; if write command is included in the AIOT paging message, the D2R feedback can be one or limit number of bits. For effective resource scheduling, the reader at least needs to understand the type of the command from CN, e.g. read/write/disable to assistant resource allocation.

Proposal 1: RAN2 assumes that the A-IoT command type is visible to gNB, i.e., gNB is aware of read/write/disable.

RAN2 have concluded that the (proximity) number of target devices from CN is necessary for gNB to assistant RA type selection and resource allocation. If no device number are provided by the CN, the gNB or reader can estimate the device number to assistant the access resource configuration to avoid waste of resource or collision, as we discussed in RA paper [3]. We suggest that RAN2 discuss whether and how to obtain the (approximate) number of devices in gNB or reader, if CN does not provide device number information to gNB.

Proposal 2: If no device number can be obtained from CN, RAN2 discuss whether and how to obtain the (approximate) number of devices in gNB or reader.

Segmentation

Segmentation in A-IoT has been discussed for a few rounds in RAN2. Although RAN2 considers segmentation and reassembly would add complexity to system, companies still have concerns on the size of the transmission black can not meet the requirements of D2R and R2D transmission. In last meeting, an LS was sent to RAN1 and SA2 about the maximum and minimum TB size in physical layer and their using conditions. The feedback of LS is expected for the October or November meeting. In our understanding, before the information from RAN1 and SA2, RAN2 can further discuss on how to support segmentation if supported.

Firstly, how to handle the segment(s) at transmitting side and how to buffer the received segment(s) at receiving side. For D2R and R2D, the transmitting side and receiving side are apposite to each other, A-IoT enable UE A-IoT RAN node both have higher complexity and capability than the A-IoT devices. Therefore, we suggest considering separately whether and how segmentation can be supported for D2R and R2D.

Considering the legacy segmentation, the segments of data should be buffered at the transmitting side pending to be transmitted, and the received segments should be buffered at receiving side waiting for the other segments. The buffer may increase the complexity and cost. As proposed in some companies’ contributions, transmitting entity can transmit partial of data from the memory with an appropriate size and mark the position that has been completed for transmission, so that the subsequent transmission can be continued based on this position. Such way can avoid introducing buffer at transmitting side. However, applying a similar approach to receiving side is somewhat complex. The receiving side can write the correctly received data packets into memory without storing in buffer temporarily, but if other segments of the data are lost, the receiving side needs to clear the data segments from the memory. This is because retransmission function has been explicitly excluded by RAN2. So, for R2D, the segmentation can not be considered. For D2R, buffer-free segmentation can be studied, if segmentation is necessary for A-IoT. With the buffer-free segmentation, the segment are recorded with the position-like indicator at the device side, the legacy number of segments, the sequence number and segment length are not considered. For stop data monitoring at receiving side, an end maker like can be considered to be transmitted along with the last segment.

Proposal 3: RAN2 focuses on D2R segmentation discussion only.

Proposal 4: For D2R, buffer-free segmentation can be studied, if segmentation is necessary for A-IoT.

Proposal 5: RAN2 do not consider the number of segments, the sequence number and segment length.

Message size reporting

As discussed in last meeting, the need of message size reporting is still not clear from RAN2 point view.

RAN2 will study the need and use of a simple message “size” reporting to the reader

The discussion on message size reporting is initiated when we concluded that the legacy BSR/SR is not considered in A-IoT. The motivation to further discuss the need of simple message size reporting is to inform the reader about the remaining data to be transmitted. The message size report is beneficial for the network resource allocation. With the progress in last meeting, the gNB can know the message type, e.g., inventory or command. The rough data size can be estimated by reader. So, without considering segmentation, the message size reporting is not needed. If the segmentation is considered, message size reporting can be used to inform reader whether there is data to be transmitted.

Proposal 6: Without considering segmentation, the message size reporting is not needed for A-IoT.

Proposal 7: If the segmentation is considered, message size reporting can be considered to inform reader whether there is data to be transmitted.

Energy state report

The energy state report issue has been discussed in last RAN2 meeting.

RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

For R19 ambient IoT, efforts are made to minimize the size, complexity, and power consumption of A-IoT devices, aiming to facilitate the deployment of billions of IoT devices for a wide array of applications and to enhance value across the entire value chain. However, the energy state of A-IoT devices cannot be measured with precision by the devices themselves. They can provide limited information to the reader. The rationale for considering energy state reporting is to assist the reader in resource allocation and scheduling of D2R transmissions. However, we understand that the device can decide whether to response the paging by implementation, meaning that available devices can opt to skip the access round to continue energy harvesting and choose to later access round the paging round.

Observation:

- AIOT device can hardly precisely measure energy status, especially for device type 1.

- Whether the UE reports the energy state correctly is untestable. Operator and network side cannot rely on an untestable reporting.

- Even without energy state reporting, by device implementation, the device can choose to not to reply in the current access round if the device is in low power.

In addition, different operators have different network deployment strategies. We don’t see any need of introducing energy state reporting. Even if any operator is in favor of it, we think the reporting has to under network control. If network don’t support energy state reporting feature, the device shouldn’t report.

Proposal 8: Energy state reporting for A-IoT is not supported. How to handle the low energy case is left to device implementation.

AS layer ID

RAN2 has agreed to use 16 bits random ID in Msg1 of CBRA to identify device itself. However, RAN2 still keep open on whether introduce AS temporary device ID. As we discussed in RA Paper [3], the temporary device ID can be allocated by reader and unique among all devices within the reader coverage. The temporary device ID can lower the random access collision.

Proposal 9: RAN2 considers that reader is allowed to assign unique AS layer ID to device.

QoS handling

In TR 38.848, the latency targets of Ambient IoT system which represents the QoS is given as:

- 5.6 Latency The one-way end-to-end maximum latency targets, as defined in TR 22.840, are: - Longer latency target: 10 seconds - Shorter latency target: 1 second A use case is assigned to a latency target according to TR 22.840. RAN WGs can refine a definition of latency suitable for their work within the above. NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only. NOTE: The one-way end-to-end maximum latency is assumed to also include query/triggering time.

Further in the revised WID [1], the latency targets are considered in RAN1 led evaluation assumptions. Up to now, RAN1 has not discussed whether the definition of latency needs to be refined. As for the one-way end-to-end maximum latency targets defined above, RAN1 may need to discuss the DL/UL data rates, the design of preamble, etc. But we have not found a clear work in RAN2 scope. Besides there has not been any other QoS requirements agreed by other WGs. To this end, RAN2 can assume all Ambient IoT services have same QoS requirements before the definition of latency is refined by RAN1 or other QoS requirements are agreed by other WGs.

Proposal 10: Before the requirement of latency is refined by RAN1 or other QoS requirements are agreed by other WGs, RAN2 can assume all Ambient IoT services have same QoS requirements.

Proximity determination

As described in the revised SID of A-IoT, the feasibility and functionality for proximity determination should be studied.

Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

RAN1 has start the discussion on this issue and has some preliminary progress:

Agreement [RAN1#117]

Study the following schemes for proximity determination:

- Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

  - FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- Option 2: Device is determined to be near the reader based on measurements at the reader side

  - FFS: Details on measurement methods

- FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

Conclusion [RAN1#118]

Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

Considering the two solutions on the table in RAN1, the reader judges the devices are nearby once the reader receive the response from the device or the quality of the response is higher than some threshold. As per the discussion and conclusion in RAN P#105, RAN1 will not further discussion on proximity determination in SI. We understand RAN2 should discuss and identify the impacts of proximity determination procedure, i.e., reusing the inventory or command procedure.

Proposal 11: RAN2 should discuss and identify the possible impacts of proximity determination procedure.

Protocol stack

In topology 1, the gNB reader performs inventory or read/write command to A-IoT devices. The gNB also allocates transmission resources to A-IoT radio, and forwards the inventory or read/write command feedback to core network. In topology 2, it is the UE as intermediate node to do inventory or read/write command towards A-IoT devices. The basic end to end architecture has not been decided in RAN2 or RAN3. So, for the uu interface design between intermediate UE and network is far from clear. However, apart from the interworking between the UE reader and network, the behaivers of the UE reader are totolly same as the gNB reader at A-IoT reader. The functionality for A-IoT radio betweent A-IoT devices and gNB/UE reader is common for both topology 1 and topology 2. The A-IoT radio protocol stack betweent A-IoT devices and gNB/UE reader should be kept aligned.

With the above discussion, we draft the protocol stack for A-IoT as follows:

Figure.1-1 Protocol stack for A-IoT topology 1

Figure.1-2 Protocol stack for A-IoT topology 2

Proposal 12: RAN2 is suggested to capture the protocol stack in Figure 1-1, 1-2 for A-IoT topology 1 and 2.

Conclusion

According to the above discussion, the following observation and proposals are given:

Observation:

- AIOT device can hardly precisely measure energy status, especially for device type 1.

- Whether the UE reports the energy state correctly is untestable. Operator and network side cannot rely on an untestable reporting.

- Even without energy state reporting, by device implementation, the device can choose to not to reply in the current access round if the device is in low power.

Proposal 1: RAN2 assumes that the A-IoT command type is visible to gNB, i.e., gNB is aware of read/write/disable.

Proposal 2: If no device number can be obtained from CN, RAN2 discuss whether and how to obtain the (approximate) number of devices in gNB or reader.

Proposal 3: RAN2 focuses on D2R segmentation discussion only.

Proposal 4: For D2R, buffer-free segmentation can be studied, if segmentation is necessary for A-IoT.

Proposal 5: RAN2 do not consider the number of segments, the sequence number and segment length.

Proposal 6: Without considering segmentation, the message size reporting is not needed for A-IoT.

Proposal 7: If the segmentation is considered, message size reporting can be considered to inform reader whether there is data to be transmitted.

Proposal 8: Energy state reporting for A-IoT is not supported. How to handle the low energy case is left to device implementation.

Proposal 9: RAN2 considers that reader is allowed to assign unique AS layer ID to device.

Proposal 10: Before the requirement of latency is refined by RAN1 or other QoS requirements are agreed by other WGs, RAN2 can assume all Ambient IoT services have same QoS requirements.

Proposal 11: RAN2 should discuss and identify the possible impacts of proximity determination procedure.

Proposal 12: RAN2 is suggested to capture the protocol stack in Figure 1-1, 1-2 for A-IoT topology 1 and 2.

Figure.1-1 Protocol stack for A-IoT topology 1

Figure.1-2 Protocol stack for A-IoT topology 2

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[2] RAN#105 meeting report.

[3] R2-2409093, Further consideration on random access for ambient IoT, CMCC