3GPP TSG-RAN WG2 Meeting #127bis	R2-2408100

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.2

Source: 	vivo

Title:         	Remaining issues of on-demand SSB SCell operation

Document for: 	Discussion and Decision

- Introduction

For the below objective of Rel-19 network energy saving enhancement WI, RAN1 and RAN2 have made some progress which can be found in the Appendix.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

With these progresses, this paper will discuss the remaining issues of on-demand SSB (OD-SSB SCell) operation.

- Discussion

  - Indication of OD-SSB

RAN2#127 meeting made the following agreement for the indication of OD-SSB:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

For the above FFS, if RRC signaling can be used to indicate the OD-SSB transmission after configuration (e.g., when reconfiguring OD-SSB configuration or other parameters), there may have potential impact in RAN4, for example, RAN4 may need to discuss the new requirement for that case. Considering anyway MAC CE based OD-SSB indication can be used for the reconfiguration case, we suggest to restrict that RRC based OD-SSB indication can be only used to indicate the initial activation state of OD-SSB configuration.

- RAN2 to confirm RRC based OD-SSB transmission indication is only used upon setup of OD-SSB configuration.

For MAC CE based OD-SSB transmission indication, the below FFS should be discussed.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

If separate MAC CE are used to indicate OD-SSB transmission and SCell activation, then RAN4 needs to discuss the requirement for the case that two MAC CEs are not received simultaneously. To avoid such situation, we think the optimization for scenario 2A is needed, i.e., the new MAC CE can also indicate the activation state of SCell for scenario 2A. Thus, we have the following proposal:

- The new MAC CE should support indicating only OD-SSB transmission for Scenario #2, and indicating both SCell activation and OD-SSB transmission for Scenario #2A.

For the design of new MAC CE, we understand multiple OD-SSB configurations may be provided per SCell, and the network may indicate OD-SSB transmission of multiple SCells in the MAC CE at the same time, so we have provided the below options.

- Option 1: The OD-SSB fields and SCell A/D state fields are mandatorily included in the new MAC CE.

- Option 2: The OD-SSB fields are mandatorily included in the new MAC CE, and 1 byte or 4 bytes SCell A/D state fields are optionally included in the new MAC CE depending on how many SCells are configured.

- Option 3: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of each are independently indicated by separate indicators in the new MAC CE.

- Option 4: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of both are indicated by a common indicator in the new MAC CE.

Examples of Option 1~4 are shown in the below figures, where Ci field is used to indicate the activation/deactivation state of SCell with SCellIndex i. ith OD-SSB field with N bits indicates the activation/deactivation state of OD-SSB configuration of the SCell with SCellIndex i. In figure 3, Bi field indicates whether the ith OD-SSB field is present in the new MAC CE. In figure 4, Ai field is used to indicate the presence of the OD-SSB field and A/D field of the SCell with SCellIndex i. In both figure 2 and 3, the length of the variable size MAC CE included in the MAC subheader can indicate whether 1/4  byte(s) SCell A/D state fields are included in the new MAC CE.

Figure 1: Mandatory OD-SSB fields and mandatory SCell A/D state fields

Figure 2:  Mandatory OD-SSB fields and optional SCell A/D state fields

Figure 3: Optional OD-SSB fields and optional SCell A/D state fields with separate indicators

Figure 4: Optional OD-SSB fields and optional SCell A/D state fields with a common indicator

Each option has its own advantages and disadvantages. For example, in case the number of SCells configured is less than 8 and 2 bits are used for each OD-SSB field, Option 1 can have the minimum siganlling overhead compared to the other Options. In case the number of SCells configured is more than 7, or more bits are needed to indicate OD-SSB transmission per SCell, Option 2, 3, 4 can be considered. Among Option 2/3/4, Option 4 seems easier to capture in the specification. Given above, we propose the following:

- For the format of the new MAC CE, RAN2 to consider the below options:

- Option 1: OD-SSB fields and SCell A/D state fields are mandatorily included in the new MAC CE;

- Option 2: OD-SSB fields are mandatorily included in the new MAC CE, and 1 byte or 4 bytes SCell A/D state fields are optionally included in the new MAC CE depending on how many SCells are configured.

- Option 3: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of each are independently indicated by separate indicators in the new MAC CE.

- Option 4: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of both are indicated by a common indicator in the new MAC CE.

  - L3 measurement and reporting

In RAN1#116bis meeting, L1 and/or L3 measurement based on on-demand SSB is supported for a cell supporting on-demand SSB SCell operation. RAN2 can focus on L3 measurement based on OD-SSB.

Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement

L3 measurement:

In RAN2#127, the below agreements were made:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In the current specification, for each frequency, the UE will not consider SSB transmission outside the SMTC occasion for RRM measurements based on SSB. For the case of OD-SSB, we don’t see the need to consider SMTC framework for OD-SSB based L3 measurement as OD-SSB transmission is cell specific and could be triggered at any time. Therefore, the UE can perform L3 measurement of OD-SSB based on the configured periodicity and time location of OD-SSB and the MAC CE indicating the configured pattern is activated.

- Upon receiving the indication of OD-SSB, L3 measurement of OD-SSB is based on the configured periodicity and time location of OD-SSB of the SCell.

Another issue is how to report the measurement results of OD-SSB. The below listed options can be considered for triggering the report.

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the MAC CE used for indicating OD-SSB.

In option 1, if the OD-SSB configuration is provided under SCell configuration, how to provide the report configuration for this SCell needs to be discussed, for example, the network can explicitly configure a whole set of parameters of report event under SCell configuration, or configure an associated measurement report ID. If the OD-SSB configuration is provided under serving MO configuration (FFS a new measurement object for on demand SSB can be configured for SCell or measurement object including on-demand SSB information and/or including always on periodic SSB information can be configured), then the existing MR configuration can be reused. Moreover, a separate event can be considered for OD-SSB measurement, if the legacy event is used for the always on SSB measurement. And in option 2/3, network can determine when to collect the available results from the UE based on the arrival of traffic.

- RAN2 to discuss the below options to trigger the OD-SSB measurement report:

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the signalling used for indicating OD-SSB.

- Conclusion

In this paper, the following proposals are given:

- RAN2 to confirm RRC based OD-SSB transmission indication is only used upon setup of OD-SSB configuration.

- The new MAC CE should support indicating only OD-SSB transmission for Scenario #2, and indicating both SCell activation and OD-SSB transmission for Scenario #2A.

- For the format of the new MAC CE, RAN2 to consider the below options:

- Option 1: OD-SSB fields and SCell A/D state fields are mandatorily included in the new MAC CE;

- Option 2: OD-SSB fields are mandatorily included in the new MAC CE, and 1 byte or 4 bytes SCell A/D state fields are optionally included in the new MAC CE depending on how many SCells are configured.

- Option 3: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of each are independently indicated by separate indicators in the new MAC CE.

- Option 4: Both OD-SSB fields and SCell A/D state fields are optionally included in the new MAC CE, and the presence of both are indicated by a common indicator in the new MAC CE

- Upon receiving the indication of OD-SSB, L3 measurement of OD-SSB is based on the configured periodicity and time location of OD-SSB of the SCell.

- RAN2 to discuss the below options to trigger the OD-SSB measurement report:

- Option 1: report is triggered by measurement report configuration (i.e., periodic, existing/new/separate event);

- Option 2: report is triggered by a new MAC CE sent from the network;

- Option 3: report is triggered after a certain time following the reception of the signalling used for indicating OD-SSB.

- References

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

- Appendix (RAN2/1 Agreements)

RAN2#127

Scenarios on OD-SSB: - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

RAN1#118

Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4 Agreement LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438. Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

RAN1#117

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

RAN1#116bis

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement The following agreement from RAN1#116 is modified (in red) - For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement) Agreement For a cell supporting on-demand SSB SCell operation, further study the following options. - Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication. - Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided. - FFS: Details of the signaling - Other options are not precluded. - FFS: Details on On-demand SSB transmission indication

RAN1#116

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations. Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE Agreement - For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options. - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B. - FFS: The combination of above options - FFS: How to define time instance A/B and the value of N per option - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)