3GPP TSG-RAN WG2 Meeting #127bis		R2-2408102

Hefei, China, 14th Oct. – 18th Oct. 2024

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation of common signal transmissions

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, adaptation of paging occasion for network energy saving was further discussed and the agreements below were achieved [1]:

- Option-a) is about the bundling of PF for R19 NES UEs. - R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Moreover, RAN1 had made some progress on the PRACH adaptation in time domain. In this contribution, we will further provide our consideration on the adaptation of paging occasions in time domain and solutions for RACH adaptation with the latest RAN1 progress.

Discussion

  - Adaptation of paging occasions in the time domain

- Paging adaptation options

In RAN2#125bis meeting, RAN2 identified the following two options [2].

2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

In RAN2#127 meeting, RAN2 continued to discuss the understanding of these two options and agreed that option-a and option-b can achieve the same time position for the POs. However, we still have some concerns on option-b for that it may have signalling overhead issue as it needs more bits in the SI to indicate firstPDCCH-MonitoringOccasionOfPO for POs. In the table below, we analyze the additional signalling overhead of option-b compared to option-a when SCS is 30kHz and T is 320ms for instance.

| SCS=30khz T=320ms | Option a：（Assuming PF is continuous） firstPDCCH-MonitoringOccasionOfPO=1 radio frame =10 sub frame =10 slot =140 symbol; - The number of bits used for a PO offset: 8 bit Option b： Assuming firstPDCCH-MonitoringOccasionOfPO=32 radio frame =320 sub frame =320 slot =4480 symbol - The number of bits used for a PO offset: 13 bit The number of extra bits for option b compared to option a =(13*n-8*x) bit Where x is the number of PO in one PF in option a, n is the number of PO in one PF in option b |  |  |  |
| --- | --- | --- | --- | --- |
| SCS=30khz T=320ms |  | Option a |  |  |
|  |  | PO=1 | PO=2 | PO=4 |
| Option b (N=onethirtysecondT) | PO =1 | 5 bit |  |  |
|  | PO =2 | 18 bit | 10 bit |  |
|  | PO =4 | 44 bit | 36 bit | 20 bit |
|  | PO =8 | 96 bit | 88 bit | 72 bit |
|  | PO =16 | 200 bit | 192 bit | 176 bit |

Table 1: Comparison of options.

For option-a, since the gap between bundled PFs is still FFS, we now evaluate the signalling overhead based on the assumption that the bundled PFs are continuous. We can see from the table that the number of extra bits for option-b compared to option-a is (13*n-8*x) bits where n is the number of POs in one PF in option-b and x is the number of POs in one PF in option-a. For option-a, firstPDCCH-MonitoringOccasionOfPO will be signalled x times with a maximum value of 4. For option-b, n can be greater than 4. The larger n being introduced, the higher signalling overhead of scheduling PO.

Observation 1: Option-b has potential additional signalling overhead compared to option-a, due to the value range of the firstPDCCH-MonitoringOccasionOfPO and the maximum configured number of POs in one PF.

Option-b is preferred by the majority of companies in email discussions [3] which is assumed to have less spec impact. However, as we analyzed above, the RRC signalling overhead may be a concern of option-b and optimization may be needed, which will result in additional spec impact. Consider, for instance, PO offset repetition patterns which can save some signalling overhead, but is not flexible. Therefore, the spec impact of option-b may be comparable with option-a. Therefore, we propose to first analyse the signalling overhead of option-b, evaluate the spec impact of it, and then compare it with option-a for further down-selection.

Proposal 1: RAN2 to first study the signalling overhead issue and spec impact of option-b, and then compare it with option-a for further down-selection.

- Legacy UE handling

In RAN2#125bis meeting, RAN2 discussed the following options on compatibility of legacy RRC_IDLE and RRC_INACTIVE UE [2]:

- Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs);

Based on the following note for adaptation of common signal/channel transmissions in NES WID [4], the legacy UEs can camp on the cell configured with paging adaptation feature. Therefore, the target scenario in Rel-19 NES is a single cell with the legacy UEs and the Rel-19 UEs supporting paging adaptation. Option 1 is not in the scope of R19 NES WID.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown.

However, some companies argue that legacy UEs may affect the NES gain of the network. In this regard, we think that the network can configure the legacy PFs sparsely and make the Rel-19 PFs as closed to the legacy PF as possible. It will not have a significant impact on the NES gain of the network.

Proposal 2: The NW does not bar legacy UEs for the purpose of paging adaptation.

- UE capability reporting

For paging adaptation, a per UE capability should be introduced. The UE can report the capability of supporting paging adaptation to the gNB after registration with AMF, and the gNB can report the capability to the AMF. The AMF forwards this capability information to all gNBs in the latest TA of the UE for CN paging. Similarly, the anchor gNB can also forward this capability information to other gNBs for RAN paging. Then, the serving gNB can identify whether the paging is for a UE supporting paging adaptation or not based on the exchanged UE capability, which assists the network to determine the paging location. For example, when the NW wants to page a UE which supports paging adaptation, if the NW is not awared of whether paging adaptation is supported by this UE, the NW will not know whether to apply legacy configuration or the NES configuration. Thus, we think the capability of supporting paging adaptation should be introduced.

Proposal 3: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

## 2.2 Adaptation of PRACH resources in time domain

- Legacy UE handling

To achieve the NES purpose in PRACH adaptation, the network could configure legacy PRACH resources sparsely to reduce the monitoring time between legacy ROs when the system load is low. When the number of camping UEs increase a lot, additional NES-related PRACH resources with shorter period can be activated for RACH offloading. Meanwhile, the legacy UEs can still perform random access procedure with the legacy PRACH resources. The following note in the WID [4] for adaptation of common signal/channel transmissions also points out that there shall be no negative impact to legacy UE. Since we do not see significant benefits to bar the legacy UEs, we propose RAN2 to not introduce cell barring mechanism for PRACH adaptation.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown.

Proposal 4: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

- Legacy PRACH resources for NES-capable UEs

Based on the below agreements made in RAN1#116bis [5], RAN1 had agreed to support additional PRACH resources for NES-capable UEs, and it was noted that NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs. In RAN1#118 meeting [6], DCI based adaptation is supported.

RAN1#116bis Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 RAN1#118 Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

One issue that needs clarification is whether the NES-capable UEs can still use legacy PRACH resources after the additional PRACH resources are activated. From our understanding, if there are fewer legacy UEs and NES-capable UEs only select additional PRACH resources when there are legacy PRACH resources, the congestion of RACH in additional PRACH resources would be increased heavily and legacy PRACH resources would be wasted. In addition, the additional PRACH resources are generally on-demand resources. If the NES-capable UEs must wait for the additional PRACH resources, it will increase the delay of random access procedure for NES-capable UEs. Thus, we propose RAN2 to clarify and confirm the understanding on RAN1 agreement ‘Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs’.

Proposal 5: RAN2 to clarify and confirm the understanding of the RAN1 agreement ‘Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs’ means the NES-capable UEs can still use legacy PRACH resources after the additional NES-related PRACH resources are activated.

- RACH triggering events

Currently, a number of triggering events for random access are specified in TS38.300 as following:

The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

RAN2 needs to discuss whether all the above triggering events can be supported for additional PRACH resources. In RAN1#116bis, RAN1 had agreed to support additional PRACH resources for IDLE/INACTIVE and CONNECTED UEs, so the corresponding triggering events of IDLE/INACTIVE and CONNECTED should be considered. For the events like “handover”, “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM cell switch”, even if the PRACH resources are configured for neighbor cells, the function of the additional PRACH resources is no different from that of the legacy PRACH resources, since the random access in additional PRACH resources is only to reduce the PRACH congestion when the legacy PRACH resources may be configured sparsely for NES. Therefore, the PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

RAN1#116bis Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Proposal 6: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

Conclusion

Based on the discussion, we have the following proposals:

Adaptation of paging occasions in the time domain

Observation 1: Option-b has potential additional signalling overhead compared to option-a, due to the value range of the firstPDCCH-MonitoringOccasionOfPO and the maximum configured number of POs in one PF.

Proposal 1: RAN2 to first study the signalling overhead issue and spec impact of option-b, and then compare it with option-a for further down-selection.

Proposal 2: The NW does not bar legacy UEs for the purpose of paging adaptation.

Proposal 3: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

Adaptation of PRACH resources in time domain

Proposal 4: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

Proposal 5: RAN2 to clarify and confirm the understanding of the RAN1 agreement ‘Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs’ means the NES-capable UEs can still use legacy PRACH resources after the additional NES-related PRACH resources are activated.

Proposal 6: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

# 4. Reference

- RAN2#127 meeting minutes

- RAN2#125bis meeting minutes

- [POST127][109][NES] Paging Enh

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

- RAN1#116bis meeting minutes

- RAN1#118 meeting minutes