3GPP TSG-RAN WG2 Meeting #127b	R2-2408103

Hefei, China, 14th Oct. – 18th Oct., 2024

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on random access procedure in SBFD

Document for:	Discussion and Decision

Introduction

According to R2#127bis, the following agreements were made[1]:

Agreement Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

In this contribution, we will further discuss the random access procedure in SBFD.

Discussion

RACH triggering events

During last meeting, RAN2 agreed on the following working assumption:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

The major comments from the companies include:

- No strong concern is observed to exclude any of the events;

- Since the main benefit of SBFD is to reduce latency, we may focus on the latency sensitive events and connected state.

RAN1 agreed during last meeting to extend the previous RAN1 agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode. This implies not only the benefit from SBFD operation applies to both RRC_CONNECTED and RRC_IDLE/INACTIVE, but also we strive for a common design for these modes. Since we do not observe any strong concern to exclude any of the events, we propose RAN2 to confirm the working assumption.

Proposal 1: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

SBFD UE early identification

Whether early identification is needed for SBFD had been discussed in both RAN1 and RAN2, and it is now RAN2 to focus on this issue. So far, companies had mentioned about Msg1 based early identification and Msg3 based early identification.

For Msg-1 based method, the major consideration of early identification is that the NW can schedule SBFD resources for Msg3 transmission, and two options have been discussed:

- Opt.a: The NW can identify SBFD-aware UE if the UE transmits on ‘additional ROs’;

- Opt.b: SBFD is considered as a new feature for PRACH preamble partitioning;

During last meeting, RAN1 has defined ‘additional ROs’ as follows [2]:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

For opt.a, only the SBFD-aware UEs can use the ‘additional ROs’, and hence the NW can identify the SBFD-aware UE regardless of whether opt.b is supported or not. For opt.b, it needs to define SBFD as a feature considered for feature combination, where we observe there are not many available feature spot left for new features.

FeatureCombination-r17 ::= SEQUENCE {

redCap-r17                 ENUMERATED {true}                                OPTIONAL,  -- Need R

smallData-r17              ENUMERATED {true}                                OPTIONAL,  -- Need R

nsag-r17                   NSAG-List-r17                                    OPTIONAL,  -- Need R

msg3-Repetitions-r17       ENUMERATED {true}                                OPTIONAL,  -- Need R

msg1-Repetitions-r18       ENUMERATED {true}                                OPTIONAL,  -- Need R

eRedCap-r18                ENUMERATED {true}                                OPTIONAL,  -- Need R

spare2                     ENUMERATED {true}                                OPTIONAL,  -- Need R

spare1                     ENUMERATED {true}                                OPTIONAL   -- Need R

}

The only case which requires opt.b to enable early identification is that the SBFD-aware UE transmits on ‘legacy ROs’. In this case, the NW cannot distinguish SBFD-aware UEs from legacy UEs, and thus need to define SBFD as a feature in the FeatureCombination. However, there is ongoing discussion about how the UE selects between additional ROs and legacy ROs. If certain criteria for RO selection is introduced, e.g. the UE determines whether to select additional RO based on DL pathloss reference, it may affect the conclusion on whether the NW should schedule SBFD resources for Msg3 transmission to the SBFD-aware UE. For instance, if the SBFD-aware UE determines to select legacy RO based on the fact that the measured DL RSRP is below a certain threshold, then this decision implies that the NW should not schedule SBFD resources to the SBFD-aware UE for Msg3 transmission. As a result, early identification with opt.b is useless for Msg3 resource scheduling optimization.

In sum, we think it is up to the NW implementation how to handle if it identifies the SBFD-aware UE, and propose RAN2 to first discuss about the two options listed above and determine whether to down-select from them.

Observation 1: The motivation of Msg1-based early identification for SBFD-aware UE is that the NW may schedule SBFD resources for Msg3 transmission.

Observation 2: The discussion on RO selection may affect the conclusion whether we need opt.b, e.g. if the UE avoids to select additional ROs based on certain criteria, then the NW should also avoids to schedule SBFD resource for Msg3 transmission to the SBFD-aware UE, even the UE applies SBFD dedicated preamble.

Proposal 2: For Msg1-based early identification for SBFD-aware UE, RAN2 to discuss and determine whether to down-select from the following two options:

- Opt.a: The NW can identify the SBFD-aware UE if the UE sends Msg1 on ‘additional ROs’ as defined by RAN1;

- Opt.b: SBFD is considered as a new feature for PRACH preamble partitioning, i.e. the NW can configure SBFD as a feature as ‘true’ for a set of random access resources, which is similar to RedCap, Msg3 repetition, etc,.

Proposal 3: It is up to the NW implementation how to handle if it identifies the SBFD-aware UE with Msg1-based early identification, e.g. the NW decides whether to schedule SBFD resources to the SBFD-aware UE for Msg3 transmission.

As for Msg3-based early identification, we do not observe real need for it, as at least one Msg1-based solution will likely be determined.

Proposal 4: RAN2 to confirm not to support Msg3-based early identification for SBFD-aware UE.

SBFD RACH configuration

RAN1 agreed on two options for SBFD RACH configuration [3], i.e. use one single RACH configuration or two separate RACH configurations to indicate ROs within UL sub-band in SBFD symbols. RAN1 also agreed on the following principles:

- Enabling both options at the same time for a UE is not supported;

- UE is not required to support both options;

For RRC_CONNECTED UE, the NW can configure the corresponding SBFD RACH configuration according to the UE Capabilities report. However, we need to determine whether a new UE capability for the supported SBFD RACH configuration should be defined. Although RAN1 agreed that the UE is not required to support both options, from our perspective, RAN2 can discuss whether a UE supporting the SBFD operation naturally supports both options. If this consensus cannot be reached, then a new UE capability should be defined for the supported SBFD RACH configuration.

Proposal 5: RAN2 to discuss whether a UE supporting SBFD operation naturally supports both SBFD RACH configuration options, or needs a new UE capability to indicate the supported SBFD RACH configuration option.

If a new UE capability for supported SBFD RACH configuration option, then one issue is whether UE can report the capability of supporting both options for SBFD RACH. One alternative is that the UE can only supports one of the two options, and another alternative is that the UE can supports either only one option or both of the options. From our point of view, it is straight forward that the UE can indicate the support of either one of the options or both options, since RAN1’s understanding is just the UE is not required to support both options. Hence, we propose RAN2 to confirm this understanding.

Proposal 6: If a new UE capability for supported SBFD RACH configuration option is defined, the UE can indicates the support of either one of the SBFD RACH options or both options.

For RRC_CONNECTED UE, note that ‘Enabling both options at the same time for a UE is not supported’ according to RAN1 agreement is to reduce the complexity of UE implementation. For RRC_IDLE/INACTIVE UE, the same principle still applies. In IDLE/INACTIVE state, NW does not have information on UE capability report about which option is supported, and how many UEs supporting SBFD operation are within its coverage. Therefore, we propose RAN2 to discuss how the NW configures SBFD RACH. One alternative is that the UE assumes the NW will configure only one of the options. Another alternative is that the NW can configure both options, but which option is used and how to define the UE behavior for this case need to be further discussed.

Proposal 7: For RRC_IDLE/INACTIVE UE, RAN2 to discuss and clarify how the NW configures SBFD RACH configuration and the corresponding UE behavior.

- Alt 1a: The NW configures only one of the options in SIB. If the UE does not support the corresponding RACH configuration option, the UE does not perform RACH procedure using SBFD resources.

- Alt 1b: The NW configures only one of the options in SIB. A UE supporting SBFD operation should also supports both SBFD RACH configuration options, and applies the configured SBFD RACH configuration in SIB.

- Alt 2: Both of the SBFD RACH configuration options can be configured in SIB. The UE applies the RACH configuration option which it supports.

Random access procedure

Carrier selection

A serving cell can be configured with supplementary uplink (SUL) for the purpose of coverage enhancement. In the Rel-18 WI Coverage Enhancement (CovEnh), it was discussed and determined that:

- Msg1 repetition can be configured on SUL;

- It is up to the NW whether to configure both NUL with Msg1/3 repetition and SUL for the purpose of CovEnh;

In SBFD, we propose that similar principle can also be agreeable.

Proposal 8: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

The next issue is whether SBFD operation has an impact on carrier selection for random access procedure. In legacy procedure, the UE directly determines the UL carrier if it is explicitly indicated by the NW. Otherwise, the UE compares the DL RSRP with the NW configured SUL threshold to determine it [4].

In SBFD, the NW can enable Msg1 repetition in SBFD resources on TDD-NUL according to RAN1 agreement [2]. The UE compares the DL RSRP with Msg1 repetition threshold to determine the number of repetition. Msg1 repetition on SBFD resources enhance the NUL coverage to some extent, which weakens the motivation for the UE to select SUL. Therefore, we propose RAN2 to discuss whether the NW configuring SBFD on NUL has an impact on NUL/SUL selection. For example, if the UE selects to perform SBFD operation on NUL, the UE can rely on Msg1 repetition with SBFD on NUL instead of choosing SUL.

Proposal 9: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

SSB/CSI-RS selection is a step in random access procedure. If not indicated by the NW, the UE compares the RSRP of the SSB/CSI-RS with the threshold and select an SSB/CSI-RS with SS-RSRP/CSI-RSRP above rsrp-ThresholdSSB/rsrp-ThresholdCSI-RS. The UE then determines RO based on the selected SSB/CSI-RS [4].

In SBFD, legacy ROs and additional ROs can be both mapped to the same SSB/CSI-RS. Considering the NW can evaluate the CLI in SBFD resources based on the UE reports and adjust the SBFD configuration accordingly, the NW may configure different threshold for the UE to select additional ROs or legacy ROs to minimize the CLI in SBFD resources. However, we may just rely on the legacy SSB/CSI-RS selection method. We propose RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

Proposal 10: RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

RO selection

For RO selection, the legacy UE selects RO randomly amongst the consecutive PRACH occasions once the SSB/CSI-RS for RACH is determined [4]:

1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

During last meeting, RAN1 agrees as follows to trigger further RAN2 discussion on RO selection [2]:

Agreement For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations. - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs. Agreement For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs.

From our point of view, the two RACH configuration options can configure the same additional ROs, and they differ mainly in that Option 2 can provides SBFD dedicated parameters, like power ramping related parameters. This difference should not affect how UE performs RO selection for SBFD. We propose RAN2 to strive for a common solution for RO selection in SBFD with either RACH configuration Option 1 with Alt 1-1 or Option 2.

Proposal 11: RAN2 to strive for a common solution for RO selection with either RACH configuration Option 1 with Alt 1-1 or Option 2.

In SBFD, the key issue for RO selection is whether we need to differentiate additional ROs and legacy ROs from an RO selection perspective. As such, there are several options for RO selection in SBFD:

- Option 1: An SBFD aware UE prioritizes selection of valid additional ROs or legacy ROs;

There are motivations for the UE either to prioritize valid additional ROs or legacy ROs. For instance, to minimize the CLI in SBFD resources, the UE should prioritize selection of legacy ROs. To increase RACH capacity, the UE should prioritize selection of valid additional ROs.

- Option 2: An SBFD aware UE determine whether to select valid additional ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

This is also for minimizing the CLI in SBFD resources. For instance, if the RSRP is too low compared to a dedicated SBFD threshold, then the UE should not select valid additional ROs to avoid significant interference to SBFD operation on neighbor cells.

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are valid additional ROs or legacy ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

Actually, the UE may not need to differentiate valid additional ROs or legacy ROs. The UE still randomly selects RO amongst the consecutive ROs. For example, the UE can benefit if the selected RO comes earlier regardless whether it is SBFD RO or non-SBFD RO. Random selection helps the NW to achieve the balance between the said motivations for the UE either to prioritize additional ROs or legacy ROs in option 1.

In sum, we propose RAN2 to discuss how an SBFD-aware UE to perform RO selection in SBFD.

Proposal 12: RAN2 to discuss how an SBFD-aware UE perform RO selection amongst valid additional ROs or legacy ROs:

- Option 1: An SBFD aware UE prioritizes selection of valid additional ROs or legacy ROs;

- Option 2: An SBFD aware UE determine whether to select valid additional ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are valid additional ROs or legacy ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

PRACH transmission re-attempt

During last meeting, RAN1 agrees as follows to trigger further RAN2 discussion on PRACH transmission re-attempt [2]:

Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Similar to the reason why we propose above for RO selection, we propose RAN2 to first confirm to strive for a common solution for PRACH transmission re-attempt with either RACH configuration Option 1 with Alt 1-1 or Option 2.

Proposal 13: RAN2 to strive for a common solution for PRACH transmission re-attempt with either RACH configuration Option 1 with Alt 1-1 or Option 2.

From the listed options above, we think Option 2 achieves the balance between the utilization of valid additional ROs and legacy ROs. Option 1 is easy for implementation, but the UE cannot timely adjust the RACH attempt strategy to enjoy the benefit of SBFD operation or avoid the negative impact from SBFD CLI. Option 3 is the most flexible solution, but it is kind of unfriendly for UE implementation, and the benefit it can achieve above Option 2 is doubtful. Therefore, we suggest RAN2 to down-select the options to Option 2.

Proposal 14: RAN2 to down-select the PRACH transmission re-attempt solution to Option 2 according to RAN1 agreement: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure.

Conclusion

Based on the discussion, we have the following observation and proposals:

RACH triggering events

Proposal 1: RAN2 to confirm the following working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

SBFD UE early identification

Observation 1: The motivation of Msg1-based early identification for SBFD-aware UE is that the NW may schedule SBFD resources for Msg3 transmission.

Observation 2: The discussion on RO selection may affect the conclusion whether we need opt.b, e.g. if the UE avoids to select additional ROs based on certain criteria, then the NW should also avoids to schedule SBFD resource for Msg3 transmission to the SBFD-aware UE, even the UE applies SBFD dedicated preamble.

Proposal 2: For Msg1-based early identification for SBFD-aware UE, RAN2 to discuss and determine whether to down-select from the following two options:

- Opt.a: The NW can identify the SBFD-aware UE if the UE sends Msg1 on ‘additional ROs’ as defined by RAN1;

- Opt.b: SBFD is considered as a new feature for PRACH preamble partitioning, i.e. the NW can configure SBFD as a feature as ‘true’ for a set of random access resources, which is similar to RedCap, Msg3 repetition, etc,.

Proposal 3: It is up to the NW implementation how to handle if it identifies the SBFD-aware UE with Msg1-based early identification, e.g. the NW decides whether to schedule SBFD resources to the SBFD-aware UE for Msg3 transmission.

Proposal 4: RAN2 to confirm not to support Msg3-based early identification for SBFD-aware UE.

SBFD RACH configuration

Proposal 5: RAN2 to discuss whether a UE supporting SBFD operation naturally supports both SBFD RACH configuration options, or needs a new UE capability to indicate the supported SBFD RACH configuration option.

Proposal 6: If a new UE capability for supported SBFD RACH configuration option is defined, the UE can indicates the support of either one of the SBFD RACH options or both options.

Proposal 7: For RRC_IDLE/INACTIVE UE, RAN2 to discuss and clarify how the NW configures SBFD RACH configuration and the corresponding UE behavior.

- Alt 1a: The NW configures only one of the options in SIB. If the UE does not support the corresponding RACH configuration option, the UE does not perform RACH procedure using SBFD resources.

- Alt 1b: The NW configures only one of the options in SIB. A UE supporting SBFD operation should also supports both SBFD RACH configuration options, and applies the configured SBFD RACH configuration in SIB.

- Alt 2: Both of the SBFD RACH configuration options can be configured in SIB. The UE applies the RACH configuration option which it supports.

Random access procedure

Carrier selection

Proposal 8: The NW can configure both NUL with SBFD operation and SUL for the purpose of coverage enhancement.

Proposal 9: For NUL/SUL carrier selection when NUL is configured with SBFD, RAN2 to discuss whether an SBFD aware UE selects based on SBFD configuration or relies on legacy NUL/SUL carrier selection method.

SSB/CSI-RS selection

Proposal 10: RAN2 to discuss whether SBFD operation has an impact on SSB/CSI-RS selection for random access procedure.

RO selection

Proposal 11: RAN2 to strive for a common solution for RO selection with either RACH configuration Option 1 with Alt 1-1 or Option 2.

Proposal 12: RAN2 to discuss how an SBFD-aware UE perform RO selection amongst valid additional ROs or legacy ROs:

- Option 1: An SBFD aware UE prioritizes selection of valid additional ROs or legacy ROs;

- Option 2: An SBFD aware UE determine whether to select valid additional ROs based on a threshold, e.g. RSRP threshold, CLI threshold;

- Option 3: The UE randomly selects an RO amongst the consecutive PRACH occasions regardless whether the ROs are valid additional ROs or legacy ROs. Whether the definition of ‘consecutive PRACH occasions’ needs to be revised is up to RAN1.

PRACH transmission re-attempt

Proposal 13: RAN2 to strive for a common solution for PRACH transmission re-attempt with either RACH configuration Option 1 with Alt 1-1 or Option 2.

Proposal 14: RAN2 to down-select the PRACH transmission re-attempt solution to Option 2 according to RAN1 agreement: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure.

References

- R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- R1#118 meeting minutes

- R1#117 meeting minutes

- TS 38.321, V18.2.0

- R1#116 meeting minutes