3GPP TSG-RAN WG2 Meeting #127b	R2-2408104

Hefei, China, 14th Oct. – 18th Oct., 2024

Agenda Item:	8.11.3

Source:	vivo

Title:	Discussion on other aspects in SBFD

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss about Configuration for L1 based UE-to-UE CLI measurement and reporting.

Discussion

L1 based UE-to-UE CLI measurement and reporting

For UE-to-UE CLI handling schemes, L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework will be specified according to the updated WID [1].

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In our view, L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework can be treated as one kind of CSI reporting. For example, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity as agreed in the following agreement [2]. Regarding the configuration of the number of reported CLI resources or reporting criteria, it can generally reuse that of existing L1-RSRP.

Agreement For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity. - FFS: configuration of number of reported CLI resources in the reportConfig - FFS: reporting criteria

For the measurement resource configuration, SRS-RSRP resource or CLI-RSSI resource can be configured as IMR for a CSI report configuration, i.e. configure as a new pattern of SRS resource and CLI-RSSI resource under CSI-IM resource (set). In the last meeting, it was also discussed to configure as a new measurement resource (set) in CSI-reportConfig. In our view, there is no need to configurate new measurement resource (set), configure a new pattern of SRS resource and CLI-RSSI resource under CSI-IM resource (set) is sufficient.

For aperiodic and semi-persistent L1 CLI on PUSCH if supported, the DCI format for A-CSI or SP-CSI triggering can be used to trigger the A-CLI or SP-CLI report, CLI measurement resource for aperiodic CLI and semi-persistent L1 CLI on PUSCH is configured in aperiodicTriggerStateList or semiPersistentOnPUSCH-TriggerStateList.

Regarding configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource, for current L3 CLI measurement, same QCL as the latest received PDSCH or monitored CORESET is assumed. For L1 CLI measurement, it was discussed whether to support configuration of ‘typeD’ QCL assumptions for the CLI measurement resource. On one hand, configuration of the Rx beam for each L1 CLI measurement resource is beneficial for gNB to get the knowledge of interference information on each beam direction to enable gNB beam management for CLI mitigation. On the other hand, the number of supported L1 CLI measurement and reporting may be limited by UE capability, it is hard for the gNB to determine which Rx beam(s) is configured before knowing the best Rx beams. It may not be meaningful to measure the interference on a Rx beam with a small probability to be used in the future. Determination of ‘typeD’ QCL assumptions for the CLI measurement resource same as current L3 CLI can be considered as baseline, e.g. when the configuration is not provided, and configuration of configuration of ‘typeD’ QCL assumptions for the CLI measurement resource can be further considered.

Proposal: For L1 based UE-to-UE CLI measurement and reporting, it is based on existing CSI framework.

- New report quantities for CLI is configured within reportQuantity for a CSI report configuration.

- Measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, is configured as interference measurement resource for a CSI report configuration.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal: For L1 based UE-to-UE CLI measurement and reporting, it is based on existing CSI framework.

- New report quantities for CLI is configured within reportQuantity for a CSI report configuration.

- Measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource is configured as interference measurement resource for a CSI report configuration.

References

- RP-241614	Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

- R1#118 meeting minutes