3GPP TSG-RAN WG2 Meeting #127b	R2-2408108

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon, Turkcell, China Southern Power Grid

Title:	Further consideration on Store and Forward

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were achieved for the Store & Forward (S&F) satellite operation [1]:

Agreements: 1. RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) 2. RAN2 will consider both single satellite pass and multiple satellite pass scenarios 3. RAN2 will consider both MO and MT data within scope 4. UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) 5. RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. 6. RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we provide some further considerations on the Store and Forward satellite operation.

# 2	Discussion

2.1	Remaining time of current operating mode

In the last meeting, we discussed the scenario of serving satellite switching between the normal mode and S&F mode. There was no consensus on whether NW needs to inform UE of the remaining time of the satellite’s current operating mode. Before determining whether to support this indication, we should first discuss why this is useful for UE to know.

Switching from normal mode to S&F mode

If the serving cell switches from normal mode to S&F mode, it means the feeder link becomes unavailable at some time point.

In the legacy mechanism, the t-service in SIB19 indicates when the current serving cell will stop the service in the area currently covering. If the cell supports both normal mode and S&F mode, it may become unavailable for the legacy UEs when switching to S&F mode. But at this time, it has not reached the t-service time yet. So it is not clear how NW and legacy UEs should behave in this case. Even if the NW releases the legacy UEs to RRC_IDLE, legacy UEs may not perform neighbouring cell measurement before t-Service. One possible NW implementation is to set the t-Service to the switching time from the normal mode to S&F mode. In this case, t-service serves as the indication of the remaining time of current operating mode. For the S&F UEs, a new t-service time would be needed to indicate the actual stop serving time for the cell.

Switching from S&F mode to normal mode

If the serving cell switches from S&F mode to normal mode, it means the feeder link becomes available at some time point. In this case, if UE knows the switch beforehand when the cell is operating in S&F mode, UE may choose not to initiate ATTACH/TAU immediately, which requires two stage under S&F mode. The overall latency of NAS procedure is shorter if UE can wait until the cell switches to normal mode. And the UEs not supporting S&F can also know when it can camp on this cell, i.e., when the cell switches to normal mode. In this case, the switching remaining time indication is useful.

Proposal 1: RAN2 should first discuss how UE uses the indication of the remaining time of current operating mode before introducing it.

2.2	UE’s RRC state when service link is not available

Another issue that needs to be discussed is UE’s RRC state when the service link is not available. There are the following candidates:

1) UE is released to RRC_IDLE

2) UE is released to RRC_INACTIVE

3) UE stays in RRC_CONNECTED with the AS operation suspended.

For 2) and 3), since the multiple satellite scenario is supported, we need to consider whether it is always possible to transfer the UE context between the satellites. This may depend on RAN3 discussion.

Proposal 2: UE can be released to RRC_IDLE when service link is not available. FFS on whether UE can be released to RRC_INACTIVE or kept in RRC_CONNECTED with the AS operation suspended.

2.3	Paging in S&F satellite operation

In the legacy paging procedure, the arrival of MT data triggers NW to page UE. In the scenario of S&F, the CN sends MT data to satellite when feeder link is available, the satellite receives MT data and stores it. When service link is available, the satellite pages UE or the UE initiates RRC connection and then the NW sends MT data to UE. But if the satellite does not receive any MT data when feeder link is available, the satellite does not need to page UE when service link is available. And the UE doesn’t needs to monitor paging or initiate RRC connection in this case.

Therefore, if UE is informed when the NW does not have any MT data buffered for an area, the UE does not need to monitor the paging or initiate RRC connection, which saves UE power.

Proposal 3: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

2.4	Access control

For both S&F architectures, the UE’s DL NAS data can be stored at the MME on board the satellite serving the UE at next pass. In this case, if the UE is unable to access the intended satellite, the stored data will be lost. Considering the current access control mechanism may lead to rejection of the UE’s RRC connection request due to the load status, it is beneficial for the eNB to prioritize the access of the UE, if it is subsequently re-attempting the access to the NW, to avoid potential data loss.

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW.

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Remaining time of current operating mode

Proposal 1: RAN2 should first discuss how UE uses the indication of the remaining time of current operating mode before introducing it.

UE’s RRC state when service link is not available

Proposal 2: UE can be released to RRC_IDLE when service link is not available. FFS on whether UE can be released to RRC_INACTIVE or kept in RRC_CONNECTED with the AS operation suspended.

Paging in S&F satellite operation

Proposal 3: If the NW doesn’t have any MT data buffered for an area, it informs the UE so that the UE doesn’t need to monitor paging or initiate RRC connection.

Access control

Proposal 4: S&F NW should prioritize the access of the UE, if it is subsequently re-attempting the access to the NW.

4	References

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 126.