3GPP TSG-RAN WG2 Meeting #127bis	R2-2408111

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	RRM measurement relaxation and offloading in RRC_IDLE or RRC_INACTIVE

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting [1], the following agreements are reached on RRM measurement relaxation and offloading.

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

The following agreements are reached in RAN4 for RRM measurement relaxation and offloading.

RAN4#111 Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4#112 Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

In this contribution, we further discuss criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE modes.

- Discussion

  - RRM measurement when UE is not operating with LR

It was agreed that “RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs”, which means only the UE capable of LR/LP-WUS can perform RRM measurement offloading/relaxation discussed in Rel-19 LP-WUS/WUR WI. Furthermore, in our understanding, if a LR/LP-WUS capable UE is not operating with LR (i.e. UE is not monitoring LP-WUS, and UE only operates with MR), the NR legacy behaviors for RRM measurement (including legacy RRM measurement relaxation) should be performed. In other words, the RRM measurement offloading/relaxation in RRC_IDLE/INACTIVE discussed in Rel-19 LP-WUS/WUR WI only applies to UEs operating with LR.

Proposal 1: If a UE is not operating with LR, i.e. UE only operates with MR, the NR legacy behaviors for RRM measurement are performed.

  - RRM measurement when UE is operating with LR

    - Criteria for RRM measurement fully offloading/relaxation

RAN4 has agreed on case #1 and case #3 when serving cell measurement by LP-WUR is being performed:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

If a UE is operating with LR, the number of transitions from LR to MR should be minimized to obtain the power saving gain. As stated in TR 38.869 [2], using the MR for RRM measurements has a significant impact on UE energy consumption and reduces the power saving gain. Thus, the RRM measurements by MR should be relaxed (case#3 relaxation) or stopped (case#1 fully offloading) once UE is operating with LR, which is aligned with RAN4 agreements.

Proposal 2: RAN2 confirms that if UE is operating with LR, RRM measurements are either fully offloaded or relaxed RRM measurements.

For case#1, since neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency) is off, it means UE is not likely to perform cell re-selection. For example, UE is near the center of the cell. On the other hand, for case#3, UE may be near the cell edge so that neighboring cell measurement is on. So obviously the RSRP threshold for case#1 should be higher than for case#3.

Observation 1: The case#1 is the RRM measurement behaviors for UEs in center of the cell and the case#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage. The criterion for case#1 should be stricter than criterion for case#3.

Based on Observation 1, how LP-WUR or MR performs RRM measurement is illustrated in the figure below.

Fig. 1. Examples of UE behaviors for RRM measurement fully offloading and relaxation

Two options (or methods) were discussed in the last meeting for RRM measurement fully offloading or relaxation:

- Reuse entry/exit condition of LP-WUS monitoring as entry/exit condition for RRM measurement fully offloading/relaxation.

If the entry condition of LP-WUS monitoring is fulfilled, the UE starts LP-WUS monitoring, and performs either fully offloading (case#1) or relaxation (case#3). If the exit condition of LP-WUS monitoring is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR. Based on Observation 1, the criterion for case#1 should be stricter than criterion for case#3, thus the entry/exit condition of LP-WUS monitoring can be used as entry/exit condition for RRM measurement relaxation, and the entry/exit condition for RRM measurement fully offloading should be further defined.

- Define a separate entry/exit condition for RRM measurement fully offloading/relaxation, e.g. by introducing a new Rel-19 condition or reusing the Rel-16 RRM measurement relaxation condition.

It requires that threshold for condition of RRM measurement fully offloading/relaxation be equal to or higher than threshold for condition of LP-WUS monitoring. Introducing a new Rel-19 condition for RRM measurement fully offloading/relaxation may lead to great specification impact. On the other hand, if the Rel-16 RRM measurement relaxation condition is reused, it will unnecessarily add network complexity of configuring LP-WUS to fulfill the restriction and impacts LP-WUS coverage, e.g. the entry/exit condition of LP-WUS monitoring should be tighter than the Rel-16 RRM measurement relaxation condition. And in the most reasonable case, NW will just configure as option 1) illustrates.

Based on the analyses above, we prefer 1) as it has less specification impact and less complexity on NW configuration.

Proposal 3: The entry/exit condition of LP-WUS monitoring can be regarded as the criteria for entry/exit condition of MR RRM measurement relaxation.

As illustrated in Fig.1, additionally, the criteria should be defined to determine whether RRM measurement fully offloading (case#1) or relaxation (case#3) should be performed. Specifically, when UE is inside LP-WUS coverage, if the serving cell quality is above a threshold, UE performs RRM measurement fully offloading (case#1); otherwise, UE performs RRM measurement relaxation (case#3).

Observation 2: If UE is operating with LR, criteria should be defined to determine whether RRM measurement fully offloading (case#1) or relaxation (case#3) should be performed.

Proposal 4: If UE is operating with LR:

- If the serving cell quality is above a threshold, RRM measurement fully offloading (case#1) can be performed by UEs;

- Otherwise, if the serving cell quality is below a threshold, RRM measurement relaxation (case#3) can be performed by UEs.

    - Evaluating fully offloading criteria based on LR or MR measurement?

In last meeting, RAN2 discussed the entry/exit condition for RRM measurement fully offloading (case#1) without taking case#3 (which was agreed in parallel by RAN4 during the last meeting) into account. Currently, with both case#1 and case#3 supported, RAN2 needs to further think about the entry/exit condition considering: switch between case#1 and case#3.

If UE is operating with LR, the UE needs to perform serving cell measurement by LR, meanwhile the serving cell measurement by MR is relaxed or stopped. This means even if serving cell measurement by MR is not stopped, it will be relaxed many times more than normal serving measurement. In this case, it is very likely that serving cell measurement by MR is not timely enough to be used for the evaluation of the entry/exit condition of fully offloading case. Thus, the serving cell quality measured by LR is more appropriate to be used for determining between RRM measurement fully offloading (case#1) or relaxation (case#3).

Observation 3: If UE is operating with LR, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry/exit condition of fully offloading case.

Proposal 5: If UE is operating with LR, serving cell measurement by LR is used for the evaluation of the entry/exit condition of fully offloading case.

    - Other aspects on the criteria

Besides, measurement relaxation for higher priority frequency needs to be further considered. Higher priority frequency may be still needed even when UE is in the centre of the cell, and basically the measurement for higher priority frequency, when UE is in the centre of the cell, is already relaxed according to existing RRM requirement, and may be further relaxed which is up to RAN4 discussion. If UE only performs higher priority frequency measurement, without performing neighbor cell measurement of equal or lower priority, UE can only rely on the measurement results of higher priority frequency to perform cell reselection. Moreover, RAN4 is discussing case#4 especially for higher priority frequency, thus, RAN2 waits for RAN4 further conclusion.

Proposal 6: RAN2 waits for RAN4 further conclusion on measurement relaxation for higher priority frequency.

Based on Proposal 3 above, the entry condition of LP-WUS monitoring can be regarded as serving/neighbor cell measurement relaxation by MR. Thus, additional neighbor cell measurement relaxation criterion is not needed. And basically the “not at cell edge” has been considered for entry/exit condition of LP-WUS monitoring or criteria for performing neighbor cell measurement. In addition to the criteria for not being at cell edge, it can be further discussed whether the criteria for low mobility is needed or how to combine it if needed.

Proposal 7: The introduction of low mobility criteria for measurement relaxation of MR can be further studied.

Furthermore, UE obtains the serving and neighbor cell measurement results to evaluate whether the RRM measurement related criteria or cell reselection criteria is fulfilled or not. For fully offloading case, since there is no serving cell measurement by MR, UE relies on serving cell measurement by LR to determine whether the criteria for starting neighbor cell measurement relaxation of equal or lower priority is fulfilled or not. However, for cell reselection criteria, in some cases there is comparison between serving cell measurement results and neighbor cell measurement results, thus, both serving and neighbor cell measurement results by MR can be used. RAN2 can further discuss the criteria based on more inputs from RAN1/4.

Proposal 8: Whether the criteria evaluation for cell reselection is based on measurement by LR (LP-SS or PSS/SSS) or by MR can be further studied based on RAN1/4 progress.

- Conclusion

In this paper, we discuss RRM measurement relaxation and offloading for IDLE/INACTIVE modes, and have the following proposals.

RRM measurement when UE is not operating with LR

Proposal 1: If a UE is not operating with LR, i.e. UE only operates with MR, the NR legacy behaviors for RRM measurement are performed.

Criteria for RRM measurement fully offloading/relaxation

Proposal 2: RAN2 confirms that if UE is operating with LR, RRM measurements are either fully offloaded or relaxed RRM measurements.

Proposal 3: The entry/exit condition of LP-WUS monitoring can be regarded as the criteria for entry/exit condition of MR RRM measurement relaxation.

Proposal 4: If UE is operating with LR:

- If the serving cell quality is above a threshold, RRM measurement fully offloading (case#1) can be performed by UEs;

- Otherwise, if the serving cell quality is below a threshold, RRM measurement relaxation (case#3) can be performed by UEs.

Evaluating fully offloading criteria based on LR or MR measurement?

Proposal 5: If UE is operating with LR, serving cell measurement by LR is used for the evaluation of the entry/exit condition of fully offloading case.

Other aspects on the criteria

Proposal 6: RAN2 waits for RAN4 further conclusion on measurement relaxation for higher priority frequency.

Proposal 7: The introduction of low mobility criteria for measurement relaxation of MR can be further studied.

Proposal 8: Whether the criteria evaluation for cell reselection is based on measurement by LR (LP-SS or PSS/SSS) or by MR can be further studied based on RAN1/4 progress.

- References

- R2-2407572, Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

- TR 38.869, Study on low-power wake up signal and receiver for NR.