3GPP TSG-RAN WG2 Meeting #127bis		R2-2408114

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_IDLE and INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#125bis meeting, the following agreements for LP-WUS/WUR in RRC_IDLE/INACTIVE were made [1].

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - LP-SS configuration - LP-WUS configuration - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In RAN2#126 meeting, we further discussed the LP-WUS/WUR in RRC_IDLE/INACTIVE and reached the following agreements [2]:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

In RAN2#127, we had the following agreements [3]:

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, we will further discuss the issues for RRC_IDLE/RRC_INACTIVE procedures with LP-WUR, including LP-WUS configuration, the entry and exit condition, wake up procedure, and subgrouping. Besides, we analyze the RAN2 impacts based on the RAN1 LS on LP-WUS operation on separate bands.

- Discussion

  - LP-WUS configuration

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - LP-SS configuration - LP-WUS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

In RAN2#125bis and RAN2#126 meeting, it was agreed that the LP-WUS related configuration is provided via system information.

Regarding the specific system information for LP-WUS configuration, both the existing SIB(s) and new SIB are feasible. However, considering paging-related configuration and PEI information are included in SIB1, it is straightforward to follow the same design for LP-WUS configuration. Besides, introducing a new SIB will result in more specification work, e.g., the scheduling information of the new SIB and the on-demand request for the new SIB. Thus, SIB1 is preferred.

Proposal 1: LP-WUS configuration is provided in SIB1.

  - Entry/exit condition of using LP-WUS

Regarding the specific entry/exit condition for LP-WUS monitoring and UE behaviours, RAN1 has achieved the following Working Assumption [4].

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. Note: This may be revisited based on the RAN2/RAN4 discussion.

And RAN2 had the following agreement in RAN2#126 meeting [2] and RAN2#127 meeting [3]:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

Regarding the UE behaviours, it can be seen that even though the entry/exit condition is fulfilled, it is up to UE implementation whether to start/stop LP-WUS monitoring, i.e. UE “may” start LP-WUS monitoring, or “may” stop LP-WUS monitoring. For example, when the entry condition for LP-WUS monitoring is fulfilled, if there is expected UL data arrival later, MR could select not to go to ultra-deep sleep. Similarly, it is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled. However, in this case, MR needs to perform legacy PO monitoring.

Observation 1:  It is up to UE implementation to determine whether to start the LP-WUS monitoring when the entry condition is fulfilled.

Observation 2:  It is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled.

According to RAN2 agreement and RAN1 working assumption above, if the entry/exit condition is configured, MR-based threshold is always configured and LR-based threshold is optionally configured.

Observation 3: If the entry condition of LP-WUS monitoring is configured, the MR-based threshold is always configured and LR-based threshold is optionally configured.

In case LR-based threshold is also configured together with MR-based threshold for the entry condition, how UE determines the entry condition for LP-WUS monitoring is fulfilled should be considered, i.e. when either condition or both conditions are fulfilled. Considering MR-based entry condition has been agreed as the baseline, and in some case, MR-based measurement cannot reflect the channel condition of LR measurement, then, LR-based measurement should be considered as the complimentary on top of MR-based measurement for entry condition. Thus, it is more reasonable that the entry condition is fulfilled when both MR-based threshold and LR-based threshold are met.

Proposal 2: If LR-based threshold is configured for the entry condition, the entry condition is fulfilled when both MR-based threshold and LR-based threshold are met.

In RAN2#126 meeting, it was agreed that exit condition is defined as the serving cell measurement result based on LR is below a threshold. In case LR-based threshold is configured for the entry condition, another issue is whether the value of LR-based threshold for entry condition is the same as that for exit condition. In our view, the entry and exit conditions of LR measurement could reflect the coverage of LP-SS/WUS, if serving cell measurement via LR is above the threshold, it means UE is in the coverage of LP-SS/WUS. On the contrary, if serving cell measurement via LR is below the threshold, it means UE is out of the coverage of LP-SS/WUS. There is no motivation to use different thresholds for coverage determination. But on the other hand, one threshold used for both entry condition (above the threshold) and exit condition (below the threhosld) may cause pingpang issue. Thus, we propose:

Proposal 3: RAN2 to discuss whether the value of LR-based threshold for the entry condition (above the threshold) is the same as that of LR-based threshold for the exit condition (below the threshold).

Regarding the exit condition, in addition to LR-based threshold, whether MR-based threshold should be used has not been decided. In our view, the LR-based threshold would be enough, as the quality of LP-SS can reflect channel quality for LP-WUS, and the UE should switch back to normal operation via MR in case the measurement via LR is lower than the threshold. Thus, there is no need for MR-based threshold for exit condition.

Proposal 4: The exit condition for LP-WUS monitoring doesn’t need to include MR-based threshold.

  - LP-WUS wake up procedure

The purpose of LP-WUS in RRC_IDLE/RRC_INACTIVE mode is to trigger the paging monitoring. Regarding the paging occasion associated with the paging triggered by LP-WUS, RAN1 achieved the following agreement in RAN1#116 meeting [5]. However, RAN1 did not reach a consensus on the support of dynamic PO in RAN1#118 meeting [6].

Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO

The benefit for LP-WUS is to achieve significant UE power saving gain, while the paging latency is also an important metric for LP-WUS performance. As shown in Figure 1 below, if the dynamic PO is designed to be located in front of the legacy PO after UE receives the LP-WUS and wakes up from ultra-deep sleep state, the paging latency could be reduced.

Figure 1: Latency for LP-WUS based on dynamic PO

In the ideal situation, the dynamic PO is located just in/after the occasion that UE completes the ramp-up and synchronization procedure. Given the current typical DRX cycle is 1.28s, the dynamic PO solution could reduce paging latency by an average of around 600 ms. Although it's hard to get the perfect effect as the ideal case, we suppose with suitable dynamic PO designs, the reduced latency could be close to 600 ms when DRX cycle of 1.28s is applied. Thus, we think the dynamic PO is worth to be studied from the paging latency aspect.

Proposal 5: RAN2 to discuss whether dynamic PO is supported.

Besides, as legacy PEI design, the MR in ultra-deep sleep/monitoring LP-WUS should also wake up in case there is UL data/signalling arrival.

Proposal 6: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when monitroing LP-WUS.

  - LP-WUS Subgrouping

In RAN2#125bis meeting, the subgrouping methods were agreed and the maximum number of subgroups is FFS [1]. And in RAN2#126 meeting, RAN2 determines the maximum number of subgroups for LP-WUS is no less than 8 [2]. In our view, the detail number would be further discussed and determined by RAN1 and there is no need to discuss further in RAN2.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

Regarding the subgrouping method for LP-WUS, RAN2 agreed that the PEI subgrouping method is taken as baseline, i.e. CN assigned and UE_ID based subgrouping. In the following part, we will analyse the specific details for CN assigned and UE_ID-based subgrouping.

For the CN assigned subgrouping, RAN2 has the following agreement in RAN2#126 meeting [2]:

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

We think the similar mechanism and NAS signalling defined for PEI could be reused directly for LP-WUS subgrouping. Which may include the following characteristics:

- UE indicates its CN assigned subgrouping capability for LP-WUS subgrouping to CN during the registration procedure;

- CN assigns the subgrouping ID for LP-WUS to UE during the registration procedure and UE stores the subgrouping ID for LP-WUS in the UE context;

- UE determines whether the current serving cell supports the CN assigned subgrouping for LP-WUS based on the information in SI. If the serving cell supports the CN assigned subgrouping for LP-WUS, UE will monitor the LP-WUS with the CN assigned subgrouping ID.

Proposal 7: For CN assigned LP-WUS subgrouping, RAN2 assumes similar signalling defined for PEI should be reused for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

For the UE_ID based subgrouping, the similar formula as PEI which is as follows could be used as the baseline.

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: - N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 - Ns: number of paging occasions for a PF - UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 - subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

Proposal 8: For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

In RAN1#116 meeting, RAN1 made the following agreement [5]. According to RAN1 agreement, it is possible that one UE monitors both LP-WUS and PEI.

Agreement For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

In case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgrouping. According to the following Figure 2 and Figure 3, assuming there are 1024 UEs supporting both LP-WUS and PEI subgrouping sharing the same PO and the subgrouping number of LP-WUS and PEI are both 4, in case UE_ID#0 is paged:

- In Figure 1, LP-WUS subgrouping and PEI subgrouping are totally same, 256 UEs will be woken up by LP-WUS to monitor PEI, and all of them will be indicated by PEI to monitor the following PO.

- In Figure 2, LP-WUS subgrouping and PEI subgrouping are orthogonal, 256 UEs will be woken up by LP-WUS to monitor PEI, while only 64 UEs will be indicated by PEI to monitor the corresponding PO.

Figure 2: LP-WUS and PEI subgrouping are same

Figure 3: LP-WUS and PEI subgrouping are diverse/ orthogonal

It could be observed that if the subgroup ID of LP-WUS and PEI are the same, there is no benefit for PEI monitoring, instead it leads to more power consumption as UE has to monitor PEI after LP-WUS. However, if the subgroup ID of LP-WUS and PEI are independent, the false alarm rate could be further reduced with PEI.

Observation 4: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

To achieve additional power saving gain as analyzed above and considering both LP-WUS and PEI support UE_ID based subgrouping and CN assigned subgrouping, the following options could be considered to achieve more power saving gain:

- For CN assigned subgrouping, it is expected that the CN could assign different subgrouping ID for LP-WUS and PEI for the same UE as much as possible. However, it is up to the SA2/CT1 design and up to network implementation to assign same or different subgrouping ID for the same UE.

- For UE_ID based subgrouping, different bits in 5G-S-TMSI could be used as UE_ID for LP-WUS and PEI subgrouping.

- For case CN assigned subgrouping is used for one feature (i.e. LP-WUS or PEI), while UE_ID based subgrouping is used for another feature, considering different subgrouping methods are used, it is expected that independent subgrouping ID is assigned/calculated for LP-WUS and PEI, naturally. It could be achieved by the restriction that the number of CN assigned subgrouping is same within the registration area, which is up to network implementation.

Proposal 9: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected independent subgroup ID is applied for a UE.

Proposal 10: For UE_ID based subgrouping, different bits in 5G-S-TMSI are used as UE_ID for LP-WUS subgrouping and the PEI subgrouping.

  - RAN2 impacts on RAN1 LS

In RAN1#118 meeting, RAN1 sent an LS to RAN2 which is shown as follows [7]:

# 1 Overall description In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. # 2 Actions To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

According to the LS, it could be observed that the scenario that the UE’s MR is on one band, e.g. band#1, which couldn’t support the LP-WUS. In this case, some one may argure that the LP-WUS feature couldn’t be used when UE camps on this band. However, in our understanding, the intention of the LS sent by RAN1 is to trigger RAN2/RAN4 to identify the coresonding impacts if LP-WUS is to be supported in this case.

Observation 5: The intention for RAN1 LS is to check RAN2/RAN4 impacts on supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR.

In our view, the following alternatives could be considered to support LP-WUS in this case:

- Alt1: UE camps on the band supporting LP-WUS with higher priority.

- Alt2: Supporting the scenario that UE camps on one band, LP-WUS monitoring on another band.

To achieve Alt1, the frequency priority should be set based on whether supporting LP-WUS in cell reselection. That is to say, the frequency priority of the band supporting LP-WUS should have higher for UE supporting LP-WUS. It could be observed that the Alt1 will impact the legacy UE cell reselection procedure. Besides, since all UEs supporting LP-WUS will camp on the band(s) supporting LP-WUS and the number of UEs supporting LP-WUS may be large, the cell supporitng LP-WUS will have high load in this case, which will impact the network performance.

Observation 6: Alt1, i.e. UE camps on the band supporting LP-WUS with higher priority, has the following RAN2 impacts:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

To achieve Alt2, the LR measurement should always be considered for entry condition for LP-WUS monitoring, since MR measurement couldn’t refelect the LR measurement well in case MR and LR are on different bands. However, since RAN2 have agreed the LR threshold could be configured for entry condition for LP-WUS monitoring, this impact is small. Besides, since the LP-WUS frequency may be different from current serving cell and there may be several frequencies supporting LP-WUS, the serving cell may need to broadcast more than one LP-WUS frequencies for UEs supporting LP-WUS. However, this impact only requires multiple but similar configurations for LP-WUS for different frequencies, which is simple and has little specification impact.

Observation 7: Alt2, i.e. supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, has the following RAN2 impacts:

- LR threshold is mandatory configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast multiple but similar configuration for LP-WUS for different frequencies.

With the above, we think Alt2 should be considered as it less RAN2 impacts from RAN2 point of view, considering Alt1 will impact the cell reselection principle largely due to one feature and impact the network performance. Anyway, this could be further discussed after RAN1 makes the decision on whether/how to support this case. Regarding the reply LS, we should provide the corresponding RAN2 imapcts.

Proposal 11: Reply LS to RAN1 to include the solutions to supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR and the corresponding impacts.

A draft reply LS is provided in Annex.

Proposal 12: RAN2 to consider the draft reply LS in Annex.

- Conclusion

In this contribution, we discuss LP-WUS configuration, the entry and exit condition, wake up procedure, subgrouping and RAN1 LS. Based on the discussion, we have the following observations and proposals.

Observations：

Observation 1:  It is up to UE implementation to determine whether to start the LP-WUS monitoring when the entry condition is fulfilled.

Observation 2:  It is up to UE implementation to determine whether to stop the LP-WUS monitoring when the exit condition is fulfilled.

Observation 3: If the entry condition of LP-WUS monitoring is configured, the MR-based threshold is always configured and LR-based threshold is optionally configured.

Observation 4: False alarm rate could be further reduced if the subgrouping ID of LP-WUS and PEI are independent.

Observation 5: The intention for RAN1 LS is to check RAN2/RAN4 impacts on supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR.

Observation 6: Alt1, i.e. UE camps on the band supporting LP-WUS with higher priority, has the following RAN2 impacts:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

Observation 7: Alt2, i.e. supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, has the following RAN2 impacts:

- LR threshold is mandatory configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast multiple but similar configuration for LP-WUS for different frequencies.

Proposals：

LP-WUS configuration:

Proposal 1: LP-WUS configuration is provided in SIB1.

Entry/Exit condition of using LP-WUS:

Proposal 2: If LR-based threshold is configured for the entry condition, the entry condition is fulfilled when both MR-based threshold and LR-based threshold are met.

Proposal 3: RAN2 to discuss whether the value of LR-based threshold for the entry condition (above the threshold) is the same as that of LR-based threshold for the exit condition (below the threshold).

Proposal 4: The exit condition for LP-WUS monitoring doesn’t need to include MR-based threshold.

LP-WUS wake up procedure:

Proposal 5: RAN2 to discuss whether dynamic PO is supported.

Proposal 6: MR should wake up for UE-initiated UL transmission, e.g. RACH/MO-SDT/RNA update/TA update, due to UL data and signaling arrival when monitroing LP-WUS.

LP-WUS Subgrouping:

Proposal 7: For CN assigned LP-WUS subgrouping, RAN2 assumes similar signalling defined for PEI should be reused for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

Proposal 8: For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

- UE_ID is related to 5G-S-TMSI,

- N is the number of total paging frames in one DRX cycle,

- Ns is the number of the PO for a PF,

- Np is the number of subgroupNumForUEID for PEI, if configured; otherwise, Np is 1,

- subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Proposal 9: In order to allow UE to use LP-WUS together with PEI to reduce paging false alarm rate, it is expected independent subgroup ID is applied for a UE.

Proposal 10: For UE_ID based subgrouping, different bits in 5G-S-TMSI are used as UE_ID for LP-WUS subgrouping and the PEI subgrouping.

RAN2 impact on RAN1 LS:

Proposal 11: Reply LS to RAN1 to include the solutions to supporting LP-WUS in case UE may not support LP-WUS on all the bands supported by the MR and the corresponding impacts.

Proposal 12: RAN2 to consider the draft reply LS in Annex.

References

- R2-2403732, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD

- Draft_Minutes_report_RAN1#116b_v030

- R1-2401937, Report of RAN1#116 meeting

- Draft_Minutes_report_RAN1#118_v030

- R2-2407921/R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode

Annex - draft reply LS to RAN1 on LP-WUS operation

Title:	[Draft] reply LS on LP-WUS operation in IDLE/INACTIVE mode

Response to:	R2-2407921/R1-2407559

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	TBD [to be RAN2]

To:	RAN1

Cc: 	RAN4

Contact Person:

Name:	              TBD

E-mail Address:	 TBD

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in [R2-2407921/R1-2407559].

RAN2 has discussed this issue, and has identified the following solutions on how to support it and the corresponding RAN2 impacts [To be updated based on the discussion conclusion]:

Alt1: UE camps on the band supporting LP-WUS with higher priority, the corresponding RAN2 impacts include:

- Cell reselection: The frequency supporting LP-WUS should have higher priority for UE supporting LP-WUS.

- Network load: The frequency supporting LP-WUS will have high load for UEs supporting LP-WUS.

Alt2: Supporting the scenario that UE camps on one band, LP-WUS monitoring on another band, the corresponding RAN2 impacts include:

- LR threshold is mandatory configured for entry condition for LP-WUS monitoring.

- Network needs to broadcast multiple but similar configuration for LP-WUS for different frequencies.

2. Actions:

To RAN WG 1:

RAN2 kindly request RAN1 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #128		18th Nov. – 22nd Nov. 2024		Orlando, USA

TSG RAN WG2 Meeting #129		17th Feb. – 21st Feb. 2025		Athens, Greece