3GPP TSG-RAN WG2 Meeting #127bis		R2-2408116

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:	Discussion on LP-WUS/WUR in RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In RAN2#126 and RAN2#127 meetings [1][2], some progress on LP-WUS operation methods to trigger PDCCH monitoring identified by RAN1 has been made in RAN2, as below.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Meanwhile, RAN1 has agreed some detailed design for option 1-2 and working assumption to support both Option 1-1 and Option 1-2 in RAN1#118 meeting [3].

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18. - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration). - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration. - LP-WUS triggers the start of a timer during which UE monitors PDCCH. - FFS the timer is existing timer and/or new timer. - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected. - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL. - No impact on RRM/RLM/BFD measurement requirements is assumed. - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not. - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS. Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

In this contribution, we will further discuss the RAN2 impacts on RRC_CONNECTED procedures with LP-WUS/WUR from RAN2 perspective.

- Discussion

  - RAN2 impacts on Option 1-2

2.2.1 LP-WUS monitoring and PDCCH monitoring behaviour

As agreed in RAN1 above, for Option 1-2, “LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring”, this is basic design for option 1-2.

Observation 1: For Option 1-2, RAN1 agreed LP-WUS monitoring is performed outside at least legacy C-DRX Active Time.

Regarding the legacy C-DRX Active Time, according to the MAC specification [4], it could be summarized as below:

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

One example of C-DRX Active Time is shown in Figure 1, in which UE monitors the LP-WUS at least within Part 1 as below.

Figure 1: C-DRX time partitioning example

Furthermore, according to the latest agreement in RAN1#118 meeting for Option 1-2 “UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS”, it could be observed that the legacy PDCCH monitoring trigger mechanism via C-DRX cycle is not applied, and PDCCH monitoring should be triggered by LP-WUS for this duration. That is to say, UE monitors the LP-WUS within the duration indicated by legacy drx-onDurationTimer (i.e., Part 3 in Fig. 1) and monitors the PDCCH upon being triggered by LP-WUS in case there is downlink transmission for the UE to avoid the unexpected data latency.

Proposal 1: For Option 1-2, LP-WUS monitoring is performed outside legacy C-DRX Active Time and within the duration indicated by legacy drx-onDurationTimer, i.e. Part 1 and Part 3 as above.

In RAN2#127 meeting, both RAN1 and RAN2 agreed that in Option 1-2, LP-WUS triggers a timer for PDCCH monitoring, which is shown in the following figure. During this time, UE will monitor the PDCCH as legacy.

Figure 2: PDCCH monitoring triggered by LP-WUS for Option 1-2

It was agreed in both RAN1 and RAN2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

Observation 2: After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH.

According to MAC specification [4], the definition for C-DRX Active Time is the duration when UE is expected to monitor PDCCH. With the same logic and definition, the timer for PDCCH monitoring triggered by LP-WUS should belong to the C-DRX Active Time.

Proposal 2: For Option 1-2, the timer duration for PDCCH monitoring triggered by LP-WUS is defined as part of C-DRX Active Time.

Besides, it is still FFS in both RAN1 and RAN2 whether the timer for PDCCH monitoring triggered by LP-WUS is a newly defined timer or legacy timer. In our understanding, the essential point is the behaviour during this time duration, including PDCCH monitoring and L1 reporting, e.g. CSI, L1-RSRP, etc., instead of the concept on the term of timer. In the following, we will discuss the other corresponding behaviour during this time triggered by LP-WUS.

If P3 is agreeable, i.e. the timer for PDCCH monitoring triggered by LP-WUS is defined as part of C-DRX Active Time, the UE behaviour within this timer duration triggered by LP-WUS (i.e., Part 4-1 and Part 4-2 in Fig.3 as below) should be the same as the legacy DRX Active Time. There is no motivation to differentiate the L1 reporting behaviour as legacy C-DRX Active Time. Thus, it is straightforward for UE to report the periodic CSI on PUCCH, Semi-periodic CSI report on PUCCH and Semi-periodic CSI report on PUSCH during this timer triggered by LP-WUS.

Figure 3: Time duration while introducing Option 1-2 LP-WUS

Proposal 3: For Option 1-2, during the timer for PDCCH monitoring triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report during this timer triggered by LP-WUS.

In RAN2#127 meeting, it was agreed UE monitors PDCCH during this time (i.e., Part 4-1 and Part 4-2 in Fig.3), then, there is no need for UE to monitor LP-WUS, as UE already starts the PDCCH monitoring. On the other hand, considering RAN1 has agreed UE only monitors the LP-WUS outside the C-DRX Active Time, and the timer duration for PDCCH monitoring triggered by LP-WUS is defined as part of C-DRX Active Time as in Proposal 3, then, LP-WUS is not expected to be monitored during this timer.

Proposal 4: For Option 1-2, UE is not expected to monitor LP-WUS within the timer for PDCCH monitoring triggered by LP-WUS.

Furthermore, another key point on whether the timer is a newly defined timer or legacy timer is the length of this time. We understand it should be configured by gNB similar as the existing drx-onDurationTimer timer. The existing drx-onDurationTimer timer is designed to cover the uncertainty of whether the traffic arrivals including traffic jitter, while the timer for PDCCH monitoring triggered by LP-WUS here is used for the confirmed traffic which initiates LP-WUS transmission to trigger the following PDCCH monitoring. There is no motivation to restrict the timer length for this timer is the same as the legacy drx-onDurationTimer. Thus, a cleaner and flexible approach is preferred, i.e., the length should be separate from the legacy drx-onDurationTimer. Usually, this time should be shorter than the legacy drx-onDurationTimer, but it is up to gNB configuration anyway.

Proposal 5: For Option 1-2, the length of the timer for PDCCH monitoring triggered by LP-WUS is configured by gNB. The length of this time is separate from the legacy drx-onDurationtimer.

After this timer expires, it is obvious that the UE will not continue to monitor PDCCH and should switch back to monitor the LP-WUS. According to the definition of C-DRX Active Time in MAC specification [4], it is not C-DRX Active Time after this timer expires.

Proposal 6: For Option 1-2, after the timer for PDCCH monitoring triggered by LP-WUS expires, the UE stops PDCCH monitoring, and switch back to LP-WUS monitoring.

Another remaining issue for Option 1-2 is the behaviour in the legacy drx-onDurationTimer duration (i.e., Part 3 in Fig.3).  RAN1 has the following agreement in RAN1#118 meeting [5], regarding the periodic CSI/L1-RSRP reporting:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1). - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up. - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not.

The same issues happened in Rel-16 DCP and we concluded that UE determines whether to report the periodic CSI/L1-RSRP based on the network configuration, i.e., UE determines whether to report the periodic CSI that is L1-RSRP on PUCCH based on the ps-TransmitPeriodicL1RSRP parameter and whether to report the periodic CSI that is not L1-RSRP on PUCCH based on the ps-TransmitOtherPeriodicCSI parameter. We think the same principle could be considered for here for legacy drx-onDurationTimer in Option 1-2. Thus, we propose:

Proposal 7: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Besides, we think LP-WUS could also work together with the existing feature, e.g. PDCCH skipping. In this case, PDCCH skipping indication could also be considered to stop the PDCCH monitoring. More details could be further discussed based on RAN1 progress.

Proposal 8: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring triggered by LP-WUS upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

2.2.2 Impacts on other legacy C-DRX Active Time

RAN1 agreed when Option 1-2 is configured, PDCCH monitoring behaviour related to other legacy DRX timers won’t be affected. However, according to current C-DRX pattern, the other C-DRX Active Time not only includes the time duration triggered by DRX timers but also includes the time duration related to ra-ContentionResolutionTimer, msgB-ResponseWindow, SR, CFRA for BFR, RACH-less LTM/handover procedure as we have shown in section 2.2.1.

Since the LP-WUS monitoring behaviour for Option 1-2 will not be performed within C-DRX Active time, the PDCCH monitoring behaviour should not be affected. Meanwhile, there is no need to differentiate the other C-DRX Active Time related to DRX timer and the C-DRX Active Time not related to DRX timer.

With the above, we think in addition to the conclusion in RAN1, the other C-DRX Active time should also be included, which is as follows:

Proposal 9: For Option 1-2, PDCCH monitoring behaviors related to legacy C-DRX Active Time other than drx-onDurationTimer are not affected. The C-DRX Active Time includes:

  - ra-ContentionResolutionTimer, msgB-ResponseWindow, SR related C-DRX Active Time, CFRA for BFR related C-DRX Active Time, RACH-less LTM/handover procedure.

  - RAN2 views on LP-WUS options

In RAN1#118 meeting, RAN1 has the following working assumption on how to support the LP-WUS options in RRC_CONNECTED:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE. Note: Above can be revisited considering RAN2 decisions.

From the evaluation in TR [5], Option 1-2 could achieve better performance on power saving gain and UPT gain than Option 1-1.

- From the power saving gain aspect, Option 1-2 could achieve up to more than 60% power saving gain for deep sleep while Option 1-1 could achieve up to more than 30%;

- From the UPT gain aspect, Option 1-2 has up to more than 10% UPT gain for FTP traffic and up to more than 180% UPT gain for IM traffic while Option 1-2 has less than 10% loss.

Observation 3: Option 1-2 could achieve more power saving gain and UPT gain than Option 1-1.

According to the above RAN1 Working Assumption in RAN1#118 meeting, it could be observed that:

Observation 4: RAN1 supports Option 1-1 and Option 1-2 for PDCCH monitoring triggered by LP-WUS with C-DRX configuration in RRC_CONNECTED mode.

In our view, since Option 1-2 could achieve better performance than Option 1-1 or all legacy power saving mechanisms, RAN2 should support at least Option 1-2 in Rel-19.

Considering RAN1 has made the working assumption to support both Option 1-1 and Option 1-2, we are fine to follow RAN1 conclusion, i.e. support both Option 1-1 and Option 1-2. Besides, we think the design for Option 1-1 and Option 1-2 should be as common as possible.

Based on our analysis, the following RAN2 impacts could be the common part for Option 1-1 and Option 1-2:

- Periodic CSI/L1-RSRP report/SP CSI: within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP and SP CSI.

- PDCCH monitoring behaviour: UE doesn’t monitor LP-WUS within the timer duration for PDCCH monitoring triggered by LP-WUS. Besides, UE PDCCH monitoring behaviors related to legacy C-DRX Active Time other than onduration are not affected.

- LP-WUS occasion collides with C-DRX Active Time/measurement gap/BWP switching interruption length/RAR for BFR: UE won’t monitor the LP-WUS in these cases.

- Impact on RRM/RLM/BFD/CSI measurement: Since RRM/RLM/BFD/CSI measurements are performed via MR as in legacy and there is no impact on LR, LP-WUS won’t impact the RRM/RLM/BFD/CSI measurement which is similar as the Rel-16 DCP.

- Timeline design between LP-WUS reception and PDCCH monitoring: Minimum gap includes LP-WUS processing time and MR transition time for ramp up and does not include the duration for time/frequency synchronization of MR.

It could be found that most design for Option 1-1 and Option 1-2 has commonality. Thus, the total work load in RAN2 is controllable. Thus, it could be confirmed that both Option 1-1 and Option 1-2 are supported from RAN2 perspective.

With this, it could be up to network to determine whether to configure Option 1-1 or Option 1-2 or both for a cell flexibly.

Proposal 10: RAN2 confirms both Option 1-1 and Option 1-2 are supported to work alone for LP-WUS in RRC_CONNECTED mode.

Propose 11: Option 1-1 and Option 1-2 should share the same design as much as possible, including:

- Periodic CSI/L1-RSRP report and SP CSI report;

- PDCCH monitoring behaviour;

- LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR;

- Impact on RRM/RLM/BFD/CSI measurement;

- Timeline design between LP-WUS reception and PDCCH monitoring.

For Option 1-3, RAN1 didn’t have any conclusion on it. Since Option 1-3 is transparent to MAC, which only impacts the physical layers design, we think it is better to wait for RAN1 progress, if any. And RAN2 should focus on Option 1-1 and Option 1-2 and won’t discuss Option 1-3 unless RAN1 triggers.

Proposal 12: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

  - Enable and disable for LP-WUS monitoring

RAN1 has also agreed that gNB RRC signalling should be used to enable/disable the operation of PDCCH monitoring triggered by LP-WUS, but it is still FFS whether to support UE assistance for the enable/disable functionality.

For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

We think the enable/disable operation could rely on the configuration of LP-WUS as usual in RAN2. Regarding the additional UE assistance, according to the WID in [1], current RRM/RLM/BFD/CSI/BM measurement and corresponding report are performed by MR as legacy. In this way, network has enough information to enable/disable the LP-WUS functionality. Besides, the enable/disable operation is a long-term configuration. There is no need to dynamically change the configuration. Therefore, we think additional assistance information from UE to gNB for enable/disable operation is not necessary.

Proposal 13: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Regarding the RRC message to carry the LP-WUS configuration, RAN2 have agreed the dedicated RRC message is used.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In our understanding, it is straightforward to use the RRCResume and RRCReconfiguration messages as usual.

Proposal 14: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

  - Activation and deactivation procedure for LP-WUS monitoring

Regarding the activation and deactivation for LP-WUS monitoring, RAN1 has the following options:

- In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

From RAN2 perspective, the meaning of enabling LP-WUS monitoring is to activate the LP-WUS functionality while UE couldn’t use LP-WUS if the LP-WUS functionality has some activation mechanisms. In this case, UE could use the LP-WUS functionality after receiving the LP-WUS activation signalling, and UE stops using the LP-WUS functionality upon receiving the LP-WUS deactivation signalling.

Considering the LP-WUS configuration overhead is comparable to other dedicated configurations, e.g. L3 measurement configuration, besides the LP-WUS monitoring is not an urgent behaviour, there is no motivation to introduce additional activation/deactivation procedure for LP-WUS monitoring. UE could use the LP-WUS when the LP-WUS monitoring is enabled, i.e., when UE receives the RRC dedicated signalling with LP-WUS configuration. And if the network doesn’t expect the UE to use the LP-WUS, it could just send the RRC signalling to disable the LP-WUS monitoring for the UE.

Proposal 15: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- Conclusion

In this contribution, we discuss the LP-WUS triggers PDCCH monitoring in RRC_CONNECTED. Based on the discussion, we have the following observations and proposals.

Observations:

Observation 1: For Option 1-2, RAN1 agreed LP-WUS monitoring is performed outside at least legacy C-DRX Active Time.

Observation 2: After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH.

Observation 3: Option 1-2 could achieve more power saving gain and UPT gain than Option 1-1.

Observation 4: RAN1 supports Option 1-1 and Option 1-2 for PDCCH monitoring triggered by LP-WUS with C-DRX configuration in RRC_CONNECTED mode.

Proposals:

LP-WUS monitoring and PDCCH monitoring behaviour on Option 1-2

Proposal 1: For Option 1-2, LP-WUS monitoring is performed outside legacy C-DRX Active Time and within the duration indicated by legacy drx-onDurationTimer, i.e. Part 1 and Part 3 as above.

Proposal 2: For Option 1-2, the timer duration for PDCCH monitoring triggered by LP-WUS is defined as part of C-DRX Active Time.

Proposal 3: For Option 1-2, during the timer for PDCCH monitoring triggered by LP-WUS is running, periodic CSI/L1-RSRP report and SP CSI report behaviour are the same as legacy drx-onDurationTimer duration without DCP configuration, i.e. always report during this timer triggered by LP-WUS.

Proposal 4: For Option 1-2, UE is not expected to monitor LP-WUS within the timer for PDCCH monitoring triggered by LP-WUS.

Proposal 5: For Option 1-2, the length of the timer for PDCCH monitoring triggered by LP-WUS is configured by gNB. The length of this time is separate from the legacy drx-onDurationtimer.

Proposal 6: For Option 1-2, after the timer for PDCCH monitoring triggered by LP-WUS expires, the UE stops PDCCH monitoring, and switch back to LP-WUS monitoring.

Proposal 7: For Option 1-2, within legacy drx-onDurationTimer duration and the MAC entity is not in Active Time, it is up to network configuration to determine whether to report the periodic CSI/L1-RSRP, i.e. same as in Rel-16 DCP.

Proposal 8: For Option 1-2, when LP-WUS is used together with PDCCH skipping, the UE stops the PDCCH monitoring triggered by LP-WUS upon receiving the PDCCH skipping indication, and starts the LP-WUS monitoring.

Impacts on other legacy C-DRX Active Time on Option 1-2

Proposal 9: For Option 1-2, PDCCH monitoring behaviors related to legacy C-DRX Active Time other than drx-onDurationTimer are not affected. The C-DRX Active Time includes:

  - ra-ContentionResolutionTimer, msgB-ResponseWindow, SR related C-DRX Active Time, CFRA for BFR related C-DRX Active Time, RACH-less LTM/handover procedure.

RAN2 views on LP-WUS options

Proposal 10: RAN2 confirms both Option 1-1 and Option 1-2 are supported to work alone for LP-WUS in RRC_CONNECTED mode.

Propose 11: Option 1-1 and Option 1-2 should share the same design as much as possible, including:

- Periodic CSI/L1-RSRP report and SP CSI report;

- PDCCH monitoring behaviour;

- LP-WUS occasion collides with C-DRX Active Time/ measurement gap/BWP switching interruption length/RAR for BFR;

- Impact on RRM/RLM/BFD/CSI measurement;

- Timeline design between LP-WUS reception and PDCCH monitoring.

Proposal 12: RAN2 won’t discuss Option 1-3 unless RAN1 triggers.

Enable and disable for LP-WUS monitoring

Proposal 13: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 14: LP-WUS configuration is included in RRCResume/RRCReconfiguration message.

Activation and deactivation procedure for LP-WUS monitoring

Proposal 15: RAN2 assumes UE could monitor LP-WUS upon LP-WUS is enabled, no additional indication/condition are needed for activation/deactivation of LP-WUS functionality. The final decision is up to RAN1.

- References

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- R2-2407572	Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

- Chairman’s Notes of RAN1 #118.

- 3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

- TR 38.869 for LP-WUS WUR.