3GPP TSG-RAN WG2 Meeting #127bis		R2-2408188

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #127 meeting, the following agreements was reached related to L1 measurement event evaluation and reporting[1]:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we will discuss the issues related to LTM measurement event evaluation and reporting, including:

- How to accociate the event configuration and measurement resource configuration for event-trigggered L1 measurement reporting;

- Whether parameters like L3 reportOnLeave and reportAmount should be supported in event-trigggered L1 measurement reporting;

- What information should be included in the measurement reporting MAC CE.

- Discussion

  - Event configuration signaling design

In RAN2 #125bis meeting[2], it was agreed:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Later in RAN2 #127 meeting[1], the following agreements were reached:

- R19 LTM event triggered measurement configuration can be taken as baseline:

  - LTM measurement resource configuration is provided in LTM-config.

  - Event triggered report config is provided in serving cell config.

To associate L1 measurement resource configuration with L1 LTM measurement event configuration, we understand the simplest and most logical way is to reuse the legacy Rel-18 method, i.e.

- Include the L1 LTM measurement event configuration in LTM-CSI-ReportConfig;

- Associate L1 measurement resource configuration with L1 LTM measurement event configuration by using the ‘ResourceConfigId’.

LTM-CSI-ReportConfig-r18 ::=      SEQUENCE {

ltm-CSI-ReportConfigId-r18                     LTM-CSI-ReportConfigId-r18,

ltm-ResourcesForChannelMeasurement-r18         LTM-CSI-ResourceConfigId-r18,

ltm-ReportConfigType-r18                           CHOICE {

periodic-r18                                       SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList-r18                         SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH-r18                          SEQUENCE {

reportSlotConfig-r18                               CSI-ReportPeriodicityAndOffset,

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

p0alpha                                            P0-PUSCH-AlphaSetId

},

aperiodic-r18                                      SEQUENCE {

reportSlotOffsetList-r18                           SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-2-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128),

reportSlotOffsetListDCI-0-1-r18                    SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128)

},

...

},

ltm-ReportContent-r18                          LTM-ReportContent-r18,

...

}

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

By this way, the event-triggered measurement report configuration can be configured separately in a report configuration, which can be along with legacy periodic reporting by adding one new report type ‘event-triggered’.

Proposal 1: L1 LTM measurement event configuration is configured in LTM-CSI-ReportConfig, by adding one new report type ‘event-triggered’ with event parameters (e.g. TTT).

Moreover, as we agreed some beam level parameters (e.g. beam level offset) for event configuration, those parameters can be configured in L1 measurement resource configuration, e.g. LTM-CSI-SSB-ResourceSet, as the similar way in Rel-18.

Proposal 2: Beam level parameters (e.g. beam level offset) is configured in L1 measurement resource configuration.

With Proposal 1 and Proposal 2, the Rel-19 event configuration signalling (with reusing R18 framework) can be illustrated as follows:

  - Parameters related to reporting

reportOnLeave

In last RAN2 #126 meeting[3], it was agreed:

Agreements on measurement enhancements for LTM:

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In legacy L3 event-triggered measurement reporting, there is a parameter related to leaving condition which is reportOnLeave, to take L3 A4 event as the example[4]:

EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventA4 SEQUENCE { a4-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useAllowedCellList BOOLEAN },

If reportOnLeave is set to true, then the measurement reporting procedure will be initiated by the UE when the leaving condition applicable for this event is fulfilled. In this way, the network can know timely when the channel quality of candidate cells deteriorates. For L1 measurement report, the motivation for UE reporting related to leaving condition should be further discussed as the measured signalling strength can change more rapidly without L3 filtering, thus L1 reporting may be triggered more frequently. To be simple, as whether UE triggers report when leaving condition is met is configurable by this parameter in legacy L3, we can also make it configurable so the network can decide whether to enable this reporting to have more information on L1 measurement results. Therefore,

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

reportAmount

In the legacy L3 measurement, reportAmount and reportInterval are supported. A UE can repeat to send the measurement several times based on the configured reporting amount after the event is triggered.

In our understanding, by reporting multiple times, the network can know whether a triggered beam consecutively has a good/bad quality. Even the signalling strength can change more rapidly, it can be up to network implementation whether to configure and require UE for multiple beam measurement results associated with a report. Therefore,

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

  - Information in the report

In RAN1 MIMO session, they have discussed what needs to be included in L1 event-triggered measurement report and almost all companies agreed that L1-RSRP and beam ID should be included in the measurement report. In our understanding, this is reasonable and also aligned with legacy reporting framework.

Meanwhile, as in RAN2 mobility discussion, the LTM L1 measurement and reporting are mainly for mobility between cells and the UE may measure beams from multiple cells, we understand that cell ID is also needed. In Rel-18, the cell ID is signaled in an implicit way, as following:

LTM-CSI-ResourceConfig information element

-- ASN1START

-- TAG-LTM-CSI-RESOURCECONFIG-START

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18        LTM-CSI-ResourceConfigId-r18,

ltm-CSI-SSB-ResourceSet-r18         LTM-CSI-SSB-ResourceSet-r18,

...

}

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

-- TAG-LTM-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

That is, LTM candidate cell IDs will be associated to the SSBs in the ltm-CSI-SSB-ResourceList. We understand whether the cell ID in L1 measurement report is explicit or implicit can be FFS for Rel-19 LTM.

At last, according to the agreements from RAN2 #125bis meeting[2]:

Agreements on measurements: 1. L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

We understand that one report configuration includes one event configuration (while one resource configuration ID may be associated to different events), so the report configuration ID should also be informed to network so that the network can know e.g. this measurement reporting is for which event, what is the related configured threshold, etc.

Therefore, we propose:

Proposal 5: The contents in measurement report for event triggered L1 measurement reporting at least include:

-	L1-RSRP;

-	Beam ID;

-	Cell ID, FFS whether explicit or implicit way;

-	Reporting configuration ID (indicates e.g. event type, threshold, etc);

Another issue is how to decide the number of beams of measurement results to be included in the measurement report. Actually, RAN1 has also discussed about this issue and various proposals were raised, e.g. whether variable number of reported beams are supported, whether the reported beams should all satisfy the triggering condition, etc[5].

In Rel-18, the number of cells and RSs per cell are reported within a single L1 measurement report instance are configured by network as follows:

LTM-ReportContent-r18 ::=     SEQUENCE {

nrOfReportedCells-r18                          ENUMERATED {n1,n2,n3,n4},

nrOfReportedRS-PerCell-r18                     ENUMERATED {n1,n2,n3,n4},

spCellInclusion-r18                            ENUMERATED {true}                                          OPTIONAL -- Need R

}

Similarly, we could first decide the maximum number of beams(s) to be reported. It would be simple to take Rel-18 as the baseline, so the total number of beams to be included can be restricted to 16. As for the actual number, we could wait for more RAN1 progress, as they already have some kinds of proposals as below[6]:

RAN1 #117 Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

The agreement is based on UCI to be the UL signalling to carry L1 measurement report. If we agree with MAC CE, the UE can simply report all beams that satisfies the reporting condition, so we could come back to this one after we decide MAC CE or UCI. On the other hand, this is anyway left to RAN1, thus no proposal is given here.

Proposal 6: Taking R18 LTM as the baseline, the maximum number of beam(s) to be reported in the L1 measurement reporting is 16.

Moreover, it is not clear whether measurement result of serving cell should be included. In L3 measurement reporting, as servingCellMO is configured to be related to the MO, the UE would report serving cell measurements result to gNB. For L1 measurement reporting, we understand it is also beneficial to report the measurement results of current beam to gNB for better decision to trigger LTM. Here the current beam, according to RAN1’s agreement, can be understood as a beam corresponding to the indicated TCI state.

In Rel-18, spCellInclusion is introduced to control whether the UE shall include a L1 measurement report associated to the current SpCell. Similarly, network can also control whether L1 measurement results of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be included in the measurement report.

Furthermore, RAN1 also agreed in RAN1 #117 meeting that:

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

Therefore, we propose:

Proposal 7: For event triggered L1 measurement reporting, RAN2 confirm that network can configure whether the L1 measurement result of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be always included in the measurement report.

- Conclusion

In this contribution, we discussed the issues related to LTM measurement event evaluation and reporting. We have the following proposals:

Proposal 1: L1 LTM measurement event configuration is configured in LTM-CSI-ReportConfig, by adding one new report type ‘event-triggered’ with event parameters (e.g. TTT).

Proposal 2: Beam level parameters (e.g. beam level offset) is configured in L1 measurement resource configuration.

Proposal 3: It is configurable by gNB whether a UE initiates L1 measurement reporting when leaving condition is satisfied (e.g. reportOnLeave-like parameter in event configuration).

Proposal 4: It is configurable by gNB whether a UE performs L1 measurement reporting multiple times for one triggered report (e.g. reportAmount-like parameter in event configuration).

Proposal 5: The contents in measurement report for event triggered L1 measurement reporting at least include:

-	L1-RSRP;

-	Beam ID;

-	Cell ID, FFS whether explicit or implicit way;

-	Reporting configuration ID (indicates e.g. event type, threshold, etc);

Proposal 6: Taking R18 LTM as the baseline, the maximum number of beam(s) to be reported in the L1 measurement reporting is 16.

Proposal 7: For event triggered L1 measurement reporting, RAN2 confirm that network can configure whether the L1 measurement result of current beam (i.e. a beam corresponding to the indicated TCI state) in SpCell should be always included in the measurement report.

- References

- RAN2 #127 meeting chairman notes;

- RAN2 #125bis meeting chairman notes;

- RAN2 #126 meeting chairman notes;

- 3GPP TS 38.331 V18.3.0 (2024-09);

- R1-240xxxx, Moderator Summary #4 on UE-initiated/event-driven beam management;

- RAN1 #117 chairman notes;