3GPP TSG-RAN WG2 Meeting #127bis	R2-2408130

Hefei, China, Oct 14 – 18, 2024

Agenda item:	8.7.4.1

Source:	Qualcomm Incorporated

Title:	Discussion on LCP enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we follow up on the agreements from RAN2#126 and discuss additional enhancements needed to support dynamic LCP priority.

# Discussion

In RAN2#126, the following agreements on LCH prioritization were made:

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

Before we discuss technical details of the enhancement and its potential impacts on LCP and other scheduling procedures, we think it is worth discussing how it may be be modelled in the MAC spec. For instance,

- In legacy, after data from an LCH is multiplexed in a TB, the Bj of that LCH is updated (i.e. Bj is deduced by the size of the data). If this data has had its priority adjusted, it needs to be discussed which LCH’s Bj should be adjusted? Because Bj is associated with an LCH, not a priority. Similarly, since LCP restrictions are associated with LCH, not priority, which LCP restrictions should be applied after data has priority adjustment, the ones configured for its original LCH or another one?

- If the new priority of data after its priority adjustment is the highest among all the buffered data, should a BSR be triggered? If yes, when the BSR MAC CE is sent, should the size of the data be included in the data volume calculation of its original LCG or the new LCG? Although LCHs with the same priority are likely to be associated with the same LCG, it is more direct and clearer, at least from modelling perspective, to specify which new LCH instead of which new priority that the data is associated with.

- After a BSR is triggered due to priority adjustment for some data, UE needs to select a SR configuration to request UL grant. This selection depends on the mapping between SR configurations and LCHs. For the same reason as for BSR, it is also more direct and clearer to specify which new LCH instead of new priority that the data is associated with, so that UE can choose the right SR configuration accordingly.

Observation 1.	Priority adjustment may have impact on Bj update, LCP restriction, triggering and reporting in BSR and selection of SR configuration, all of which are associated with LCH instead of priority.

We therefore think that it makes more sense, at least from spec modelling perspective, to assign data to another LCH instead of another priority.

Proposal 1. 	To simplify spec modelling, data is re-assigned to another LCH, rather than to another priority, after its remaining time falls below a configured adjustment threshold.

Next, we discuss configuration aspects of the enhancements, e.g. the two FFS issues.

DSR triggering and LCH/priority adjustment serve different purposes in the UL scheduling process. DSR is triggered to inform network that buffered data starts running low in its remaining time and needs to be scheduled sooner. Priority adjustment is needed only when remaining time of an SDU becomes critical and more/higher scheduling opportunities are needed to ensure it will not miss its deadline.

Based on the differences in their purposes, it is not hard to see that it makes more sense for these two procedures to happen sequentially instead of simultaneously. Or network should at least have the option of being able to separate them (i.e. configuring different triggering thresholds for them). Therefore, a separate threshold should be configured for the LCH/priority adjustment.

Proposal 2.	The LCH adjustment threshold is configured separately from the DSR triggering threshold.

The reason that delay-critical data is given a higher priority than its default is because it is at risk of missing its delay deadline and needs to be scheduled sooner than normal. Otherwise, if data has enough remaining time, it is sufficient to schedule it at its default priority first. This decision does not need to depend on whether there is delay-critical data in the same LCH or not.

To illustrate this point further, consider the following example: suppose two PDU sets arrive in tandem but with a large interval between them. The first PDU set has already become delay critical and is eligible for priority/LCH adjustment when the second PDU set arrives. In this case, it does not make any sense that the second PDU set is given a higher priority immediately upon its arrival only because there is some data in the same LCH has become delay critical.

Therefore, priority adjustment should be applied to only delay-critical data.

Proposal 3. 	Within an LCH, only data whose remaining time is below the adjustment threshold is re-assigned to another LCH.

We think it is necessary to add an additional restriction to the LCH/priority adjustment. Within an LCH, only data which has not been re-assigned from other LCHs can be considered for reassignment, i.e. data can be re-assigned for at most once. This is because otherwise, re-assigned data can be adjusted to another LCH again as its remaining time meets the reassignment threshold. This behavior is not a result of the configuration in which the adjustment threshold is per MAC entity. It can still happen if each LCH has a different adjustment threshold, i.e. once the remaining time of re-assigned data meets the adjustment threshold of its current LCH, it can re-assigned to another LCH again.

Proposal 4.	Within an LCH, only data which has not been re-assigned from other LCHs can be considered for reassignment, i.e. data can be re-assigned for at most once.

Next, we discuss the impact of LCH/priority adjustment on the LCP and other scheduling procedures.

In the LCH prioritization procedure, Bj controls how much data per unit time can be scheduled from an LCH. Now consider an LCH (denoted as LCH#1) that has some data which is re-assigned from another LCH (denoted as LCH#2). After the re-assigned data is multiplexed in a TB, if Bj of LCH #1 is deducted as a result of that, then it is not fair to the data whose original LCH is LCH#1, because the re-assigned data takes away some bandwidth from LCH#1’s PBR. Therefore, even after data is re-assigned to a different LCH, only the Bj of its original LCH is updated when the data is transmitted.

Proposal 5. 	When re-assigned data is transmitted, the Bj of its original LCH is updated. No other LCHs need to update their Bj.

The reason that delay-critical data is re-assigned to a different LCH is to give it more scheduling opportunities. Therefore, after its reassignment, as far as scheduling is concerned, it should be treated the same way as the data incumbent in its re-assigned LCH. This principle should apply to LCP restriction too.

Proposal 6. 	After data is re-assigned to a new LCH, it should be treated the same in the LCH prioritization and apply the same LCP restriction as the incumbent data in that LCH.

After data is re-assigned to a new LCH, if it has a priority higher than any other data in UE’s buffer, then according to the current BSR triggering criteria, a regular BSR should be triggered. We think this behavior makes sense, because in this case network should be informed of the new buffer status as a result of the reassignment, so that the re-assigned data can get a proper UL grant.

Proposal 7. 	After data is assigned to a new LCH, a BSR is triggered if its new priority is higher than that of any other data in the buffer.

After a BSR is triggered, SR is transmitted if no PUSCH resource is available. If there are multiple SR configurations, UE chooses the SR configuration which is associated with the LCH which triggered the BSR. If the BSR is triggered by re-assigned data, we think UE should choose an SR configuration based on the priority of the new LCH of the re-assigned data. The reason is the same as argued in Proposal 6, i.e. the re-assigned data should be scheduled with its new priority.

Proposal 8. 	If an SR is triggered due to data re-assigned to a new LCH, UE should use the SR configuration associated with the new LCH.

Lastly, we discuss how re-assigned data should be reported in BSR and DSR.

In BSR and DSR, data is reported separately for each LCG instead of a total volume in the buffer. That is because network can better schedule LCHs, using per LCG information, e.g. based on their priorities. Therefore, BSR and DSR should accurately reflect the amount of data which is going to be handled/scheduled by the same priority. This means that if data is re-assigned to a new LCH, then the re-assigned data should be reported together with the incumbent data in that LCH.

Proposal 9.	In BSR and DSR, re-assigned data is included in the buffer size of its new LCH.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	Priority adjustment may have impact on Bj update, LCP restriction, triggering and reporting in BSR and selection of SR configuration, all of which are associated with LCH instead of priority.

Proposal 1. 	To simplify spec modelling, data is re-assigned to another LCH, rather than to another priority, after its remaining time falls below a configured adjustment threshold.

Proposal 2.	The LCH adjustment threshold is configured separately from the DSR triggering threshold.

Proposal 3. 	Within an LCH, only data whose remaining time is below the adjustment threshold is re-assigned to another LCH.

Proposal 4.	Within an LCH, only data which has not been re-assigned from other LCHs can be considered for reassignment, i.e. data can be re-assigned for at most once.

Proposal 5. 	When re-assigned data is transmitted, the Bj of its original LCH is updated. No other LCHs need to update their Bj.

Proposal 6. 	After data is re-assigned to a new LCH, it should be treated the same in the LCH prioritization and apply the same LCP restriction as the incumbent data in that LCH.

Proposal 7. 	After data is assigned to a new LCH, a BSR is triggered if its new priority is higher than that of any other data in the buffer.

Proposal 8. 	If an SR is triggered due to data re-assigned to a new LCH, UE should use the SR configuration associated with the new LCH.

Proposal 9.	In BSR and DSR, re-assigned data is included in the buffer size of its new LCH.