3GPP TSG-RAN WG2 Meeting #127bis	R2-2408131

Hefei, China, Oct 14 – 18, 2024

Agenda item:	8.7.4.2

Source:	Qualcomm Incorporated

Title:	Discussion on DSR enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In the RAN2#127 meeting, the following agreements on DSR enhancements were made:

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. - There will be a single triggering threshold, as in Rel-18 - Issues for FFS - whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this paper, we first discuss the FFS issues from the last RAN2 meeting and then discuss additional issues in supporting the enhanced DSR.

# Discussion

One of the FFS issues is whether there should be any constraints on how the NW configures DSR triggering and reporting thresholds.

The motivation to introduce multiple reporting thresholds for DSR is to improve the granularity of delay status information reported in a DSR MAC CE. This goal is achieved if all the reporting thresholds are equal to or lower than the DSR triggering threshold, because all the remaining times reported in a DSR MAC CE are smaller than the triggering threshold.

If one or more reporting thresholds are set higher than the triggering threshold, the additional bytes in DSR MAC CE for the additional reporting thresholds are used to indicate buffer size of non-delay-critical data. That, however, undermines the original purpose of having multiple reporting thresholds, which is to improve the granularity of delay status information. Therefore, in our view, all reporting thresholds for DSR should be equal to or lower than the triggering threshold.

Proposal 1. 	All reporting thresholds are equal to or lower than the triggering threshold.

Another FFS issue is whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. It was also discussed in contributions such as [1][2]. Although this scenario may not be common, it can occur. When it does, the legacy DSR reporting procedure can result in the network under-serving that logical channel. When reporting that type of non-delay critical data in DSR, we think the right remaining time to report is the remaining time of the delay critical data which arrived after it. That is because the purpose of including it is to ensure in-time delivery of the delay critical data buffered behind it.

On the other hand, if one does not want to change the definition of delay critical data and data volume calculation in DSR, an alternative is to have RLC always transmit delay critical data first, irrespective of the order of arrivals in its transmission buffer. We think this approach is simpler and have much less impact on the current spec.

Proposal 2.	In the scenario where non-delay critical data is buffered ahead of delay critical data in the same logical channel, RAN2 discuss and down select the following options:

- Alt 1. The non-delay-critical data should be included in the DSR data volume calculation, and its remaining time should be set to the shortest remaining time among the delay critical data which arrived after it.

- Alt 2. RLC should prioritize the transmission of delay-critical data, regardless of the order in which data arrives in its transmission buffer.

Next, we discuss how buffer size should be reported in DSR MAC CE.

The first issue is defining buffer sizes in DSR MAC CE when there are multiple reporting thresholds. There can be different options to consider. For example, for each reporting threshold, the UE may report the amount of delay critical data whose remaining time is below the reporting threshold. Alternatively, the UE may report the amount of delay critical data whose remaining time falls within the interval between this reporting threshold and the next smaller one (including remaining time of zero). We think the latter option can be more efficient in signaling, as nothing needs to be included in DSR MAC CE for a reporting threshold if there is no data is associated with that threshold.

Proposal 3.	For each reporting threshold, UE reports the amount of data whose remaining time falls within the interval between this reporting threshold and the next smaller one (including remaining time of zero).

For a reporting threshold, if there is no data whose remaining time falls within the interval between this reporting threshold and its next smaller one (including remaining time of 0), then to reduce signaling overhead, nothing corresponding to this reporting threshold needs to be included the DSR MAC CE.

Proposal 4.	If buffer size associated with a reporting threshold is zero, then no delay status information associated with this reporting threshold is included in the DSR MAC CE.

In Rel-18, a new (refined) BSR table was introduced to enhance the granularity of buffer size in BSR MAC CE. The buffer size field in the Rel-18 DSR MAC CE can use either the legacy or the refined BSR table, depend on whether the buffer size of an LCG falls within the range of the refined BSR table. This approach can give network a more accurate view of the amount of delay-critical data that UE has. We think the same approach should be applied to Rel-19 enhanced DSR MAC CE too.

Proposal 5.	If the buffer size associated with a reporting threshold is within the range of the refined BSR table, then it is reported using the refined BSR table.

With multiple reporting thresholds, the Rel-19 DSR MAC CE can be larger than the Rel-18 one. If there is a pending DSR but the available PUSCH resource is too small to include it completely, we think Rel-18 DSR MAC CE can be considered instead. That is because it is still useful to send some basic delay information than sending nothing.

Proposal 6.	If there is a pending DSR but the available PUSCH resource is not large enough to accommodate a R19 enhanced DSR MAC CE, a R18 DSR MAC CE can be sent instead if it fits in the TB.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1. 	All reporting thresholds are equal to or lower than the triggering threshold.

Proposal 2.	In the scenario where non-delay critical data is buffered ahead of delay critical data in the same logical channel, RAN2 discuss and down select the following options:

- Alt 1. The non-delay-critical data should be included in the DSR data volume calculation, and its remaining time should be set to the shortest remaining time among the delay critical data which arrived after it.

- Alt 2. RLC should prioritize the transmission of delay-critical data, regardless of the order in which data arrives in its transmission buffer.

Proposal 3.	For each reporting threshold, UE reports the amount of data whose remaining time falls within the interval between this reporting threshold and the next smaller one (including remaining time of zero).

Proposal 4.	If buffer size associated with a reporting threshold is zero, then no delay status information associated with this reporting threshold is included in the DSR MAC CE.

Proposal 5.	If the buffer size associated with a reporting threshold is within the range of the refined BSR table, then it is reported using the refined BSR table.

Proposal 6.	If there is a pending DSR but the available PUSCH resource is not large enough to accommodate a R19 enhanced DSR MAC CE, a R18 DSR MAC CE can be sent instead if it fits in the TB.

# References

- R2-2407047, Discussion on scheduling enhancements, Ericsson, August 2024.

- R2-2407062, Scheduling Enhancements for XR, Nokia, Nokia Shanghai Bell, August 2024.