3GPP TSG-RAN WG2 Meeting #127bis	R2-2408134

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.7.4.1

Source: 	OPPO

Title:  	Discussion on delay-aware LCP enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in the last RANP#105 meeting, the detailed objectives on enhancement for Scheduling are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on delay-aware LCP enhancements for scheduling.

# Discussion on delay-aware LCP enhancements

In the last few meetings, RAN2 studied two candidate solutions for delay-aware LCP enhancement in Rel-19 XR, and the enhancement of the LCH restriction is ruled out. Here we further discuss the leftover issues on the other alternative, i.e., enhancement of the LCH prioritization, as highlighted below.

LCP enhancements – LCH prioritization The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH LCP enhancements – granularity LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

For LCH prioritization-based LCP enhancement, it is agreed to use additional priority configured to LCHs in case of these LCHs with delay-critical data, but it is FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH. In our understanding, RAN2 has already made conclusion to not consider LCP prioritization within a logical channel, therefore it is straightforward that the priority should apply to the whole LCH. It is still good to further confirm this.

- RAN2 has already made conclusion to not consider LCP prioritization within a logical channel.

- For LCH prioritization-based LCP enhancement, RAN2 to confirm that the additional priority applies for the whole LCH with delay-critical data, i.e., no differentiation for delay-critical/non-critical data within the LCH.

Another leftover issue is that whether a separate remaining time threshold different from the one used for DSR can be configured for delay aware LCP enhancement. In our understanding, a separate threshold has benefit on decoupling DSR and delay-aware LCP enhancement, which could provide more flexibility to NW configuration. Thus, separate threshold is slightly preferred.

- Separate remaining time threshold is introduced for delay-aware LCP enhancement (i.e., different from the one used for DSR).

Other than the leftover issues listed in the previous agreements, the leftover issues on delay-aware LCP enhancement also include how the additional priority should be applied.

In current LCP procedure, for an uplink grant allocating uplink resource for a new transmission, UE needs to select the LCHs that satisfy all LCH restrictions, and then the uplink resource is allocated for selected LCHs in a decreasing LCH priority order. During the phase of resources allocation, it is divided into two rounds, i.e.,

- 1/ Initial resources allocation, i.e., Bj-based resources allocation to achieve PBR-based fairness for all selected LCHs,

- 2/ Remaining resources allocation, i.e., if any resource remains, allocating resource for all selected LCHs regardless of the number of tokens (i.e., Bj).

Specifically, there needs to be clarity on which round during LCP procedure the additional priority is applicable. In our understanding, the determined priority for a LCH should be applied consistently across both rounds of resource allocation. Meanwhile, based on whether there is delay-critical data, MAC entity should determine the priority of a LCH only once before executing the LCP procedures. It implies that there should be no further checks, updates, or fallback mechanisms during the second round of remaining resource allocation in the LCP procedure. In this way, LCP implementation complexity from UE perspective would be simplest.

- From UE perspective, LCP implementation would be simplest if MAC entity determines the priority of a LCH only once before executing the LCP procedures.

- The determined priority for a LCH is applied for both rounds of resource allocation in LCP procedure, i.e., the initial resource allocation considering Bj and the remaining resource allocation regardless of Bj.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- RAN2 has already made conclusion to not consider LCP prioritization within a logical channel.

- For LCH prioritization-based LCP enhancement, RAN2 to confirm that the additional priority applies for the whole LCH with delay-critical data, i.e., no differentiation for delay-critical/non-critical data within the LCH

- Separate remaining time threshold is introduced for delay-aware LCP enhancement (i.e., different from the one used for DSR).

- From UE perspective, LCP implementation would be simplest if MAC entity determines the priority of a LCH only once before executing the LCP procedures.

- The determined priority for a LCH is applied for both rounds of resource allocation in LCP procedure, i.e., the initial resource allocation considering Bj and the remaining resource allocation regardless of Bj.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia