3GPP TSG-RAN WG2 Meeting #127-bis                                 R2-2408138

Hefei, China, 14th – 18th October, 2024

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

In the last RAN2#127 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

Agreements:

- The intended broadcast services area is defined by a geographical area represented by a (set of) reference Location and radius or by a (set of) polygon(s).

- RAN understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2).

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 support of different MBS services/contents in cells covering big area in footprint of the satellite

As mentioned in the last RAN2 meeting agreement, the MBS broadcast intended service area is provided via system information. We aim to further discuss the case that different MBS broadcast is associated with different intended service area, as indicated as follows:

Figure 1: different sub-area belonging to a cell in/corresponding to a satellite footprint could be served with different MBS service

We think the information of different MBS broadcast intended service area for different MBS session could be embedded in the system information for the convenience of the UE to recognize the available MBS services at its current positioning and the areas of broadcasting interested MBS services in the currently camped cell.

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: RAN2 to agree that different MBS sessions could be associated with different MBS broadcast intended service areas, and such information should be indicated in the system information.

- 2.2 support of MBS services/contents in cells area of more than one NTN cells (or portions thereof)

In our opinion, there are two possible situations when MBS services/contents are available in more than one NTN cells:

- MBS services are available in multiple cells

- MBS services are available in at least one whole cell and in only portion of at least one cells

If the MBS services are available in multiple cell, in each such cell, the targeting intended broadcast area does not need to be provided to UEs, since the UE camping on such cell can receive MBS services everywhere in the cell. On the other hand, if the MBS services are available in only part of a cell, related enhancements to be considered are mentioned in the last section. Anyway, we should embrace both of two situations: 1. MBS services are available in one whole cell, and 2. MBS services are available in part of one cell. Therefore, we suggest to mark any stage-3 enhancement for indicating intended targeting MBS service area in the NTN cell as optional.

Proposal 2: RAN2 to agree that, to support MBS services/contents in cell area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

- 2.3 how to indicate the service area information in the Uu signalling?

In the last RAN2 meeting, it was agreed that the expected UE behaviour is that the UE may not need to (re)acquire up-to-date MCCH when the UE is not in any intended service area of its interested broadcast services.

Mainly, there are two options on table: including the service area information in SIB20, or the MCCH consisting of MBSBroadcastConfiguration IE. According to [1], SIB20 contains the information required to acquire the MCCH/MTCH configurations for MBS broadcast. On the other hand, according to [2], the MCCH carries the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session such as MBS session ID, associated G-RNTI scheduling information, etc. MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset.

Apparently, the current spec only arms the SIB20 with the functionalities of carrying the MCCH and MTCH scheduling information. The size of the SIB20 is relatively small, considering the details of the MBMS service information is further included in the MCCH, rather than itself. It seems that SIB20 informing of the intended service area information does not align with its current functionality.

Observation 2: It seems that SIB20 informing of the intended service area information does not align with its current functionality carrying the both of the MCCH and MTCH scheduling information.

On the other hand, indicating the intended service area in the MBSBroadcastConfiguration seems more acceptable to UE. The logic behind is that the information of the intended service area is only viewed as one piece of MBS transmission related information; any UE interested in the MBMS service in the NTN network could find it from the MCCH, as the way how current UEs reads other MBS related detailed information from the MCCH also.

In addition, we find that inclusion of the intended service area related information in the SIB20 or the MBSBroadcastConfiguration is nothing to do with the demand that the UE may not need to perform (re)acquiring up-to-dated MCCH when the UE is not in any intended service area of its interested broadcast services. For the MBSconfiguration option, if the UE finds it is not in any intended services area previously stored in the memory, it can simply choose not to read the MCCH. Also, if the UE is entering in an area matched with any intended service area of its interests stored in its memory, it can trigger the procedure of receiving of the MCCH. In case that the associated intended area has been found to be changed, it can memorize the updated intended broadcast area information.

Observation 3: inclusion of the intended service area related information in the SIB20 or the MBSBroadcastConfiguration is nothing to do with the demand that the UE may not need to perform (re)acquiring up-to-dated MCCH when the UE is not in any intended service area of its interested broadcast services. If the intended service area is included in the MCCH, the UE could just choose not to read the MCCH until it enters an area matched with an intended service area of its interests stored in the memory.

Proposal 3: RAN2 to agree to embed the intended area information in the MBSBroadcastConfiguration IE transmitted on the MCCH.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: RAN2 to agree that different MBS sessions could be associated with different MBS broadcast intended service areas, and such information should be indicated in the system information.

Proposal 2: RAN2 to agree that, to support MBS services/contents in cell area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

Observation 2: It seems that SIB20 informing of the intended service area information does not align with its current functionality carrying the both of the MCCH and MTCH scheduling information.

Observation 3: inclusion of the intended service area related information in the SIB20 or the MBSBroadcastConfiguration is nothing to do with the demand that the UE may not need to perform (re)acquiring up-to-dated MCCH when the UE is not in any intended service area of its interested broadcast services. If the intended service area is included in the MCCH, the UE could just choose not to read the MCCH until it enters an area matched with in the intended service area stored in the memory.

Proposal 3: RAN2 to agree to embed the intended area information in the MBSBroadcastConfiguration IE transmitted on the MCCH.

Reference:

[1] TS 38.331 V18.0.0, Radio Resource Control (RRC) protocol specification, 3GPP RAN2.

[2] TS 38.300 V18.0.0, NR and NG-RAN Overall description, RAN2