3GPP TSG-RAN WG2 Meeting #127bis                                 R2-2408141

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.1.2.2

Source: 	Fujitsu

Title: 	Discussion on LCM for UE-sided Model for Beam Management Use Case

Document for:	Discussion

Introduction

In previous meetings, RAN2 made several agreements on functionality identification [1] related issues and one LS [2] has been sent to RAN1 pending for response. In this contribution, we would like to give our views on other aspects of LCM for UE-sided model for beam management use case as below：

- Performance monitoring.

- Model control.

Discussion

Performance Monitoring

Performance monitoring of UE-side model for beam management has been continuously discussed in RAN1 with minor progress per meeting, the following agreements were made during Rel-19 [3][4]:

Agreement [RAN1 #117] For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report. - FFS on other contents. - The report is at least configured/triggered by NW. - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report. - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring. Agreement [RAN1 #118] For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resource for monitoring. - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resource for monitoring. - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam. - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g., whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

It can be seen from the above agreements that RAN1 has not determined whether the down-selection between Type I/II monitoring or between option 1/2 of Type I monitoring should be made, nor the details of reporting from UE to NW for option 2 monitoring. RAN1 will continue discussing the remaining issues for the next several consecutive meetings.

Observation 1 The details of UE-side BM performance monitoring are still pending RAN1 further conclusions.

RAN2 need to wait for further RAN1 input for further actions. However, no matter NW or UE derives the metrics and what will be reported from UE to NW, RAN2 can have some discussions on at least the mechanism to trigger the performance monitoring, and the potential signaling to complete the monitoring procedure.

Proposal 1 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

From RAN2 point of view, the performance monitoring procedure can be triggered either periodically or event-based, e.g., for option 1, NW can configure UE to periodically report L1-RSRP or BM index to assist NW derives the monitoring metrics, or UE can trigger the procedure for certain events such as UE moving or poor performance.

Proposal 2 The performance monitoring procedure can be triggered either periodically or event based.

For the signaling of the reporting, it is natural that L1 signaling to be used as baseline for L1-RSRP or BM index for option 1 as well as the listed alternatives in option 2. MAC/RRC signaling are also valid to be used regarding the near-real-time latency requirements of performance monitoring.

Proposal 3 L1 signaling can be used as baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

Model control within the functionality

For functionality-based LCM, it is considered the model activation is decided by the UE implementation. The model selection, (de)activation or switching are transparent to the network. Without the instruction from the network, it is possible that the UE wrongly activates a model which is not applicable to the current NW configuration or cannot meet the requirement of the network. To address this issue, NW can send instruction information on applicable configuration/additional condition/requirements for UE to determine the activated functionalities, then UE can do model control within the activated functionalities.

Proposal 4 The network sends the instruction information to UE for determining the activated functionalities and subsequent model control.

Conclusion

Observation 1 The details of UE-side BM performance monitoring are still pending RAN1 further conclusions.

Proposal 1 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

Proposal 2 The performance monitoring procedure can be triggered either periodically or event based.

Proposal 3 L1 signaling can be used as baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

Proposal 4 The network sends the instruction information to UE for determining the activated functionalities and subsequent model control.

References

[1] RAN2 127 Chair Notes.

[2] R2-2407848 LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG2#127	, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[3] Xiaodong's Session Notes RAN1#117 (9.1 AI&ML).

[4] Xiaodong's Session Notes RAN1#118 (9.1 AI&ML).