3GPP TSG-RAN WG2 Meeting #127bis	R2-2408146

Hefei, China, October 14th –18th, 2024

Agenda item:	8.2.3

Source: 	Fujitsu

Title: 	Discussions on AIoT Paging

Document for:	Discussion and decision

1. Introduction

According to the objective specified in the New SID [1], RAN2 needs to study and decide whether Paging is needed for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission and study the Paging mechanism for an Ambient IoT, if needed.

At previous RAN2 meetings, the following agreements on the Paging for an Ambient IoT are reached [2] [3]:

Agreement

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

This contribution will provide more consideration on the Ambient IoT Paging.

2. Discussion

Based on RAN2 discussions, the overall AS procedures is formulated in draft TR38.769 as:

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

Figure 6.2.1-1 Overall AS procedures between A-IoT device and reader [4]

Step B related aspects are discussed in our companion contribution [5].

Step A related aspects are discussed in this contribution.

It seems that the AS procedures with different steps will be used for different use cases. For example, the AS procedure with step A and step B can be used to support inventory-only use case. While, the use case of inventory and command, or use case of command-only can be supported by the procedure with step A, step B, step C1 and step C2. Therefore, it is better to indicate paging cause/type in the AIoT paging message.

Proposal 1: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the AIoT paging message.

RAN2 has agreed that the AIoT paging message can contain an ID of a single device, a group ID mapping to multiple devices or does not contain the ID to trigger all devices. It means that different ID may be included in the message. It is useful to include an indication to identify the ID type. One possibility is to include an explicit indication to notify if device ID or group ID is used in the AIoT paging message. Alternatively, if device ID or group ID is used, it can be indicated implicitly. For example, device ID can be differentiated from group ID according to the component/structure of the AIoT device ID.

Proposal 2: ID type, e.g., device ID or group ID is indicated explicitly or implicitly in the AIoT paging message.

In addition, RAN2 assumes that the AIoT paging message containing multiple IDs of AIoT devices will be studied. On one hand, it is possible that AIoT devices served by one reader don’t belong to a group identified by the group ID(s). On the other hand, AIoT devices in a group identified by the group ID may be served by different readers. During SA2 discussion for e.g. Topology 2, an intermediate UE serving a group of AIoT devices can move among different gNBs. When the UE moves far from these AIoT devices, the AF may reselect another intermediate UE as reader. In this case, device ID(s) are known to the network and thus multiple IDs of these AIoT devices in the AIoT paging message can be used to trigger these AIoT devices for response. Therefore, RAN2 needs to design the signaling or procedure to support the use case.

Also, the AIoT paging message indicates the information from which the AIoT device can determine the resources to be used for response. However, it needs more study on how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device. No matter of dedicated or shared resources, the parameters for resources configuration depend on RAN1. For example, explicit/implicit information of time domain resources, frequency domain resources and repetitions. For each information, RAN1 can further study whether to use the A-IoT paging message or L1 R2D control signaling.

Dedicated resource(s) can be configured for one device or multiple devices. When the resource(s) are configured for multiple devices, multiple device IDs should be included for mapping between the dedicated resource and the device. The mapping can be configured or pre-defined. Therefore, from RAN2 perspective, if it is not pre-defined, the mapping between the dedicated resource and the device can be configured in the AIoT paging message.

Proposal 3: RAN2 confirms that the AIoT paging message containing multiple IDs of A-IoT devices will be studied.

Proposal 4: Mapping between the dedicated resource and the AIoT device can be configured in the AIoT paging message, when dedicated resources are configured for multiple devices.

In case of shared resources provided to multiple devices, a RFID-like method can be used. For example, a parameter Q is included in the AIoT paging message and the number of access occasions is 2Q. Alternatively, a parameter N, i.e. the number of access occasions can be indicated directly. With this information, each triggered AIoT device can determine the access occasion for D2R data transmission by itself.

Proposal 5: When shared resources are configured for multiple devices, the following parameter needs to be configured in the AIoT paging message:

- a parameter Q (like RFID mechanism); or

- a parameter N indicating the number of access occasions

Also, RAN1 has agreed that FDMA is studied when a response is expected from multiple devices. Then the information to allocate frequency domain resources for devices access or the information for a device to determine the frequency domain resource allocation can be included in the A-IoT paging message.

Proposal 5bis: To support FDMA, the information to allocate frequency domain resources or the information for a device to determine the frequency domain resource allocation needs to be included in the A-IoT paging message.

Abovementioned information should be considered for all use cases, i.e. use case of “inventory-only”, “inventory and command” and “command-only”. There is some information required only for the use case of “command-only”.

Since there is no conclusion on whether the command can be included in the paging message, the information on whether to monitor or receive subsequent R2D transmission including the command needs to be indicated in the AIoT paging message, if the command is not included in the AIoT paging message. Some information about response to the command can be included in the AIoT paging no matter if the command is included in the AIoT paging or not. The benefit to include the information is that the UE may discard the random number, or the temporary identifier earlier, e.g., if no response is needed. If the command is not so critical and the network can send this command without expecting any response, then the response to the command may be not needed. From this perspective, indication if any or no response to the command is included in the AIoT paging. Besides, if the response is required, the paging message can also indicate if the response can be delayed. If the response cannot be delayed, the triggered AIoT device with low battery can take some actions.

Proposal 6: For the use case of “command-only”, the AIoT Paging message also includes:

- Indication if any response to the command is needed.

- Indication if the response can be delayed when the response is needed.

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Paging and made the following proposals:

Proposal 1: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the AIoT paging message.

Proposal 2: ID type, e.g., device ID or group ID is indicated explicitly or implicitly in the AIoT paging message.

Proposal 3: RAN2 confirms that the AIoT paging message containing multiple IDs of A-IoT devices will be studied.

Proposal 4: Mapping between the dedicated resource and the AIoT device can be configured in the AIoT paging message, when dedicated resources are configured for multiple devices.

Proposal 5: When shared resources are configured for multiple devices, the following parameter needs to be configured in the AIoT paging message:

- a parameter Q (like RFID mechanism); or

- a parameter N indicating the number of access occasions

Proposal 5bis: To support FDMA, the information to allocate frequency domain resources or the information for a device to determine the frequency domain resource allocation needs to be included in the A-IoT paging message.

Proposal 6: For the use case of “command-only”, the AIoT Paging message also includes:

- Indication if any response to the command is needed.

- Indication if the response can be delayed when the response is needed.

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March 18-21, 2024

- R2-2404102, “Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China”, ETSI MCC, RAN2#126

- RAN2#126 Chair Notes, RAN2 Chair (InterDigital), RAN2#126

- R2-240xxxx, “TP for TR 38.769 update”, Huawei, RAN2#127bis

- R2-2408147, “Discussions on AIoT Random Access”, Fujitsu, RAN2#127