3GPP TSG-RAN WG2 Meeting #127bis	R2-2408151

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on conditional LTM

Document for:	Discussion and Decision

- Introduction

In the RAN#105 meeting, it was agreed that conditional LTM would be in scope of Rel-19 as follows:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

This document discusses basic function of conditional LTM for non-DC case.

- Discussions

Conditions for triggering LTM

Regarding event-triggered L1 measurement reporting, some agreements are made. For triggering LTM by the UE, these agreements can be baseline.

For example, the following LTM events are supported for event-triggered L1 measurement reporting:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

We think that Event LTM3/Event LTM5 can be used for triggering LTM by the UE. However, whether beam level measurement or cell level measurement is used for the LTM event evaluation needs more discussion.

Proposal 1: For conditional LTM, RAN2 to agree that the following LTM events are supported for triggering LTM:

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Proposal 1bis: For conditional LTM, RAN2 to discuss whether beam level measurement or cell level measurement is used for triggering LTM.

Conditional RACH-less LTM

Intra-CU LTM cell switch execution can be performed with or without RACH. In Rel-18, intra-CU LTM cell switch execution without RACH is performed if the UE has valid TA. With similar intention, e.g., to reduce mobility latency, cell switch execution without RACH for conditional LTM has some benefits. However, more discussions are needed.

To support cell switch without RACH, in Rel-18, valid TA can be indicated by the network in the LTM cell switch command or acquired by the UE based TA evaluation. In case of conditional LTM, valid TA cannot be indicated in the LTM cell switch command since there is no such command. However, at least UE based TA can be used to support intra-CU LTM cell switch execution without RACH.

Then the next issue is whether early RACH towards candidate cell for TA acquisition can be supported or not. In Rel-18, early RACH towards candidate cell is triggered by the network via PDCCH order and valid TA for target cell can be included in the LTM cell switch command. The trigger of early RACH towards candidate cell can be reused for conditional LTM. For TA provision, one possibility is RAR-based TA acquisition, as discussed in Rel-18 and Rel-19 inter-CU LTM. However, if the RAR is received from candidate cell, it may incur more interruption. We think that TA value for candidate cell is provided by the serving cell via RAR or TA command MAC CE if early RACH towards candidate cell for TA acquisition needs to be supported.

Observation 1: If conditional RACH-less LTM is supported, a TA Command MAC CE or RAR from serving cell to provide TA value(s) for candidate cell needs to be introduced.

If TA for candidate cell is maintained by the UE, the UE needs to check whether the TA value for target cell is valid or not when conditional LTM cell switch is triggered.

For RACH-based conditional LTM, as legacy, TX/RX beam can be selected by the UE during RACH according to corresponding L1 RSRP even if there is no LTM cell switch command indicating the TCI state(s) from the network. The selected beams can be used during the LTM cell switch and after the cell switch until a new TCI state is activated/indicated by the network. In case of conditional RACH-less LTM, how to determine TX/RX beam needs more discussions. The discussion can be in the scope of RAN1. So, RAN1 needs to be involved.

Observation 2: If conditional RACH-less LTM is supported, RAN1 inputs on beam determination will be needed.

In Rel-18, in case of RACH-less LTM, the gNB considers the LTM completed when the first UL transmission is received, and the UE considers the LTM completed when the UE confirms that the first UL transmission is received at the gNB. The first UL transmission can be dynamic scheduled by the gNB or carried by the configured grant corresponding to the TCI state indicated in the LTM cell switch command. However, there are some concerns for conditional LTM. For dynamic scheduling, the target cell or the network cannot know when the UE will perform cell switch and will not send PDCCH to schedule UL resources for the first UL data since the cell switch is triggered by the UE instead of the gNB. For configured grant option, the UE cannot determine the UL grant since there is no LTM cell switch command indicating the TCI state(s) from the network. But, if TCI state(s) can be determined by the UE, Rel-18 mechanism for the first UL data transmitted by configured grant can be reused.

Observation 3: For conditional RACH-less LTM, dynamic scheduling cannot be used for the first UL transmission to target cell.

We think that RACH-based LTM cell switch can be supported for conditional LTM, as in Rel-18. However, the support of RACH-less LTM cell switch needs more discussions considering abovementioned concerns.

Proposal 2: RACH-based LTM cell switch is supported for intra-CU conditional LTM.

Proposal 2bis: RAN2 to discuss whether RACH-less cell switch is supported or not for intra-CU conditional LTM, taking Observation 1~3 into accounts.

Signalling procedure for conditional LTM

Based on these discussions and signalling procedure for Rel-18 intra-CU LTM, the signalling procedure for conditional LTM can be captured in the figure below:

Figure 1: Signalling procedure for conditional LTM

Proposal 3: For conditional LTM, RAN2 to agree signaling procedure in Figure 1.

- Conclusion

This document discussed basic function of conditional LTM for non-DC case and made the following observations and proposals:

Conditions for triggering LTM

Proposal 1: For conditional LTM, RAN2 to agree that the following LTM events are supported for triggering LTM:

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Proposal 1bis: For conditional LTM, RAN2 to discuss whether beam level measurement or cell level measurement is used for triggering LTM.

Conditional RACH-less LTM

Observation 1: If conditional RACH-less LTM is supported, a TA Command MAC CE or RAR from serving cell to provide TA value(s) for candidate cell needs to be introduced.

Observation 2: If conditional RACH-less LTM is supported, RAN1 inputs on beam determination will be needed.

Observation 3: For conditional RACH-less LTM, dynamic scheduling cannot be used for the first UL transmission to target cell.

Proposal 2: RACH-based LTM cell switch is supported for intra-CU conditional LTM.

Proposal 2bis: RAN2 to discuss whether RACH-less cell switch is supported or not for intra-CU conditional LTM, taking Observation 1~3 into accounts.

Signalling procedure for conditional LTM

Proposal 3: For conditional LTM, RAN2 to agree signaling procedure in Figure 1.