3GPP TSG-RAN WG2 Meeting #127bis	R2-2408169

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.3

Source:	Spreadtrum Communications

Title:	Discussion on RRM measurement relaxation in IDLE/INACTIVE mode

Document for:	Discussion and Decision

1 Introduction

In RAN2#127 meeting, RAN2 discussed the RRM measurement relaxation and offloading in RRC_IDLE INACTIVE mode and achieved the following agreements [1]:

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In this contribution, we will discuss the RRM measurement relaxation of RRC_IDLE/INACTIVE UE MR for neighbor cell.

2 Discussion

## 2.1 RRM measurement relaxation for neighboring cell

In R16, the neighboring cell RRM measurement relaxation for intra-frequency or inter-frequency/inter-RAT frequency is allowed for UEs not at cell edge and/or with low mobility, detailed measurement relaxation rules are shown in the figure 1.

Figure 1 the legacy RRM measurement relaxation for neighboring cell

Observation 1: In Rel-16, the neighboring cell RRM measurement relaxation is allowed for UEs not at cell edge and/or with low mobility.

In general, the neighbour cell measurements only need to be performed by the UE if the serving cell is not strong enough (e.g., when the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is not fulfilled). However, the LP-WUS may be deactivated before the neighbour cell measurements since there will be some neighboring cells areas which are not covered by LP-WUS, as shown in figure 2. Moreover, RAN2 has agreed that the RRM measurement of the neighboring cell can only be performed by MR.

Figure 2 the LP-WUS coverage

Observation 2: The RRM measurement of the neighboring cell can only be performed by MR.

The current neighbor cell measurement relaxation can achieve UE energy saving, RAN2 considers whether further need to relax neighbor cell RRM measurement for LP-WUS MR. And specify the criteria for relaxing neighbor cell RRM measurement if necessary.

In our view, to simplify the complexity of the design LP-WUS, it at least can reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR, which includes two scenarios: not at cell edge and low mobility, respectively.

When the intra-frequency cell re-selection criterion (Srxlev > SIntraSearchP and Squal > SIntraSearchQ) or the inter-frequency cell re-selection criterion (Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ) is fulfilled, and if low mobility is also fulfilled, UE does not measure intra-frequency, lower priority frequency and higher priority frequency. Otherwise, UE will measure higher priority frequency.

When SSearchThresholdP < Srxlev < SIntraSearchP/nonIntraSearchP or SSearchThresholdQ < Squal < SIntraSearchQ/nonIntraSearchQ, and if low mobility is fulfilled, UE does not measure intra-frequency, lower priority frequency and higher priority frequency. Otherwise, UE will perform the measurement relaxation, i.e. measure intra-frequency every 2nd DRX, measure lower priority frequency every 2nd DRX and higher priority frequency every minute.

When Srxlev < SSearchThresholdP or Squal < SSearchThresholdQ, and if low mobility is fulfilled, UE will perform the measurement relaxation, i.e. measure intra-frequency every 4th DRX and lower priority frequency every 4th DRX, and does not measure higher priority frequency. Otherwise, UE will perform the measurement criterion of R15, i.e. measure intra-frequency every DRX, measure lower priority frequency every DRX and higher priority frequency every minute.

Proposal 1: RAN2 reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR.

3 Conclusion

In this contribution we discuss the RRM measurement relaxation for idle/inactive UE, with the following proposals:

Observation 1: In Rel-16, the neighboring cell RRM measurement relaxation is allowed for UEs not at cell edge and/or with low mobility.

Observation 2: The RRM measurement of the neighboring cell can only be performed by MR.

Proposal 1: RAN2 reuse the neighboring cell RRM measurement relaxation of R16 for LP-WUS MR.

- References

- Draft report of 3GPP TSG RAN WG2 #126, RAN2 chair (interDigtal), Maastricht, Holland, Aug 19th – 23rd, 2024.